Document Number 	EDCS-459338

Revision		31.0

Author		Jack Daih

Cisco BTS 10200 Softswitch Software Upgrade for Release

3.5.5 to 4.5

October 6, 2006

Corporate Headquarters

Cisco Systems, Inc.

170 West Tasman Drive

San Jose, CA 95134-1706

USA

http://www.cisco.com

Tel: 	408 526-4000

800 553-NETS (6387)

Fax:	408 526-4100

THE SPECIFICATIONS AND INFORMATION REGARDING THE PRODUCTS IN THIS MANUAL ARE SUBJECT TO CHANGE WITHOUT NOTICE. ALL STATEMENTS, INFORMATION, AND RECOMMENDATIONS IN THIS MANUAL ARE BELIEVED TO BE ACCURATE BUT ARE PRESENTED WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED. USERS MUST TAKE FULL RESPONSIBILITY FOR THEIR APPLICATION OF ANY PRODUCTS.

THE SOFTWARE LICENSE AND LIMITED WARRANTY FOR THE ACCOMPANYING PRODUCT ARE SET FORTH IN THE INFORMATION PACKET THAT SHIPPED WITH THE PRODUCT AND ARE INCORPORATED HEREIN BY THIS REFERENCE. IF YOU ARE UNABLE TO LOCATE THE SOFTWARE LICENSE OR LIMITED WARRANTY, CONTACT YOUR CISCO REPRESENTATIVE FOR A COPY.

The Cisco implementation of TCP header compression is an adaptation of a program developed by the University of California, Berkeley (UCB) as part of UCB’s public domain version of the UNIX operating system. All rights reserved. Copyright © 1981, Regents of the University of California.

NOTWITHSTANDING ANY OTHER WARRANTY HEREIN, ALL DOCUMENT FILES AND SOFTWARE OF THESE SUPPLIERS ARE PROVIDED “AS IS” WITH ALL FAULTS. CISCO AND THE ABOVE-NAMED SUPPLIERS DISCLAIM ALL WARRANTIES, EXPRESSED OR IMPLIED, INCLUDING, WITHOUT LIMITATION, THOSE OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT OR ARISING FROM A COURSE OF DEALING, USAGE, OR TRADE PRACTICE.

IN NO EVENT SHALL CISCO OR ITS SUPPLIERS BE LIABLE FOR ANY INDIRECT, SPECIAL, CONSEQUENTIAL, OR INCIDENTAL DAMAGES, INCLUDING, WITHOUT LIMITATION, LOST PROFITS OR LOSS OR DAMAGE TO DATA ARISING OUT OF THE USE OR INABILITY TO USE THIS MANUAL, EVEN IF CISCO OR ITS SUPPLIERS HAVE BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

CCIP, CCSP, the Cisco Arrow logo, the Cisco Powered Network mark, the Cisco Systems Verified logo, Cisco Unity, Follow Me Browsing, FormShare, iQ Breakthrough, iQ FastTrack, the iQ Logo, iQ Net Readiness Scorecard, Networking Academy, ScriptShare, SMARTnet, TransPath, and Voice LAN are trademarks of Cisco Systems, Inc.; Changing the Way We Work, Live, Play, and Learn, The Fastest Way to Increase Your Internet Quotient, and iQuick Study are service marks of Cisco Systems, Inc.; and Aironet, ASIST, BPX, Catalyst, CCDA, CCDP, CCIE, CCNA, CCNP, Cisco, the Cisco Certified Internetwork Expert logo, Cisco IOS, the Cisco IOS logo, Cisco Press, Cisco Systems, Cisco Systems Capital, the Cisco Systems logo, Empowering the Internet Generation, Enterprise/Solver, EtherChannel, EtherSwitch, Fast Step, GigaStack, Internet Quotient, IOS, IP/TV, iQ Expertise, LightStream, MGX, MICA, the Networkers logo, Network Registrar, Packet, PIX, Post-Routing, Pre-Routing, RateMUX, Registrar, SlideCast, StrataView Plus, Stratm, SwitchProbe, TeleRouter, and VCO are registered trademarks of Cisco Systems, Inc. and/or its affiliates in the U.S. and certain other countries.

All other trademarks mentioned in this document or Web site are the property of their respective owners. The use of the word partner does not imply a partnership relationship between Cisco and any other company. (0301R)

Cisco BTS 10200 Softswitch Software Upgrade

Copyright © 2005, Cisco Systems, Inc.

All rights reserved.

| Revision History |  |  |  |
| --- | --- | --- | --- |
| Date | Version | Revised By | Description |
| 7/26/2005 | 1.0 | Jack Daih | - Initial Version |
| 8/5/2005 | 2.0 | Das Tarun | Added manual steps required during Fallback. |
| 8/10/2005 | 3.0 | Jack Daih | - Modified Appendix G link configuration |
| 8/11/2005 | 4.0 | Jack Daih | - Added new comments in Chapter 3, Task 7 for verifying SS7 CLI file generated by upgrade script program. |
| 8/11/2005 | 5.0 | Jack Daih | - Added steps to remove ssh key information in Chapter 4, Task 4-5, and Task 9-10. |
| 8/12/2005 | 6.0 | Jack Daih | - Changed DoTheChange parameter for side A from –s to -p |
| 8/12/2005 | 7.0 | Jack Daih | - Correct chapter and task reference for getting the IP and netmask information |
| 8/13/2005 | 8.0 | Jack Daih | - Added steps to change the /etc/default/init file to avoid message: couldn't set locale correctly |
| 8/14/2005 | 9.0 | Jack Daih | - Corrected typo in command “ls –l c/hostname.*” in Chapter 4, Task 4 and 5 to “ls –l /etc/hostname.*” |
| 8/14/2005 | 10.0 | Jack Daih | - Added reboot step after changing TIMEZONE information |
| 8/18/2005 | 11.0 | Jack Daih | - Added Appendix K for enabling the disk mirroring |
| 8/19/2005 | 12.0 | Jack Daih | - Correct typo in Appendix K. |
| 8/22/2005 | 13.0 | Jack Daih | - Added hyper link to the 9/5 to 4/2 network interface migration procedure |
| 9/15/2005 | 14.0 | Sateesh Hegde | - TWC feedback included |
| 9/22/2005 | 15.0 | Jack Daih | - Revised Disk Preparation steps and removed flash archive Appendix. |
| 9/30/2005 | 16.0 | Jack Daih | - Added Netra 240 hardware lay out and removed Sunfire V120 |
| 10/17/2005 | 17.0 | Jack Daih | - Incorporated comments from system testing |
| 11/10/2005 | 18.0 | Jack Daih | - Revised Appendix J to remove unnecessary steps. |
| 11/29/2005 | 19.0 | Jack Daih | - Removed parameters NAMED_ENABLED, NSCD_ENABLED, MARKET_TYPE. Added pre-upgrade on table call-agent-profile for billing cdb support flag and Task 1 to remove Second Quad Ethernet card from CA/FS machines. |
| 12/6/2005 | 20.0 | Jack Daih | - Added verification bullets in Chapter 5, Task 6, Step 18 and Step 20. Modified the disk mirroring steps to have the system done one side at a time. |
| 12/9/2005 | 21.0 | Jack Daih | - Removed patch applying steps. |
| 1/6/2006 | 22.0 | Jack Daih | - Added pre-upgrade step to make sure there are terminations associated with ss7 trunk groups. |
| 1/30/2006 | 23.0 | Jack Daih | - Correct the error in the SQL statement for pre-checking the SS7 trunk groups. |
| 2/26/2006 | 24.0 | Jack Daih | - Added warning message in the full system fallback procedure about loss of new provisioned data during upgrade. - Added pre-upgrade check step for defect CSCsd35533 - Added break point when side B systems are upgraded and in Active state - Changed CD staging steps to account for the introduction of externally built oracle DB engine. |
| 2/27/2006 | 25.0 | Jack Daih | - Added two steps in Appendix G to remove the redundant E-links and enable Sigtran. |
| 3/6/2006 | 26.0 | Matthew Lin | - Added expected result for SQL command when no trunk groups without termination. - Added “sync;sync” for “shutdown –i5” commands. - Changed “shutdown –i6” commands to “reboot”. - Added “sync;sync;” before all the reboot command |
| 3/11/2006 | 27.0 | Matthew Lin / Jack Daih | - Resolve defect CSCsd60179 which contains 9 other defects for upgrade. - Resolve defect CSCsd62192 which contains 3 other defects for fallback. - Added Time-to-Live DNS check steps |
| 4/6/2006 | 28.0 | Jack Daih | - Resolve defect: CSCsd61186, CSCsd61208, CSCsd61219, CSCsd61937, CSCsd62192, CSCsd75892, CSCsd77641 |
| 5/1/2006 | 29.0 | Jack Daih | - After conference call with TWC engineering and AS, the final agreement for preparing the disks are to enable disk mirroring to save one extra maintenance window with previous method. Moved steps in Appendix J Disk mirror to be part of the disk preparation and removed Appendix. |
| 5/6/2006 | 30.0 | Jack Daih | - Added a new task 1 in Chapter 5 to save the shared memory before starting the upgrade process. - Rewrote entire half fallback procedure to account for the new SHM requirement for fallback. - When mid-upgrade point is reached (Side B active, Side A standby), instead of having side A in hot-standby, the new approach is to shutdown the side B down in preparation of fallback if the system unable to sustain either soak or new traffic on 4.5. - Added Task 6 and 7 in Chapter 6 to verify DB Heartbeat process on both EMS and verify SSH daemon and listening port status |
| 10/06/2006 | 31 | Mahmood Hadi | - Updated to resolve CSCsf96809 |

Table of Contents

Table of Contents	6

Table of Contents	6

Preface	11

Obtaining Documentation	11

World Wide Web	11

Documentation CD-ROM	11

Ordering Documentation	11

Documentation Feedback	12

Obtaining Technical Assistance	12

Cisco.com	12

Technical Assistance Center	13

Cisco TAC Web Site	13

Cisco TAC Escalation Center	14

Chapter 1	15

Upgrade Requirements	15

Introduction	15

Assumptions	16

Requirements	16

Important notes about this procedure	18

Chapter 2	20

Preparation	20

Complete 4-6 weeks before the scheduled upgrade	20

Task 1: Remove second Quad Ethernet Card from CA/FS machines	20

Task 2: Install signaling gateways and links	21

Task 3: Setting up MTP and M3UA configuration on the signaling gateway	21

Task 4: Create E-links and linksets between BTS and ITP	21

Task 5: Prepare ITP SCTP configuration command files	22

Task 6: Purchase and Prepare Disks	22

Prerequisites	22

Chapter 3	24

Complete 2-4 weeks before the scheduled upgrade	24

Task 1: Pre-construct opticall.cfg for the system to be upgraded to 4.5 release	24

Add new domain names to DNS	25

Task 2: Save customized cron jobs	27

Task 3: Collecting network and IP information	28

From EMS side A	28

From CA/FS side A	28

From EMS side B	28

From CA/FS side B	29

Task 4: Stage upgrade programs	29

From EMS Side A, EMS Side B, CA/FS Side A and	29

CA/FS Side B	29

Task 5: Prepare configuration files	30

Stage Release 4.5 opticall.cfg	30

Prepare SS7 CLI script and for migration testing	31

From EMS Side A	31

Prepare for SS7 migration testing	35

From EMS Side A	35

Prepare the critical SS7 trunk group file	37

From EMS Side A	37

Task 6: Prepare Scripts for post-upgrade new feature activation	38

Create announcement CLI file	38

From EMS Side A	38

Create CLI file for new feature creation and existing feature update	39

From EMS Side A	39

Task 7: Pre-checking database integrity and correction	39

From EMS Side A	39

Task 8: Store the pre-constructed 4.5 opticall.cfg file	40

From EMS Side A	40

Task 9: Prepare IOS commands	41

Task 10: Verify console server lines to BTS do not have a timeout entry.	41

Task 11: Add logical MGCP domain name IP addresses to MGX/VISM	41

Task 12: Verify value of CA CONTROL PORT for IVR Devices connected to BTS.  	42

Note : The value of the CA_CONTROL_PORT for IVR devices connected to BTS should not be 0.	42

From Active EMS	42

Chapter 4	48

Prepare System for Upgrade	48

Task 1: Audit subscriber database	48

From Active EMS	48

Task 2: Check Oracle Database Replication and Error Correction	49

Chapter 5	50

Start Upgrade Process	50

Task 1: Saving Shared Memory	50

From Active EMS Side	50

From CA/FS Side A	51

Task 2: Change MGCP domain IP addresses and set Time-to-Live (TTL) value from DNS	51

Change MGCP domain IP addresses from DNS	51

Verify Time-to-Live (TTL) from Primary CA/FS	52

Task 3: Change the root password	53

Task 4: Force Side A Systems to be Active	53

From Active EMS Side	53

Task 5: Start upgrade process	54

From EMS Side A	54

Task 6: Upgrade EMS side B to the new release	57

From EMS side B	57

Task 7: Upgrade CA/FS Side B to the new release	62

From CA/FS side B	62

Task 8: Continue Upgrade Process	68

From EMS side B (The Active EMS)	69

Task 9: Upgrade EMS side A to the new release	72

From EMS side A	72

Task 10: Upgrade CA/FS Side A to the new release	76

From CA/FS side A	76

Task 11: Restore customized cron jobs	80

Task 12: Re-start the Upgrade program on EMS Side A	80

From EMS side A	80

Task 13: Check audit report and sync DB if necessary	81

From EMS side A	81

Step 1   Check audit report to see if there are any mismatches	81

Step 2   If mismatches are found, run the following step to synchronize DB between EMS and CA/FS.	81

Chapter 6	83

Post Upgrade Tasks	83

Task 1: Reset TTL value from DNS	83

Task 2: Change the password	84

Task 3: Provisioning new features	84

From EMS side A	84

Task 4: Restore platform.cfg with logical IPs	85

From EMS side A	86

Task 5: Check disk mirroring status	86

Task 6: Verify DB Heartbeat process on both EMS	86

From EMS side A and B	87

Task 7: Verify SSH daemon and listening port status	87

From each machine in the system	87

Task 8: Install CORBA Application	88

Appendix A	89

System Disaster Recovery Procedure	89

Appendix B	90

Half System Fallback	90

Introduction	90

Task 1: Control SCTP associations Out of Service	91

From Active EMS Side B	91

Task 2: Restore EMS side A Hub communication	91

From EMS side A	91

Task 3: Shutdown side B	91

From CA/FS side B	92

From EMS side B	92

Task 4: Restore MGCP physical IPs	92

Verify Time-to-Live (TTL) from Primary CA/FS	93

Task 5: Start Side A applications	94

From CA/FS side A	94

From EMS side A	94

Task 6: Restore Side A EMS DB replication	94

Task 7: Remove SCTP configuration and restore E-links and linkset from ITPs	94

Remove SCTP configuration from ITPs	94

Restore OMNI SS7 link(s) and linkset from ITP	94

Task 8: Activate Omni SS7 A-links on CA/FS side A	96

From CA/FS side A	96

Task 9: Restore side B and save a copy of CA/FS shared memory	97

From CA/FS side B	97

From EMS side B	97

Task 10: Activate Omni SS7 A-links on CA/FS side B	98

From CA/FS side B	98

Task 11: Reset TTL value from DNS	98

Task 12: Verify system status	99

From EMS side A	99

Task 13: Post fallback clean up steps	101

Appendix C	102

Setting up MTP and M3UA Configuration on the Signaling Gateway	102

Requirements and Prerequisites	102

Preparation	102

Task 1: Define MTP Variant	103

Task 2: Define Point Code	103

Task 3: Define Ethernet Configuration	103

Task 4: Define SGMP (When SG Mated Pair is used)	104

Task 5: Define M3UA Port Number	104

Task 6: Define Application Server Process (ASP)	104

Task 7: Define SS7 Links	105

Task 8: Define Linkset	106

Task 9: Define SS7 Route Sets	107

Task 10: Define Routing Key	108

Appendix D	109

Setting up SCTP configuration on the Signaling Gateway	109

Task 1: Define SUA Port Number	109

Task 2: Define ASP	109

Task 3: Define Routing Keys for Various Services	110

Task 4: Define GTT Configuration	111

Task 5: Saving the Configuration	112

Appendix E	113

Preparing Disks for Upgrade	113

Task 1: Locate CD-ROM Discs	113

Task 2: Locate and label the Disks	113

Label disks for EMS Side A	113

Label Disks for EMS Side B	113

Label Disks for CA/FS Side A	114

Label Disks for CA/FS Side B	114

Task 3: Disk slot lay out	114

Task 4: Construct opticall.cfg	115

Task 5: Disk preparation	115

For both EMS side A and B	115

For both CA/FS side A and B	119

Appendix F	122

Add New Announcements	122

Task 1: Record Announcements	122

Task 2: Place Announcements	122

From Active EMS	122

Appendix G	124

SS7 Migration	124

Initial State	124

Transition State	125

Final State	129

ITP configuration information	129

BTS 10200 Provisioning for the SG Mated Pair	140

Appendix H	148

Provisioning Release 4.5 Specific Configuration on the Cisco BTS 10200 Call Agent	148

Preparation	148

Appendix I	151

Provisioning Release 4.5 Specific Configuration on the Cisco BTS 10200 Feature Server	151

Appendix J	157

Check Oracle Database Replication and Error Correction	157

Check Oracle DB replication status	157

From EMS side A	157

Correct replication error	158

From EMS Side B	158

From EMS Side A	158

Appendix K	160

4/2 Port Configuration Reference Chart	160

Appendix L	161

CORBA Installation	161

Task 1: Install OpenORB CORBA Application	161

Remove Installed OpenORB Application	161

Install OpenORB Packages	162

Appendix M	164

Prepare Side A System for Fallback	164

Task 1: Restore CA/FS side A shared memory data	164

From CA/FS side A	164

Task 2: Clean up EMS side A shared memory data	164

From EMS side A	164

Task 3: Restore EMS side A oracle DB replication	165

From EMS side A	165

Preface

# Obtaining Documentation

These sections explain how to obtain documentation from Cisco Systems.

### World Wide Web

You can access the most current Cisco documentation on the World Wide Web at this URL: http://www.cisco.com/

Translated documentation is available at this URL: http://www.cisco.com/public/countries_languages.shtml

### Documentation CD-ROM

Cisco documentation and additional literature are available in a Cisco Documentation CD-ROM package, which is shipped with your product. The Documentation CD-ROM is updated monthly and may be more current than printed documentation. The CD-ROM package is available as a single unit or through an annual subscription.

### Ordering Documentation

You can order Cisco documentation in these ways:

Registered Cisco.com users (Cisco direct customers) can order Cisco product documentation from the Networking Products MarketPlace: http://www.cisco.com/cgi-bin/order/order_root.pl

Registered Cisco.com users can order the Documentation CD-ROM through the online Subscription Store: http://www.cisco.com/go/subscription

No registered Cisco.com users can order documentation through a local account representative by calling Cisco Systems Corporate Headquarters (California, U.S.A.) at 408 526-7208 or, elsewhere in North America, by calling 800 553-NETS (6387).

### Documentation Feedback

You can submit comments electronically on Cisco.com. In the Cisco Documentation home page, click the Fax or Email option in the “Leave Feedback” section at the bottom of the page.

You can e-mail your comments to mailto: bug-doc@cisco.com.

You can submit your comments by mail by using the response card behind the front cover of your document or by writing to the following address:

Cisco Systems, INC.

Attn: Document Resource Connection

170 West Tasman Drive

San Jose, CA 95134-9883

# Obtaining Technical Assistance

Cisco provides Cisco.com as a starting point for all technical assistance. Customers and partners can obtain online documentation, troubleshooting tips, and sample configurations from online tools by using the Cisco Technical Assistance Center (TAC) Web Site. Cisco.com registered users have complete access to the technical support resources on the Cisco TAC Web Site: http://www.cisco.com/tac

### Cisco.com

Cisco.com is the foundation of a suite of interactive, networked services that provides immediate, open access to Cisco information, networking solutions, services, programs, and resources at any time, from anywhere in the world.

Cisco.com is a highly integrated Internet application and a powerful, easy-to-use tool that provides a broad range of features and services to help you with these tasks:

Streamline business processes and improve productivity

Resolve technical issues with online support

Download and test software packages

Order Cisco learning materials and merchandise

Register for online skill assessment, training, and certification programs

If you want to obtain customized information and service, you can self-register on Cisco.com. To access Cisco.com, go to this URL: http://www.cisco.com/

### Technical Assistance Center

The Cisco Technical Assistance Center (TAC) is available to all customers who need technical assistance with a Cisco product, technology, or solution. Two levels of support are available: the Cisco TAC Web Site and the Cisco TAC Escalation Center.

Cisco TAC inquiries are categorized according to the urgency of the issue:

Priority level 4 (P4)—you need information or assistance concerning Cisco product capabilities, product installation, or basic product configuration.

Priority level 3 (P3)—your network performance is degraded. Network functionality is noticeably impaired, but most business operations continue.

Priority level 2 (P2)—your production network is severely degraded and is affecting significant aspects of business operations. No workaround is available.

Priority level 1 (P1)—your production network is down, and a critical impact to business operations will occur if service is not restored quickly. No workaround is available.

The Cisco TAC resource that you choose is based on the priority of the problem and the conditions of service contracts, when applicable.

### Cisco TAC Web Site

You can use the Cisco TAC Web Site to resolve P3 and P4 issues yourself, saving both cost and time. The site provides around-the-clock access to online tools, knowledge bases, and software. To access the Cisco TAC Web Site, go to this URL: http://www.cisco.com/tac

All customers, partners, and resellers who have a valid Cisco service contract have complete access to the technical support resources on the Cisco TAC Web Site. The Cisco TAC Web Site requires a Cisco.com Log in ID and password. If you have a valid service contract but do not have a Log in ID or password, go to this URL to register: http://www.cisco.com/register/

If you are a Cisco.com registered user, and you cannot resolve your technical issues by using the Cisco TAC Web Site, you can open a case online by using the TAC Case Open tool at this URL: http://www.cisco.com/tac/caseopen

If you have Internet access, we recommend that you open P3 and P4 cases through the Cisco TAC Web Site: http://www.cisco.com/tac

### Cisco TAC Escalation Center

The Cisco TAC Escalation Center addresses priority level 1 or priority level 2 issues. These classifications are assigned when severe network degradation significantly impacts business operations. When you contact the TAC Escalation Center with a P1 or P2 problem, a Cisco TAC engineer automatically opens a case.

To obtain a directory of toll-free Cisco TAC telephone numbers for your country, go to this URL: http://www.cisco.com/warp/public/687/Directory/DirTAC.shtml

Before calling, please check with your network operations center to determine the level of Cisco support services to which your company is entitled: for example, SMARTnet, SMARTnet Onsite, or Network Supported Accounts (NSA). When you call the center, please have available your service agreement number and your product serial number.

Chapter 1

Upgrade Requirements

# Introduction

Application software loads are designated as Release 900-aa.bb.cc.Vxx, where

- aa=major release number, for example, 01

- bb=minor release number, for example, 03

- cc=maintenance release, for example, 00

- Vxx=Version number, for example V04

This procedure can be used on an in-service system, but the steps must be followed as shown in this document in order to avoid traffic interruptions.

|  | Caution Performing the steps in this procedure will bring down and restart individual platforms in a specific sequence. Do not perform the steps out of sequence, as it could affect traffic. If you have questions, contact Cisco Support. |
| --- | --- |

This procedure should be performed during a maintenance window.

Note:   In this document, the following designations are used:

- EMS -- Element Management System

- CA/FS -- Call Agent / Feature Server

- Primary -- Also referred to as "Side A"

- Secondary -- Also referred to as "Side B"

# Assumptions

The following assumptions are made.

- The installer has a basic understanding of UNIX and Oracle commands.

- The installer has the appropriate user name(s) and password(s) to log on to each EMS/CA/FS platform as root user, and as Command Line Interface (CLI) user on the EMS.

- The password for the root user must be the same across all the BTS systems during the upgrade process. The password should be set to “opticall”.

|  | Note: Contact Cisco Support before you start if you have any questions. |
| --- | --- |

# Requirements

Verify that opticall.cfg has the correct information for each of the following machines.

- Side A EMS

- Side B EMS

- Side A CA/FS

- Side B CA/FS

Determine the oracle and root passwords for the systems you are upgrading. If you do not know these passwords, ask your system administrator.

Refer to local documentation to determine if CORBA installation is required on this system. If unsure, ask your system administrator.

The flow for this upgrade procedure is shown in Figure-1.

# Important notes about this procedure

Throughout this procedure, each command is shown with the appropriate system prompt, followed by the command to be entered in bold. The prompt is generally one of the following:

- Host system prompt (<hostname>#)

- Oracle prompt (<hostname>$)

- SQL prompt (SQL>)

- CLI prompt (CLI>)

- SFTP prompt (sftp>)

Note the following conventions used throughout the steps in this procedure:

- Enter commands as shown, as they are case sensitive (except for CLI commands).

Note 1:  It is recommended that you read through the entire procedure before performing any steps.

Note 2:  To shorten the upgrade window, Cisco recommends the full database audit be performed at the night before the upgrade if it is absolutely certain that no provisioning activities will occur during the next 24 hour period.

Note 3:  It will take approximately between 5-9 hours to complete the entire upgrade process depending on the number of subscribers provisioned in the system. Please plan accordingly to minimize any negative service impacts.

Note 4:  CDR delimiter customization is not retained after software upgrade. The customer or Cisco engineer must manually customize again to keep the same customization.

Note 5:  The total SS7 outage time depends on the number of SS7 CICs provisioned in the system. Based on data collected from lab testing, for a system with 10K CICs, the time it took from the blocking to unblocking and calls being made was 10 minutes.

Note 6:  There will be no CLI provisioning allowed during entire upgrade process.

After entire system is upgraded to release 4.5, if the system is provisioned with new CLI data fallback is not recommended. Attempt to fallback will result in system wide service outage and lost of data. If fallback is absolutely required, please contact Cisco support.

Chapter 2

Preparation

# Complete 4-6 weeks before the scheduled upgrade

# Task 1: Remove second Quad Ethernet Card from CA/FS machines

It is critical to have the quad Ethernet cards in the correct locations per Appendix K.  One quad Ethernet card should be in PCI slot 0 in each CA and no cards in the EMS machines.  If extra unused quad Ethernet cards are present, it is possible that when the disk is swapped out with Solaris 10, the Ethernet port will be mis-assigned. The port assignment information is kept in the /etc/path_to_inst file.  Manual correction of the Ethernet ports are then required.

If port mis-assignment happens, do “reboot -- -r” command to reconfigure the Ethernet ports.  You may need to repeat this command until the port assignment is correct.  The following are the correct and wrong examples from Sunfire 1280.  Different platforms may have different Ethernet interface.

- Correct port assignment:

"/ssm@0,0/pci@19,700000/pci@1/SUNW,qfe@3,1" 3 "qfe"

"/ssm@0,0/pci@19,700000/pci@1/SUNW,qfe@2,1" 2 "qfe"

"/ssm@0,0/pci@19,700000/pci@1/SUNW,qfe@1,1" 1 "qfe"

"/ssm@0,0/pci@19,700000/pci@1/SUNW,qfe@0,1" 0 "qfe"

"/ssm@0,0/pci@19,600000" 3 "pcisch"

"/ssm@0,0/pci@19,600000/network@1" 0 "ce"

"/ssm@0,0/pci@19,600000/network@2" 1 "ce"

- Mis-assigned ports:

"/ssm@0,0/pci@19,700000/pci@1/SUNW,qfe@3,1" 2 "qfe"

"/ssm@0,0/pci@19,700000/pci@1/SUNW,qfe@2,1" 1 "qfe"

"/ssm@0,0/pci@19,700000/pci@1/SUNW,qfe@1,1" 0 "qfe"

"/ssm@0,0/pci@19,700000/pci@1/SUNW,qfe@0,1" 3 "qfe"

"/ssm@0,0/pci@19,600000" 3 "pcisch"

"/ssm@0,0/pci@19,600000/network@1" 0 "ce"

"/ssm@0,0/pci@19,600000/network@2" 1 "ce"

# Task 2: Install signaling gateways and links

- Install required hardware --This requires advance planning to acquire necessary ITP hardware

- .

- ITP software requirement -- When upgrading to 4.5.0, the ITP Signaling Gateway must be running IOS software version 12.2(25)SW1 or later.

- It is required for all customers to have redundant ITP deployment. ITP Signaling Gateways are configured as STPs. The fully redundant SG Mated Pair is the only topology considered in this upgrade. The connection from SG Mated Pair to the SS7 Service Provider is via D-links.

- If a customer have a SS7 network topology different from the one stated above, please contact Cisco support for immediate assistance.

- Please follow steps in Appendix G for introducing ITPs to an existing SS7 network and SS7 transitional states.

# Task 3: Setting up MTP and M3UA configuration on the signaling gateway

Please follow steps specified in Appendix C for configuring ITP signaling gateway.

# Task 4: Create E-links and linksets between BTS and ITP

- Make ITP acts as an STP -- from ITP, create E-links and linksets to BTS with OPC x.x.x. Sample IOS commands are given below for a BTS 10200 with an OPC 7.7.7; and ITP port 5 for SLC 0, port 6 for SLC 1.

cs7 linkset to_bts10200 7.7.7

description linkset to bts10200

link 0 Serial1/1/5:0

link 1 Serial1/1/6:0

no shut

# Task 5: Prepare ITP SCTP configuration command files

Please use the information provided in Appendix D to prepare for the detailed SCTP configuration set up command file. There are two files needed:

- Command file 1: for setting up the SCTP configuration. This is to be performed right before switching over BTS CA/FS applications from Primary Side A in release 3.5.5 to Secondary Side B in release 4.5.0.

- Command file 2: for removing the SCTP configuration. This is to be performed right before falling back BTS CA/FS applications from Secondary Side B in release 4.5.0 to Primary Side A in release 3.5.5.

# Task 6: Purchase and Prepare Disks

Each BTS will require 8 extra disks with matching disk size to swap with the existing system during the upgrade.  The disks taken out can then be recycled.

# Prerequisites

- Four disk drives jumpstarted with Solaris 10 with the other four as mirror disks. Disks must be prepared in a hardware platform that matches the target system.  Please refer to Appendix E disk preparation details

- Two disk drives for EMS side A as a mirrored pair. The first disk is the primary disk and second disk is a mirrored disk. Disks should have:

- Jumpstarted with Solaris 10 OS.

- Staged with BTS 10200 Software Release 4.5.0

- Installed EMS application software and databases

- Two disk drives for EMS side B as a mirrored pair. The first disk is the primary disk and second disk is a mirrored disk. Disks should have:

- Jumpstarted with Solaris 10 OS

- Staged with BTS 10200 Software Release 4.5.0

- Installed EMS application software and databases

- Two disk drives for CA/FS side A as a mirrored pair. The first disk is the primary disk and second disk is a mirrored disk. Disks should have:

- Jumpstarted with Solaris 10 OS

- Staged with BTS 10200 Software Release 4.5.0

- Two disk drives for CA/FS side B as a mirrored pair. The first disk is the primary disk and second disk is a mirrored disk. Disks should have:

- Jumpstarted with Solaris 10 OS

  - Staged with BTS 10200 Software Release 4.5.0

- Locate CD-ROM Disc labeled as “BTSAUTO.tar”

- Locate CD-ROM Disc labeled as “BTS 10200 Application”

- Locate CD-ROM Disc labeled as “BTS 10200 Database”

- Locate CD-ROM Disc labeled as “BTS 10200 Oracle Engine”

- There is secure shell (ssh) access to the Cisco BTS 10200 system.

- There is console access to the Cisco BTS 10200 system.

- Network interface migration from 9/5 to 4/2 has been completed. The interface migration procedure can be found in the following link: http://lbj/push_targets1/ucdit/cc/td/doc/product/voice/bts10200/rel_3_5/upgrade/353/mig95_42.doc

- 4/2 network interface configuration exists.  See Appendix K for reference.

- Verify the target system to be upgraded has the latest 3.5.5 release deployed and the most recent patches applied if any. Please contact Cisco support if you are not sure what patch level the system is on.

- A Network File Server (NFS) accessible from the Cisco BTS 10200 system.

Chapter 3

# Complete 2-4 weeks before the scheduled upgrade

This chapter describes the tasks a user must complete one week before the scheduled upgrade.

Note:  All the scripts being run in this chapter and other chapters will display CLI session output and other output from other operations. These are only for information/logging purpose.

# Task 1: Pre-construct opticall.cfg for the system to be upgraded to 4.5 release

Step 1   Please go to Cisco CCO site in the link provided below and fill in the values in the “NIDS Opticall table” tab http://lbj/push_targets1/ucdit/cc/td/doc/product/voice/bts10200/bts4_5/install/nids_45.xls

Step 2   Once the “NIDS Opticall table” tab is filled out, the Excel spread sheet will generate the release 4.5 opticall.cfg in the “opticall.cfg” tab.

Note:   New parameters added to the 4.5 release:

Note: xxx is the application instance number specific to the site.

- SS7_ENABLED

- IPSEC_ENABLED

- MEM_CFG_SELECTION

- SGW_OPTION

- NTP_SERVERS

- CAxxx_LAF_PARAMETER

- FSPTCyyy_LAF_PARAMETER

- FSAINzzz_LAF_PARAMETER

- EMS_LAF_PARAMETER

- BDMS_LAF_PARAMETER

- DNS_FOR_CA_SIDE_A_BLG_LINK_MONITOR

- DNS_FOR_CA_SIDE_B_BLG_LINK_MONITOR

- DNS_FOR_CAxxx_MGA_COM

Note: This is a FQDN used by a MGA process in Call Agents for communication to Media Gateways. This domain name should return 2 logical IP addresses when system is upgraded to R4.5. Please define the domain name value same as the parameter DNS_FOR_CA_MGCP_COM.  The IP addresses for the domain name DNS_FOR_CA_MGCP_COM will be changed from 4 physical to 2 logical during the upgrade process (See Chapter 5 Task 3)  From each Domain Name Server that is serving the BTS 10200 to be upgraded, the IP addresses of MGCP domain name need to be changed from 4 physical IPs (2 IPs from Primary CA/FS and 2 IPs from secondary CA/FS: 2P + 2S) to 2 physical IPs plus 2 logical (2 IPs from primary CA/FS plus 2 logical IPs:  2P + 2L).  So the IP for the mgcp domain name will be: 2P + 2S  2P + 2L.  After the upgrade to R4.x, only the 2 logical IP addresses will be used and they will always associate with the active CA.  Before the upgrade, you will not be able to ping to the 2 logical IP addresses.

- DNS_FOR_CAxxx_H3A_COM

- DNS_FOR_CAxxx_SIM_COM

- DNS_FOR_CA_SIDE_A_SGA_COM

- DNS_FOR_CA_SIDE_B_SGA_COM

- DNS_FOR_FSAINzzz_ASM_COM

- DNS_FOR_FSAIN_SIDE_A_SGW_COM

- DNS_FOR_FSAIN_SIDE_B_SGW_COM

- DNS_FOR_FSPTCyyy_POTS_COM

- DNS_FOR_FSPTC_SIDE_A_SGW_COM

- DNS_FOR_FSPTC_SIDE_B_SGW_COM

# Add new domain names to DNS

This task must be performed on Domain Name Servers that are serving the Cisco BTS 10200 system.

Step 1   Log in to Domain Name Servers for Cisco BTS 10200

Step 2   Add domain names for the following opticall.cfg parameters to Domain Name Server database where xxx – is the application instance number specific to the site.

- DNS_FOR_CA_SIDE_A_BLG_LINK_MONITOR

Note: This is a qualified domain name used by a LHM process in Call Agents for monitoring network interface status used by billing. This name should return 2 IP addresses of Primary Call Agent.

- DNS_FOR_CA_SIDE_B_BLG_LINK_MONITOR

Note: This is a qualified domain name used by a LHM process in Call Agents for monitoring network interface status used by billing. This name should return 2 IP addresses of Secondary Call Agent.

- DNS_FOR_CAxxx_H3A_COM

Note: This is a qualified domain name used by a H3A process in Call Agents for communication to external devices. This name should return 2 Logical IP addresses. CAxxx – Installed instance for Call Agent

- DNS_FOR_CA_SIDE_A_SGA_COM

Note: This is a qualified domain name used by a SGA process in Call Agents for communication to external devices (ITP). This name should return 2 IP addresses of Primary Call Agent.

- DNS_FOR_CA_SIDE_B_SGA_COM

Note: This is a qualified domain name used by a SGA process in Call Agents for communication to external devices (ITP). This name should return 2 IP addresses of Secondary Call Agent.

- DNS_FOR_CAxxx_SIM_COM

Note: This is a qualified DNS name used by a SIM process in Call Agents for communication to external devices. Each name resolves to two logical IP addresses in the same subnet as primary and secondary interfaces respectively. Each instance must have a unique DNS name and two uniquely associated LOGICAL IP addresses. For a simplex, use the host name for this parameter.

- DNS_FOR_FSAIN_SIDE_A_SGW_COM

Note: This is a qualified domain name used by a TSA process in AIN Feature Server for communication to external devices (ITP). This name should return 2 IP addresses of Primary AIN Feature Server.

- DNS_FOR_FSAIN_SIDE_B_SGW_COM

Note: This is a qualified domain name used by a TSA process in AIN Feature Server for communication to external devices (ITP). This name should return 2 IP addresses of Secondary AIN Feature Server.

- DNS_FOR_FSAINzzz_ASM_COM

Note: This is a qualified DNS name used by the AIN process in Feature Server FSAIN. Each name should return two logical IP addresses of an AIN Feature Server which match the subnet of its two physical interfaces. For a simplex system, use host name for this parameter.

- DNS_FOR_FSPTC_SIDE_A_SGW_COM

Note: This is a qualified domain name used by a TSA process in POTS Feature Server for communication to external devices (ITP). This name should return 2 IP addresses of Primary POTS Feature Server.

- DNS_FOR_FSPTC_SIDE_B_SGW_COM

Note: This is a qualified domain name used by a TSA process in POTS Feature Server for communication to external devices (ITP). This name should return 2 IP addresses of Secondary POTS Feature Server.

- DNS_FOR_FSPTCyyy_GFS_COM

Note: This is a qualified domain name used by GFS module of the POTS process in POTS Feature Server for communication to external devices. This name should return 2 Logical IP addresses. FSPTCyyy – Installed instance for POTS feature server

- DNS_FOR_FSPTCyyy_POTS_COM

Note: This is a qualified DNS name used by the POTS process in Feature Server FSPTC. Each name should return two logical IP addresses of a POTS/CENTRIX Feature Server which match the subnet of its two physical interfaces. For a simplex system, use host name for this parameter.

# Task 2: Save customized cron jobs

This upgrade process requires disk replacement. Because of this, all customized cron jobs in the system will be lost. Please save the cron jobs to your network file servers to be restored once the entire system is upgraded to the 4.5 release.

# Task 3: Collecting network and IP information

# From EMS side A

Step 1   Log in as root

Step 2   Record the IP address and netmask for the management interface of the system.

- For an example, if the “hme0” is used for management interface, then execute the following command:

<hostname># ifconfig qfe0

- Record the IP address and netmask for the interface to be used in the next task.

IP: _____________ Netmask: ____________ Interface Name: ___________

# From CA/FS side A

Step 1   Log in as root

Step 2   Record the IP address and netmask for the management interface of the system.

- For an example, if the “hme0” is used for management interface, then execute the following command:

<hostname># ifconfig qfe0

- Record the IP address and netmask for the interface to be used in the next task.

IP: _____________ Netmask: ____________ Interface Name: ___________

# From EMS side B

Step 1   Log in as root

Step 2   Record the IP address and netmask for the management interface of the system.

- For an example, if the “hme0” is used for management interface, then execute the following command:

<hostname># ifconfig qfe0

- Record the IP address and netmask for the interface to be used in the next task.

IP: _____________ Netmask: ____________ Interface Name: ___________

# From CA/FS side B

Step 1   Log in as root

Step 2   Record the IP address and netmask for the management interface of the system.

- For an example, if the “hme0” is used for management interface, then execute the following command:

<hostname># ifconfig qfe0

- Record the IP address and netmask for the interface to be used in the next task.

IP: _____________ Netmask: ____________ Interface Name: ___________

# Task 4: Stage upgrade programs

# From EMS Side A, EMS Side B, CA/FS Side A and

# CA/FS Side B

Step 1   Locate CD-ROM disc labeled as BTSAUTO.tar and put disc in the CD-ROM drive

Step 2   Create /cdrom directory and mount the directory.

<hostname># mkdir -p /cdrom

<hostname># mkdir -p /opt/Build

- If the hardware is Continuous AX-MP and AX-I, and Sun Netra, please use:

<hostname># mount -o ro -F hsfs /dev/dsk/c0t6d0s0 /cdrom

- All other hardware types, please use:

<hostname># mount -o ro -F hsfs /dev/dsk/c0t0d0s0 /cdrom

Step 3   Use the following commands to copy file from the CD-ROM to the /opt directory:

<hostname># cp –f /cdrom/BTSAUTO.tar /opt/Build

Step 4   Unmount the CD-ROM.

<hostname># umount /cdrom

Step 5   Manually eject the CD-ROM and take out the disc from CD-ROM drive

Note:   At this point you may also sftp the BTSAUTO.tar to the other machines instead of mounting and reading from the CD in each machine.

Step 6   Extract tar files.

<hostname># cd /opt/Build

<hostname># tar -xvf BTSAUTO.tar

# Task 5: Prepare configuration files

# Stage Release 4.5 opticall.cfg

Place the opticall.cfg on Network File Server (NFS) that is accessible from the target system to be upgraded.

# Prepare SS7 CLI script and for migration testing

# From EMS Side A

Please choose one of the two options explained below to prepare the SS7 CLI script:

OPTION 1

Produce the file manually:

- Please use the example template in Appendix H and Appendix I to construct the script file. Then named the file as cfg_ss7 and place the file on Primary EMS Side A under directory /opt/.upgrade/cli/ss7. Please create the directory if it does not exist.

<hostname> cd /opt/.upgrade

<hostname> mkdir cli

<hostname> cd cli

<hostname> mkdir ss7

OPTION 2

Produce the template file using automated script program:

  - The variables listed in the Table 1 below are the summary list of parameters required. .  Please define the value for parameters listed in the table.

  - The automation program to generate SS7 configuration CLI file needs the input data in a file, called SS7.cfg, residing in the same location as upgrade automation program. Fill in each parameter specified in SS7.cfg file with the values from the Table 1. The SS7.cfg file will then contain all the parameters related to Sigtran SS7 configuration, e.g. signaling gateway ids, signaling group ids, origin point codes, destination point codes etc. Then run the automated script program to generate a template file to be customized for each BTS 10200 production system.

Table 1: SS7.cfg Parameter List

| # | Parameter | Description | Value |
| --- | --- | --- | --- |
|  | ITP1_TSAP_ADDR_1 | First IP address for ITP #1 |  |
|  | ITP1_TSAP_ADDR_2 | Second IP address for ITP #1 |  |
|  | ITP2_TSAP_ADDR_1 | First IP address for ITP #2 |  |
|  | ITP2_TSAP_ADDR_2 | Second IP address for ITP #2 |  |
|  | SG_ID_1 | First signaling gateway ID |  |
|  | SG_ID_2 | Second signaling gateway ID |  |
|  | SGP_ID_1 | First signaling gateway process ID |  |
|  | SGP_ID_2 | Second signaling gateway process ID |  |
|  | SG_GRP_ID | Signaling gateway group ID |  |
|  | SCTP_PROF | SCTP association profile ID |  |
|  | CA_SCTP_ASSOC_1 | First SCTP association ID for Call Agent |  |
|  | CA_SCTP_ASSOC_2 | Second SCTP association ID for Call Agent |  |
|  | OPC_ID | OPC ID |  |
|  | OPC_POC | OPC Point code |  |
|  | RK_ID_1 | Routing Key ID |  |
|  | RC_ITP | ITP routing context value |  |
|  | DPC_ID_1 | First DPC ID |  |
|  | DPC_ID_2 | Second DPC ID |  |
|  | DPC_POC_1 | First DPC point code |  |
|  | DPC_POC_2 | Second DPC point code |  |
|  | ROUTE_DPC_ID_1 | Call control route ID for the first DPC |  |
|  | ROUTE_DPC_ID_2 | Call control route ID for the second DPC |  |
|  | AIN_SCTP_ASSOC_1 | First SCTP association ID for AIN |  |
|  | AIN_SCTP_ASSOC_2 | Second SCTP association ID for AIN |  |
|  | POT_SCTP_ASSOC_1 | First SCTP association ID for POTS |  |
|  | POT_SCTP_ASSOC_2 | Second SCTP association ID for POTS |  |
|  | SCCP_NW_ID | SCCP network ID |  |
|  | POP_ID | POP ID |  |
|  | RK_CNAM | Routing key ID for CNAM service |  |
|  | DPC_CNAM_ID | DPC ID for CNAM service |  |
|  | POC_CNAM | DPC point code for CNAM service |  |
|  | SSGRP_CNAM | Subsystem group ID for CNAM service |  |
|  | SS_CNAM | Subsystem ID for CNAM service |  |
|  | RC_CNAM | Routing context value for CNAM service |  |
|  | SLHR_CNAM | SLHR ID for CNAM service |  |
|  | RK_LNP | Routing key ID for LNP service |  |
|  | DPC_LNP_ID | DPC ID for LNP service |  |
|  | POC_LNP | DPC point code for LNP service |  |
|  | SSGRP_LNP | Subsystem group ID for LNP service |  |
|  | SS_LNP | Subsystem ID for LNP service |  |
|  | RC_LNP | Routing context value for LNP service |  |
|  | SLHR_LNP | SLHR ID for LNP service |  |
|  | RK_TF | Routing key ID for Toll Free service |  |
|  | DPC_TF_ID | DPC ID for Toll Free service |  |
|  | POC_TF | DPC point code for Toll Free service |  |
|  | SSGRP_TF | Subsystem group ID for Toll Free service |  |
|  | SS_TF | Subsystem ID for Toll Free service |  |
|  | RC_TF | Routing context value for Toll Free service |  |
|  | SLHR_TF | SLHR ID for Toll Free service |  |
|  | RK_AINTF | Routing key ID for AIN Toll Free service |  |
|  | DPC_AINTF_ID | DPC ID for AIN Toll Free service |  |
|  | POC_AINTF | DPC point code for AIN Toll Free service |  |
|  | SSGRP_AINTF | Subsystem group ID for AIN Toll Free service |  |
|  | SS_AINTF | Subsystem ID for AIN Toll Free service |  |
|  | RC_AINTF | Routing context value for AIN Toll Free service |  |
|  | SLHR_AINTF | SLHR ID for AIN Toll Free service |  |
|  | RK_ACAR | Routing key ID for ACAR service |  |
|  | DPC_ACAR_ID | DPC ID #1 for ACAR service |  |
|  | DPC_ACAR_ID_2 | DPC ID #2 for ACAR service |  |
|  | DPC_ACAR_ID_3 | DPC ID #3 for ACAR service |  |
|  | POC_ACAR | DPC point code #1 for ACAR service |  |
|  | POC_ACAR_2 | DPC point code #2 for ACAR service |  |
|  | POC_ACAR_3 | DPC point code #3 for ACAR service |  |
|  | SSGRP_ACAR | Subsystem group ID for ACAR service |  |
|  | SS_ACAR | Subsystem ID for ACAR service |  |
|  | RC_ACAR | Routing context value for ACAR service |  |
|  | SLHR_ACAR | SLHR ID for ACAR service |  |

Step 1   Log in as root

Step 2	  <hostname># cd /opt/Build/btsauto

- Edit SS7.cfg and fill in each parameter with the value defined in the Table 1 above.

Step 3   Generate CLI file

<hostname># create_SS7_ITP_cfg.sh

- Above script program will generate the SS7 CLI script file for provision Sigtran on BTS 10200. The SS7 CLI script will be generated under directory /opt/.upgrade/cli/ss7 with file named as cfg_ss7.

Following are some of the salient points of the program that generates CLI file.

1. The file SS7.cfg provides the scalability by following a naming convention so that the generated CLI file can resemble the desired configuration. For example, we can define as many destination point codes as desired by appending a numerical suffix to DPC_POC_, e.g. DPC_POC_1, DPC_POC_2, DPC_POC_3 etc. The parameters that can be scaled up are as follows.

DPC_POC_ : Destination point codes

DPC_ID_ : Destination point code ids

POP_ID_ : POP ids

2. Though the automation program utilizes the input file SS7.cfg, it will generate CLI file even if the parameter is not assigned a value in input file. In that case, the resulting CLI will have a default value in it for that parameter. For example, sctp profile id is required for the CLI command 'add sctp-assoc-profile id=<sctp profile id>;'. The parameter used here is 'id' and is denoted by 'SCTP_PROF' in SS7.cfg file. If a value 'cisco-profile' is assigned to this parameter, the generated CLI will look something similar to the following.

add sctp-assoc-profile id=cisco-profile;

Otherwise, if no value is assigned to SCTP_PROF, a default value 'sctp-prof' will be assigned to the id and generated CLI will look something similar to the following.

add sctp-assoc-profile id=sctp-prof;

Likewise, all the parameters of SS7.cfg have a default value. Hence, extrapolating the above scenario, if one does not assign value to any parameter in the SS7.cfg file, the automation program will still generate a CLI file which will act like a template with default values for all the parameters. The customer is required to modify this template according to their needs.

3. There are 2 possible values for the parameter TOLL_FREE_TYPE, one id 'AIN' and the other is 'IN1'. The default, if nothing is defined, is 'IN1'. Depending on this value, the toll free service is defined.

4. The following destination point code id values should be one of the parameter values of DPC_ID_1, DPC_ID_2, DPC_ID_3 etc.

CC_DPC_ID_1, CC_DPC_ID_2

DPC_LNP_ID

DPC_AINTF_ID

DPC_ACAR_ID

DPC_TF_ID

DPC_CNAM_ID

In other words, new destination point code ids should NOT be defined for the above listed parameters.

5. The CLI generation program will also generate a file testSS7.cfg that is utilized for SS7 upgrade testing, performed inside upgrade window. This file is a collection of parameters which are required for testing.

# Prepare for SS7 migration testing

# From EMS Side A

Once the secondary side is upgraded to 4.5, the upgrade script needs to test the status of SS7 migration. The configuration file /opt/Build/btsauto/testSS7.cfg created by the script program above, create_SS7_ITP_cfg.sh, is for this purpose. If you create the cfg_ss7 file manually, you should edit the generic testSS7.cfg file and save the file under /opt/Build/btsauto directory.

Testing is performed against the following.

- SCTP associations for CA, AIN and POTS

- Signaling processes

- Destination point codes for CA and FSAIN

- Subsystems

The file testSS7.cfg incorporates a simple naming convention to make it flexible to test as many parameters as desired. For example, if the user wants more number of sctp associations tested for CA, then define more of SCTP_ASSOC_ID_CAs as SCTP_ASSOC_ID_CA_1, SCTP_ASSOC_ID_CA_2, SCTP_ASSOC_ID_CA_3 etc.

A template for testSS7.cfg file looks like the following. View the testSS7.cfg file to verify.

# Signaling group IDs for SCTP association

SCTP_ASSOC_ID_CA_1=ca-sctp-assoc

SCTP_ASSOC_ID_CA_2=ca-sctp-assoc-2

SCTP_ASSOC_ID_FSAIN_1=ain-sctp-assoc-1

SCTP_ASSOC_ID_FSAIN_2=ain-sctp-assoc-2

SCTP_ASSOC_ID_POTS_1=pots-sctp-assoc-1

SCTP_ASSOC_ID_POTS_2=pots-sctp-assoc-2

# Signaling process IDs

SGP_ID_1=sgp1

SGP_ID_2=sgp2

# Destination point code IDs

DPC_ID_CA_1=dpc1		=Destination point code of PSTN (the far end callcontrol)

DPC_ID_CA_2=dpc2		=Destination point code of PSTN (the far end callcontrol)

DPC_ID_FSAIN_1=dpc3 	=Destination point code of PSTN (CNAM service)

DPC_ID_FSAIN_2=dpc4	=Destination point code of PSTN (LNP service)

DPC_ID_FSAIN_3=dpc5	=Destination point code of PSTN (Toll Free service)

DPC_ID_FSAIN_4=dpc6	=Destination point code of PSTN (AIN Toll Free service)

ITP_AR=itp1			=Destination point code of PSTN (ACAR service)

# Subsystem Ids.

SS_ID_CNAM=ss_cnam

SS_ID_LNP=ss_lnp

SS_ID_TF=ss_tf

SS_ID_AINTF=ss_aintf

SS_ID_ACAR=ss_acar

OPC_ID=opc

NOTE: If above values are not correct, then SS7 test after migration will fail.

# Prepare the critical SS7 trunk group file

# From EMS Side A

During the SS7 migration from 3.5.5 to 4.5, the trunk groups are taken Out of Service in bulk and later taken into service in bulk. However user can define a prioritized trunk group list and given this list the upgrade process will take these trunk groups out of service at the end and take them into service first. The prioritized trunk groups needs to be listed in the file /opt/Build/btsauto/critical_trunk_grp.txt.

Step 1   <hostname># vi /opt/Build/btsauto/critical_trunk_grp.txt

- Fill in the trunk group ids in the order of priority, highest priority trunk group id being listed at the end of the file.

A sample /opt/Build/btsauto/critical_trunk_grp.txt file will look as follows for specifying the 10 priority trunk groups

########################################################################

# Copyright (c) 2002-2005 by Cisco Systems, Inc.

###### This file contains the important trunk group ids #############

###### Any id specified in this file is considered as critical ########

##### and will be taken OOS last and taken INS first         ########

6

5

3

4

1

2

7

9

8

10

Note: In this file of priority trunk groups, trunk group 10 is of highest priority. The entries 1,2….10 are the sample trunk group ids.

# Task 6: Prepare Scripts for post-upgrade new feature activation

Cisco recommends that new POTS/Centrex features to be activated only when the upgrade to release 4.5 is successful and no fallback will be needed. A separate maintenance window maybe required to activate the new features. This should be done within 1 week after completing the release software upgrade.

# Create announcement CLI file

Step 1	Create new announcements and place them to the proper media gateways using Appendix F. New announcements should either be created by each customer or provided by Cisco systems.

Step 2	Run the program to generate new announcement CLI file

Following script will generate the new announcement CLI file to be used after upgrade for provisioning. The generated CLI file location is notified at the completion of program.

# From EMS Side A

Step 1	 Log in as root user

Step 2	 <hostname># cd /opt/Build/btsauto

Step 3   <hostname># create_announcement_cli.sh

This script will ask user to upload the announcement files to the Announcement Media Gateways and waits for user to confirm.

# Create CLI file for new feature creation and existing feature update

# From EMS Side A

Step 1	Run the program to generate new feature CLI file

Following script will generate the new feature CLI file to be used after upgrade for provisioning. The generated CLI file location is notified at the completion of program.

<hostname># create_callfwd_cli.sh

# Task 7: Pre-checking database integrity and correction

# From EMS Side A

Step 1   Check billing flag in the call-agent-profile table

- Log in to CLI

- CLI> show call-agent-profile

  - If there is no record shown, you mush add a new record with “cdb-billing-supp” flag set to “y”.

  - If there is a record and the record shown with “cdb-billing-supp” flag set to “n”, you must change the value to “y”.

Step 2    Provision 3.5.5 with the DIVERSION-HEADER-SUPP with the correct value which can be carried over to 4.5:

- In 3.5.5, make sure the voice mail server support diversion_header_supp since cc_diversion_supp is not supported in 4.5 release.

- Re-provision the softsw-tg-profile with cc_diversion_supp=N and diversion_header_supp=Y before upgrade.

- Exit the CLI when provision is done.

Step 3    Check trunk_grp table to identify the trunk groups without associated terminations

- <hostname># su – oracle

- <hostname>$ sqlplus optiuser/optiuser

- sql> select id from trunk_grp where tg_type=’SS7’ and not exists (select 1 from termination where tgn_id=trunk_grp.id);

  - For any trunk group record shown above, you have to either delete the trunk-grp record or assign a proper termination. Failure to do so will result in total loss of SS7 call traffic.

  - If there are no trunk groups without terminations, the sql command returns "no rows selected".

- sql> exit

- <hostname>$ exit

Step 4    Run the program to perform data integrity check to ensure the upgrade process can go through smoothly.

<hostname># cd /opt/Build/btsauto

<hostname># plugins/03.05/bts_db_pre_check.sh

- Note: This script may also change these tables to make sure that the values are consistent for upgrade.

- If errors are generated, please try to correct the errors and run the check again.

# Task 8: Store the pre-constructed 4.5 opticall.cfg file

# From EMS Side A

Step 1   <hostname># mkdir -p /opt/.upgrade/cfg

Step 2   Stage the opticall.cfg created in Chapter 3, Task 5 under the /opt/.upgrade/cfg directory on EMS side A.

# Task 9: Prepare IOS commands

Prepare IOS commands for removing SS7 E-links and linksets on the ITP which will be used during the upgrade so that traffic will go over SIGTRAN.  Also, prepare IOS commands for setting up SCTP configuration from ITPs.

An example of these IOS commands for removing SS7 links and linksets are as follows, where 7.7.7 is the point code of the BTS:

cs7 linkset to_bts10200 7.7.7

shut

no cs7 linkset to_bts10200 7.7.7

# Task 10: Verify console server lines to BTS do not have a timeout entry.

Remove the default session timeout entry for the lines to BTS.  Do a “show run” from your terminal server to verify that the lines which are being used to access the BTS have no session-timeout entry.

# Task 11: Add logical MGCP domain name IP addresses to MGX/VISM

Please skip the following steps if MGX/VISM is not being used. The following is an example session logging into the MGX and adding the additional IP addresses to VISM cards.  In this case we are adding two additional logical IPs for the R4.5 upgrade. The two commands used are dpsdnallips and adddnip

Refer to Appendix G for the procedure to add the logical mgcp domain name IP addresses referred to in Chapter 3 Task 5 Step 3 to the MGX/VISM .

spiderma.1.28.VISM8.a > dspdnallips

ResolutionIndex    DomainName                     IP           State    Pref

---------------   ------------                ------------   ---------  -----

1         ca-mgcp.twc.cisco.com         13.200.10.11    Active     1

2         ca-mgcp.twc.cisco.com         13.200.20.11    Inactive   2

3         ca-mgcp.twc.cisco.com         13.200.10.12    Inactive   3

4         ca-mgcp.twc.cisco.com         13.200.20.12    Inactive   4

spiderma.1.28.VISM8.a > adddnip 5 ca-mgcp.twc.cisco.com 13.200.10.13 5

spiderma.1.28.VISM8.a > adddnip 6 ca-mgcp.twc.cisco.com 13.200.20.13 6

spiderma.1.28.VISM8.a > dspdnallips

ResolutionIndex    DomainName                     IP           State    Pref

---------------   ------------                ------------   ---------  -----

1         ca-mgcp.twc.cisco.com         13.200.10.11    Active     1

2         ca-mgcp.twc.cisco.com         13.200.20.11    Inactive   2

3         ca-mgcp.twc.cisco.com         13.200.10.12    Inactive   3

4         ca-mgcp.twc.cisco.com         13.200.20.12    Inactive   4

5         ca-mgcp.twc.cisco.com         13.200.10.13    Inactive   5

6         ca-mgcp.twc.cisco.com         13.200.20.13    Inactive   6

# Task 12: Verify value of CA CONTROL PORT for IVR Devices connected to BTS.

# Note : The value of the CA_CONTROL_PORT for IVR devices connected to BTS should not be 0.

Note : IVR services may be interrupted during this task.

# From Active EMS

Step 1   Login as “btsuser” User.

Step 2   Find the ANNC trunk-grp. The one with the main subscriber should be an IVR trunk-grp

CLI > show trunk_grp tg-type=ANNC;

ID=80031

CALL_AGENT_ID=CA146

TG_TYPE=ANNC

NUM_OF_TRUNKS=30

TG_PROFILE_ID=ivr-ipunity

STATUS=INS

DIRECTION=BOTH

SEL_POLICY=ASC

GLARE=SLAVE

ALT_ROUTE_ON_CONG=N

SIGNAL_PORTED_NUMBER=N

MAIN_SUB_ID=806-888-2000

DEL_DIGITS=0

TRAFFIC_TYPE=LOCAL

ANI_BASED_ROUTING=N

MGCP_PKG_TYPE=ANNC_CABLE_LABS

ANI_SCREENING=N

SEND_RDN_AS_CPN=N

STATUS_MONITORING=N

SEND_EARLY_BKWD_MSG=N

EARLY_BKWD_MSG_TMR=5

SCRIPT_SUPP=N

VOICE_LAYER1_USERINFO=AUTO

VOICE_INFO_TRANSFER_CAP=AUTO

PERFORM_LNP_QUERY=N

Step 3:  Ensure that subscriber CATEGORY is set to IVR.

CLI>show sub id=806-888-2000

ID=806-888-2000

CATEGORY=IVR

NAME=tb06 806-888-2000

STATUS=ACTIVE

DN1=8068882000

PRIVACY=NONE

RING_TYPE_DN1=1

TGN_ID=80031

PIC1=NONE

PIC2=NONE

PIC3=NONE

GRP=N

USAGE_SENS=N

SUB_PROFILE_ID=tb06-ivr-1

TERM_TYPE=ROUTE

POLICY_ID=80031

IMMEDIATE_RELEASE=N

TERMINATING_IMMEDIATE_REL=N

SEND_BDN_AS_CPN=N

SEND_BDN_FOR_EMG=N

SEND_BDN_AS_CPN=N

SEND_BDN_FOR_EMG=N

PORTED_IN=N

BILLING_TYPE=NONE

VMWI=Y

SDT_MWI=Y

Step 4: Use the trunk-grp id to find the trunks that have the IVR gateway ID

CLI>show trunk tgn-id=80031;

ID=1

TGN_ID=80031

TERM_ID=ivr/1

MGW_ID=ipunity-227-103

ID=2

TGN_ID=80031

TERM_ID=ivr/2

MGW_ID=ipunity-227-103

ID=3

TGN_ID=80031

TERM_ID=ivr/3

MGW_ID=ipunity-227-103

ID=4

TGN_ID=80031

TERM_ID=ivr/4

MGW_ID=ipunity-227-103

ID=5

TGN_ID=80031

TERM_ID=ivr/5

MGW_ID=ipunity-227-103

ID=6

TGN_ID=80031

TERM_ID=ivr/6

MGW_ID=ipunity-227-103

Step 5: Show IVR Gateway to verify CALL_AGENT_CONTROL PORT value

CLI > show mgw id = ipunity-227-103

ID=ipunity-227-13

TSAP_ADDR=ms-ipunity.ipclab.cisco.com

CALL_AGENT_ID=CA146

MGW_PROFILE_ID=ivr-ipunity

STATUS=INS

CALL_AGENT_CONTROL_PORT=0

TYPE=TGW

Reply : Success: Entry 1 of 1 returned.

Note: If the CALL_AGENT_CONTROL_PORT is set to 0 continue to step 6. If the value of the CALL_AGENT_CONTROL_PORT set to a value other than 0 like 2427 or 2428, do not execute further steps in this task.

Step 6: Control the IVR gateway to OOS state.

Note: Make sure that no calls exist on these trunks associated to the gateway.

CLI > status tt tgn-id=<IVR trunk-grp id>; cic=all.

80031  1   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

80031  2   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

80031  3   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

In the example output above note that the state of the endpoint/CIC is IDLE.

CLI>control mgw id=ipunity-227-103;mode=forced;target_state=oos;

MGW ID -> ipunity-227-103

INITIAL STATE -> ADMIN_INS

REQUEST STATE -> ADMIN_OOS

RESULT STATE -> ADMIN_OOS

FAIL REASON -> ADM found no failure

REASON -> ADM executed successfully

RESULT -> ADM configure result in success

Step 7: Change the CALL_AGENT_CONTROL_PORT for the IVR gateway

CLI>change mgw id=ipunity-227-103;CALL_AGENT_CONTROL_PORT =2427

Step 8: Control the IVR gateway to the INS state.

CLI>control mgw id=ipunity-227-103;mode=forced;target_state=ins

MGW ID -> ipunity-227-103

INITIAL STATE -> ADMIN_OOS

REQUEST STATE -> ADMIN_INS

RESULT STATE -> ADMIN_INS

FAIL REASON -> ADM found no failure

REASON -> ADM executed successfully

RESULT -> ADM configure result in success

Step 9: Control the trunk-termination for the IVR gateway to the INS state.

CLI>control tt tgn_id=80031;mode=forced;cic=all; target-state=ins

REQUEST STATE -> ADMIN_INS

RESULT STATE -> ADMIN_INS

FAIL REASON -> ADM found no failure

REASON -> ADM executed successfully

RESULT -> ADM configure result in success

TGN ID -> 80031

CIC START -> 1

CIC END -> 30

Step 10: Make sure the IVR trunk-terminations are in the INS state.

CLI>status tt tgn-id=80031;cic=all

80031  1   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

80031  2   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

80031  3   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

80031  4   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

80031  5   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

80031  6   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

80031  7   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

80031  8   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

80031  9   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

CLI > Exit

Chapter 4

Prepare System for Upgrade

This chapter describes the steps a user must complete at least 24 hours before the scheduled upgrade but not more than 48 hours.

To shorten the upgrade window, Cisco recommends the full database audit be performed at the night before the upgrade if it is absolutely certain that no provisioning activities occur during the next 24 hour period. If the database audit is not done, the upgrade procedure will perform the audit and it will take more time for the upgrade procedure.

# Task 1: Audit subscriber database

# From Active EMS

Step 1   Login to Active EMS as root

Step 2   Perform a full audit of the database

<hostname># su – optiuser

CLI > audit database type=full;

- Please ignore status mismatches if any for the following tables:

  - Feature-server

  - Call-agent

  - Termination

- Please ignore “OPC” mismatches if any for the SS7-CIC tables.

- Please fix database mismatches resulted from audit. Then perform the audit again until it is clean to start the next upgrade task.

NOTE: It may take up to many hours to complete the database audit depending on the size of the subscriber database.  If audit shows any problem correct the problem and re audit.

# Task 2: Check Oracle Database Replication and Error Correction

Perform the procedure in Appendix J on the Active EMS side A to check the Oracle database and replication status.

Chapter 5

Start Upgrade Process

To shorten the upgrade window, Cisco recommends the full database audit be performed at the night before the upgrade if it is absolutely certain that no provisioning activities occur during the next 24 hour period. If the database audit is not done, the upgrade procedure will perform the audit and it will take more time for the upgrade procedure.

Suspend all subscribers provisioning activity during the entire upgrade process.

# Task 1: Saving Shared Memory

# From Active EMS Side

Step 1   Log in to Active Side EMS as CLI user

Step 2   CLI> control call-agent id=CAxxx; target-state=forced-standby-active;

Step 3   CLI> control feature-server id=FSPTCyyy; target-state=forced-standby-active;

Step 4   CLI> control feature-server id=FSAINzzz; target-state=forced-standby-active;

Step 5   CLI> exit

# From CA/FS Side A

Step 1   Login as root

Step 2   Bring down the application platforms

<hostname># platform stop

Step 3    Save shared memory

<hostname># mkdir -p /opt/databackup

<hostname># cp -rfp /opt/OptiCall/CAxxx/bin/data /opt/databackup/data.CA

<hostname># cp -rfp /opt/OptiCall/FSAINzzz/bin/data /opt/databackup/data.FSAIN

<hostname># cp -rfp /opt/OptiCall/FSPTCyyy/bin/data /opt/databackup/data.FSPTC

Step 4   Bring up the application platforms

<hostname># platform start

- Please allow 10 minutes for Call Agent platform to stabilize before making an attempt to switchover.

# Task 2: Change MGCP domain IP addresses and set Time-to-Live (TTL) value from DNS

# Change MGCP domain IP addresses from DNS

Step 1   DNS changes

From each Domain Name Server that is serving the BTS 10200 to be upgraded, change the IP addresses of MGCP domain name (domain name for DNS_FOR_CA_MGCP_COM from /etc/opticall.cfg) from 4 physical IPs (2 IPs from Primary CA/FS and 2 IPs from secondary CA/FS: 2P + 2S) to 2 physical IPs plus 2 logical IPs (2 IPs from primary CA/FS plus 2 logical IPs:  2P + 2L).  So the IPs for MGCP domain name is changed: 2P + 2S  2P + 2L.

Please make sure the DNS settings for IPs are:

- In round-robin fashion

- Interlaced (A physical IP is followed by a logical IP)

- 600 seconds of Time-to-Live (TTL) value

Please refresh the DNS cache and make sure the new IP addresses are taking affect.

The logical IP address is a new 4.5 feature that allows these IP addresses to follow the active CA.  An external MGCP device only needs to maintain the logical IP addresses to communicate with the active CA via MGCP.

Step 2   Verify the DNS changes

To verify the change has taken affect, please log in to both CA/FS machines and perform “nslookup” of the MGCP domain name.

For an example:

<hostname># nslookup ca-mgcp.twc.cisco.com

…

Name:    ca-mgcp.twc.cisco.com

Addresses:  13.200.10.11, 13.200.10.13, 13.200.20.11, 13.200.20.13

<hostname># ping 13.200.10.11

13.200.10.11 is alive             physical IP

<hostname># ping 13.200.10.13

no answer from 13.200.10.13       logical IP

<hostname># ping 13.200.20.11

13.200.20.11 is alive             physical IP

<hostname># ping 13.200.20.13

no answer from 13.200.20.13       logical IP

Note:  After the upgrade to release 4.5, only the 2 logical IP addresses will be used and they will always associate with the active CA.  Before the upgrade, you will not be able to ping to the 2 logical IP addresses.

# Verify Time-to-Live (TTL) from Primary CA/FS

Step 1   Login to primary CA/FS as root

Step 2   Check TTL setting. Please use the actual Domain Name for parameter DNS_FOR_CA_MGCP_COM from /etc/opticall.cfg.

For an example:

- The MGCP Domain Name “ca-mgcp.twc.cisco.com” is configured with 2 physical IPs (from Primary CA/FS) and 2 logical IPs as configured in Chapter 3, Task 5.

- Verify the TTL for all 4 IPs is 600 seconds

<hostname># nslookup -debug ca-mgcp.twc.cisco.com

- The TTL will be shown in the ANSWERS section as: ttl = <time to live in seconds>

…

ANSWERS:

->  ca-mgcp.twc.cisco.com

internet address = 13.200.10.11              Primary CA/FS physical IP

ttl = 600 (10M)

->  ca-mgcp.twc.cisco.com

internet address = 13.200.10.13              logical IP

ttl = 600 (10M)

->  ca-mgcp.twc.cisco.com

internet address = 13.200.20.11              Primary CA/FS physical IP

ttl = 600 (10M)

->  ca-mgcp.twc.cisco.com

internet address = 13.200.20.13              logical IP

ttl = 600 (10M)

# Task 3: Change the root password

Make sure root password on each BTS machine is changed to ‘opticall’

Example:

<hostname># passwd

passwd:  Changing password for root

New password: opticall

Re-enter new password: opticall

# Task 4: Force Side A Systems to be Active

# From Active EMS Side

Step 1   Login to Active Side EMS as CLI user

Step 2   CLI> control call-agent id=CAxxx; target-state=forced-active-standby;

Step 3   CLI> control feature-server id=FSPTCyyy; target-state=forced-active-standby;

Step 4   CLI> control feature-server id=FSAINzzz; target-state=forced-active-standby;

Step 5   CLI> control bdms id=BDMS01; target-state=forced-active-standby;

Step 6   CLI> control element-manager id=EM01; target-state=forced-active-standby;

# Task 5: Start upgrade process

# From EMS Side A

Step 1   From CLI prompt execute the following commands to prepare QOS table for Release 4.5.

CLI> show ca-config type=DEFAULT-QOS-ID;

  - Verify the VALUE for the ca-config record:

    - TYPE=DEFAULT-QOS-ID

    - DATATYPE=STRING

    - VALUE=[default | DEFAULT]

CLI> show qos id=[default | DEFAULT];

  - If there is no record found, add it using the following command setting the parameters to match the ca-config value for VALUE=

CLI> add qos id=[default | DEFAULT];

CLI> exit

Step 2    Start the upgrade process:

<hostname># cd /opt/Build/btsauto

<hostname># bts_upgrade.exp –diskSwap

Note:  If the following error occurs, please follow the procedure in Appendix J to fix and then rerun the above diskSwap command.

==========================================================

ERROR: ORACLE REPLICATION NOT IN SERVICE

==========================================================

Step 4   If the program asks you for the default password, then for some reason it is not set as “opticall”.  This is a check, and at this point you will need to change the root passwords to be “opticall”, as mentioned earlier.

Step 5	  From this point onwards the upgrade process is self driven. Please follow the upgrade process to perform any manual steps and inputs as directed.

You will be prompted for input at various points of the upgrade.  Please answer “y” to continue the upgrade if you don’t see any error.  Otherwise please contact Cisco Support.

Below are some example displays and checkpoints that will appear on your terminal while the upgrade script is running.

priems58# bts_upgrade.exp -diskSwap

Tue Sep 27 16:19:35 CDT 2005 : Starting BTS upgrade process.

Upgrade invocation command: bts_upgrade.exp –diskSwap

*** CHECKPOINT healthCheck ***

*** CHECKPOINT alarmCheck ***

get_node_status

active

priems58# check_alarms

Checking for outstanding major/critical alarms.

==========================================================

ERROR: # of major/critical alarms = 1

==========================================================

**********************************************

It is not advisable to continue with outstanding major/critical alarms.

Please clear the alarms before continuing.

**********************************************

Question: Do you want to continue (y/n)? [n] y

*** CHECKPOINT backupOriState ***

*** CHECKPOINT prestopApps ***

*** CHECKPOINT functionalityChk ***

*** CHECKPOINT backupPriEM ***

*** CHECKPOINT backupPriCF ***

*** CHECKPOINT backupSecEM ***

*** CHECKPOINT backupSecCF ***

*** CHECKPOINT updateCFG ***

*** CHECKPOINT setPriEmsSimplex ***

*** CHECKPOINT switchToSideA1 ***

*** CHECKPOINT stopSecApps ***

The automated upgrade process will halt for the user confirmation to proceed at the time of Side B disk removal.  When the following message is displayed, DO NOT remove the disks yet, but proceed to Task 6 and Task 7.

“========================================================

Please remove the disks on <secems> and <secca> and insert new disks.

Your connection will be lost and need to relogin after the machine is up with the new disk

Please enter y when you are ready to reconnect to <secems>

Are you ready to reconnect? [y]”

Note:   At this point, please don’t answer and enter any commands. It is recommended to perform Task 6 and Task 7 in parallel to shorten the upgrade time.  Upon the completion of Task 6 and Task 7, proceed with the upgrade process by pressing ‘Enter’ or typing ‘y’.

# Task 6: Upgrade EMS side B to the new release

|  | Warning Do not proceed if you don’t have a pre-constructed opticall.cfg file for the system. The opticall.cfg file should already be constructed in Chapter 3, Task 5. |
| --- | --- |

# From EMS side B

Step 1   Login to Side B EMS from a different console server window and shutdown the machine.

<hostname># sync;sync;shutdown -i5 -g0 -y

Step 2   Remove disk0 from slot 0 off the machine and label it as “Release 3.5.5 EMS side B disk0”. Also remove disk1 from slot 1 off the machine and label it as “Release 3.5.5 EMS side B disk1”.

- SunFire V240 disk slot lay out:

| Disk 2 | Disk 3 |  |
| --- | --- | --- |
| Disk 0 | Disk 1 | DVD-ROM |

- SunFire V440 disk slot lay out:

| Disk 3 | DVD-ROM |
| --- | --- |
| Disk 2 |  |
| Disk 1 |  |
| Disk 0 |  |

- Sunfire 1280 disk slot lay out:

|  | DVD-ROM |
| --- | --- |
|  | Disk 1 |
|  | Disk 0 |

Step 3   Place new disk labeled as “Release 4.5 EMS side B disk0” in slot 0. Also place new disk labeled as “Release 4.5 EMS side B disk1” in slot 1.

Step 4   Power on the machine and allow the system to boot up by monitoring the boot process thru console.

- For Sunfire 1280 machine, please execute the following command from console:

<lom>poweron

- For other type of hardware, please use the power button to turn on the power.

Step 5   Log in as root thru console, use password “opticall”

Step 6   Rebuild network interface hardware configuration on disk since the disks used for upgrade may have been jumpstarted in a different machine:

<hostname># cp –p /etc/path_to_inst /etc/path_to_inst.save

<hostname># egrep -iv “qfe|ce|eri|bge|hme|pci_pci” /etc/path_to_inst.save > /etc/path_to_inst

Step 7   Rebuild the hardware configuration since the disks used for upgrade may have been jumpstarted in a different machine:

<hostname># sync;sync;reboot -- -r

- Wait for the system to boot up. Then log in as root.

- Verify the Ethernet port assignment in the /etc/path_to_inst file. If the ports are mis-assigned, please make the correction. Please don’t reboot the machine at this point.

Step 8   Restore interfaces since the disks used for upgrade may have been jumpstarted in a different machine:

- <hostname># ifconfig <primary interface name> plumb

  - Use Interface Name recorded in Chapter 3, Task 3

- <hostname># ifconfig <primary interface name> <Interface IP> netmask <NETMASK> broadcast + up

  - Use IP and NETMASK recorded in Chapter 3, Task 3 for the interface

- Add static routes to reach Domain Name Server and Network File Server:

  - <hostname># echo “route add -net <network to reach DNS> <gateway IP>” >> /etc/rc3.d/S96StaticRoutes

  - <hostname># /etc/rc3.d/S96StaticRoutes

Example:

  - If the primary network manager interface for the machine is qfe0, run “ifconfig qfe0”, you should have following output:

qfe0: flags=1000843<UP,BROADCAST,RUNNING,MULTICAST,IPv4> mtu 1500 index 2 inet 10.89.183.148 netmask ffffff00 broadcast 10.89.183.255 ether 0:3:ba:9d:68:85

  - If the DNS server is on network “10.89.0.0”, then add gateway 10.89.183.254 to reach that network with following command:

<hostname># echo “route add -net 10.89.0.0 10.89.183.254” >> /etc/rc3.d/S96StaticRoutes

<hostname># /etc/rc3.d/S96StaticRoutes

Step 9   Reset ssh keys:

<hostname># \rm /.ssh/known_hosts

Step 10   sftp required files:

<hostname># sftp <Side A EMS IP>

sftp> get /etc/resolv.conf /etc/resolv.conf

sftp> get /opt/.upgrade/cfg/opticall.cfg /etc/opticall.cfg

sftp> get /opt/Build/btsauto/critical_trunk_grp.txt /opt/Build/btsauto/critical_trunk_grp.txt

sftp> get /opt/Build/btsauto/testSS7.cfg /opt/Build/btsauto/testSS7.cfg

sftp> quit

Step 11   Run script program to replace the hostname

<hostname># cd /opt/ems/upgrade

<hostname># DoTheChange -s

- The system will reboot when the script DoTheChange completes its run

Step 12   Wait for the system to boot up. Then log in as root.

Step 13   Editing /etc/default/init:

<hostname># vi /etc/default/init

- Remove lines and keep only the following lines:

#

TZ=<Timezone>

CMASK=022

For an example:

The original /etc/default/init file before line removal:

# @(#)init.dfl 1.5 99/05/26

#

# This file is /etc/default/init.  /etc/TIMEZONE is a symlink to this file.

# This file looks like a shell script, but it is not.  To maintain

# compatibility with old versions of /etc/TIMEZONE, some shell constructs

# (i.e., export commands) are allowed in this file, but are ignored.

#

# Lines of this file should be of the form VAR=value, where VAR is one of

# TZ, LANG, CMASK, or any of the LC_* environment variables.

#

TZ=US/Central

CMASK=022

LC_COLLATE=en_US.ISO8859-1

LC_CTYPE=en_US.ISO8859-1

LC_MESSAGES=C

LC_MONETARY=en_US.ISO8859-1

LC_NUMERIC=en_US.ISO8859-1

LC_TIME=en_US.ISO8859-1

The /etc/default/init file after line removal:

# @(#)init.dfl 1.5 99/05/26

#

# This file is /etc/default/init.  /etc/TIMEZONE is a symlink to this file.

# This file looks like a shell script, but it is not.  To maintain

# compatibility with old versions of /etc/TIMEZONE, some shell constructs

# (i.e., export commands) are allowed in this file, but are ignored.

#

# Lines of this file should be of the form VAR=value, where VAR is one of

# TZ, LANG, CMASK, or any of the LC_* environment variables.

#

TZ=US/Central

CMASK=022

Step 14   Verify interface hardware configuration match to the host configuration:

<hostname># egrep -i “qfe|ce|eri|bge|hme” /etc/path_to_inst

<hostname># ls -l /etc/hostname.*

- If the interface names match from the above two outputs, please continue on Step 15.

- If the interface names do NOT match, please match them by changing the postfix of hostname.*.

For an example:

Output from “egrep -i “qfe|ce|eri|bge|hme” /etc/path_to_inst” is:

"/pci@1f,4000/network@1,1" 0 "hme"

"/pci@1f,4000/pci@4/SUNW,qfe@0,1" 0 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@1,1" 1 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@2,1" 2 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@3,1" 3 "qfe"

Output from “ls -l /etc/hostname.*” is:

-rw-r--r--   1 root     other          14 May 16 16:03 /etc/hostname.hme0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.hme0:1

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.eri0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.eri0:1

After change, the output should be:

-rw-r--r--   1 root     other         14 May 16 16:03 /etc/hostname.hme0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.hme0:1

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.qfe0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.qfe0:1

Step 15   Reboot the machine to pick up new TIMEZONE and interface setting:

<hostname># sync; sync; reboot

- Wait for the system to boot up. Then log in as root.

Note:  If there are patches need to be applied, please apply the patches now.  Each patch should be applied by following the steps come with each patch.

Step 16   CDR delimiter customization is not retained after software upgrade. If this system has been customized, either the Customer or Cisco Support Engineer must manually customize again to keep the same customization.

- <hostname># cd /opt/bdms/bin

- <hostname># vi platform.cfg

- Find the section for the command argument list for the BMG process

- Customize the CDR delimiters in the “Args=” line

- Example:

Args=-port 15260 -h localhost -u optiuser -p optiuser -fmt default_formatter -UpdIntvl 3300 -ems_local_dn blg-aSYS14EMS.cisco.com -FD semicolon -RD verticalbar

# Task 7: Upgrade CA/FS Side B to the new release

|  | Warning Do not proceed if you don’t have a pre-constructed opticall.cfg file for the system. The opticall.cfg file should already be constructed in Chapter 3, Task 5. |
| --- | --- |

# From CA/FS side B

Step 1   Login to Side B CA/FS from a different console server window and shutdown the machine.

<hostname># sync;sync;shutdown –i5 –g0 –y

Step 2   Remove disk0 from slot 0 off the machine and label it as “Release 3.5.5 CA/FS side B disk0”. Also remove disk1 from slot 1 off the machine and label it as “Release 3.5.5 CA/FS side B disk1”

- SunFire V240 disk slot lay out:

| Disk 2 | Disk 3 |  |
| --- | --- | --- |
| Disk 0 | Disk 1 | DVD-ROM |

- SunFire V440 disk slot lay out:

| Disk 3 | DVD-ROM |
| --- | --- |
| Disk 2 |  |
| Disk 1 |  |
| Disk 0 |  |

- Sunfire 1280 disk slot lay out:

|  | DVD-ROM |
| --- | --- |
|  | Disk 1 |
|  | Disk 0 |

Step 3   Place new disk labeled as “Release 4.5 CA/FS side B disk0” in slot 0. Also place new disk labeled as “Release 4.5 CA/FS side B disk1” in slot 1.

Step 4   Power on the machine and allow the system to boot up by monitoring the boot process through console.

- For Sunfire 1280 machine, please execute the following command from console:

<lom>poweron

- For other type of hardware, please use the power button to turn on the power.

Step 5   Log in as root from console.

Step 6   Rebuild network interface hardware configuration on disk since the disks used for upgrade may have been jumpstarted in a different machine:

<hostname># cp –p /etc/path_to_inst /etc/path_to_inst.save

<hostname># egrep -iv “qfe|ce|eri|bge|hme|pci_pci” /etc/path_to_inst.save > /etc/path_to_inst

Step 7   Rebuild the hardware configuration since the disks used for upgrade may have been jumpstarted in a different machine:

<hostname># sync;sync;reboot -- -r

- Wait for the system to boot up. Then log in as root.

- Verify the Ethernet port assignment in the /etc/path_to_inst file. If the ports are mis-assigned, please make the correction. Please don’t reboot the machine at this point.

Step 8   Restore interfaces since the disks used for upgrade may have been jumpstarted in a different machine:

- <hostname># ifconfig <interface name> plumb

  - Use Interface Name recorded in Chapter 3, Task 3.

- <hostname># ifconfig <interface name> <Interface IP> netmask <NETMASK> broadcast + up

  - Use IP and NETMASK recorded in Chapter 3, Task 3.

- Add static routes to reach Domain Name Server and Network File Server using “route add …” command:

  - Example: route add -net 10.89.0.0 10.89.232.254

Where: 10.89.0.0 is the destination DNS server network IP
              10.89.232.254 is the gateway IP

Step 9   Reset ssh keys:

<hostname># \rm /.ssh/known_hosts

Step 10   sftp opticall.cfg file:

<hostname># sftp <Side A EMS IP>

sftp> get /opt/.upgrade/cfg/opticall.cfg /etc/opticall.cfg

sftp> quit

Step 11   sftp resolv.conf file:

<hostname># sftp <Side A CA/FS IP>

sftp> get /etc/resolv.conf /etc/resolv.conf

sftp> quit

Step 12   Run script program to replace the hostname

<hostname># cd /opt/ems/upgrade

<hostname># DoTheChange -s

- The system will reboot when the script DoTheChange completes its run

Step 13   Wait for the system to boot up. Then Log in as root.

Step 14   Editing /etc/default/init:

<hostname># vi /etc/default/init

- Remove lines and keep only the following lines:

#

TZ=<Timezone>

CMASK=022

For an example:

The original /etc/default/init file before line removal:

# @(#)init.dfl 1.5 99/05/26

#

# This file is /etc/default/init.  /etc/TIMEZONE is a symlink to this file.

# This file looks like a shell script, but it is not.  To maintain

# compatibility with old versions of /etc/TIMEZONE, some shell constructs

# (i.e., export commands) are allowed in this file, but are ignored.

#

# Lines of this file should be of the form VAR=value, where VAR is one of

# TZ, LANG, CMASK, or any of the LC_* environment variables.

#

TZ=US/Central

CMASK=022

LC_COLLATE=en_US.ISO8859-1

LC_CTYPE=en_US.ISO8859-1

LC_MESSAGES=C

LC_MONETARY=en_US.ISO8859-1

LC_NUMERIC=en_US.ISO8859-1

LC_TIME=en_US.ISO8859-1

The /etc/default/init file after line removal:

# @(#)init.dfl 1.5 99/05/26

#

# This file is /etc/default/init.  /etc/TIMEZONE is a symlink to this file.

# This file looks like a shell script, but it is not.  To maintain

# compatibility with old versions of /etc/TIMEZONE, some shell constructs

# (i.e., export commands) are allowed in this file, but are ignored.

#

# Lines of this file should be of the form VAR=value, where VAR is one of

# TZ, LANG, CMASK, or any of the LC_* environment variables.

#

TZ=US/Central

CMASK=022

Step 15   Verify interface hardware configuration match to the host configuration:

<hostname># egrep -i “qfe|ce|eri|bge|hme” /etc/path_to_inst

<hostname># ls -l /etc/hostname.*

- If the interface names match from the above two outputs, please continue on Step 16.

- If the interface names do NOT match, please match them by changing the postfix of hostname.*.

For an example:

Output from “egrep -i “qfe|ce|eri|bge|hme” /etc/path_to_inst” is:

"/pci@1f,4000/network@1,1" 0 "hme"

"/pci@1f,4000/pci@4/SUNW,qfe@0,1" 0 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@1,1" 1 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@2,1" 2 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@3,1" 3 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@0,1" 4 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@1,1" 5 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@2,1" 6 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@3,1" 7 "qfe"

Output from “ls -l /etc/hostname.*” is:

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.hme0

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.eri0

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri1

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri1:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.eri1:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.eri1:3

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri2

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri2:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.eri2:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.eri2:3

After change, the output should be:

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.hme0

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.qfe0

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe1

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe1:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.qfe1:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.qfe1:3

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe2

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe2:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.qfe2:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.qfe2:3

Step 16   Reboot the machine to pick up new TIMEZONE and interface setting:

<hostname># sync; sync; reboot

- Wait for the system to boot up. Then log in as root.

Step 17   Check for configuration errors

<hostname># cd /opt/Build

<hostname># checkCFG –u

- Please ignore DNS lookup mismatch warnings for the MGCP domain name

- Correct errors generated by checkCFG

- Once the result is clean without errors, then proceed to the next step.

Step 18   Change the IPs for MGCP domain name from 4 physical IPs (2 physical IPs from primary CA/FS: 2P, and 2 physical IPs from secondary CA/FS: 2S) to two logical IPs: 2P + 2S  2L:

- <hostname># vi /etc/hosts

- Delete 2 physical IPs came from primary CA/FS.

- Delete 2 physical IPs came from secondary CA/FS.

- Add 2 new logical IPs for the MGCP domain name in release 4.5.

- Check to make sure there are only 2 logical IP addresses left for mgcp domain name

Step 19   <hostname># install.sh -upgrade

Step 20   Answer "y" when prompted. This process will take up to 15 minutes to complete.

- Answer "y" if prompted for reboot

- Wait for the system to boot up. Then log in as root.

- Edit /etc/hosts file to make sure there are only 2 logical IP addresses left for mgcp domain name

Step 21   Update CA/FS platform.cfg files:

<hostname># cd /opt/Build/btsauto

<hostname># update_platform.sh

- During this task, Warning messages and “unable to write” may appear.  These two types of warnings can be safely ignored.  The first type, Warning, "no longer a linked file" occurs during the update of the time zone files.   The second type "unable to write 'random state'" occurs while generating apache certificate.

Note:  If there are patches need to be applied, please apply the patches now.  Each patch should be applied by following the steps come with each patch.

# Task 8: Continue Upgrade Process

This is where we continue what was started in Task 5 on EMS Side A, now that Task 6 and Task 7 have been completed.  We should be at the following display:

========================================================

Please remove the disks on <secems> and <secca> and insert new disks.

Your connection will be lost and need to relogin after the machine is up with the new disk

Please enter y when you are ready to reconnect to <secems>

========================================================

Are you ready to reconnect? [y]

Step 1	 Press “Enter” or (“y”) on EMS Side A where the upgrade process is waiting for user confirmation to proceed after completing two tasks above: Task 6 and Task 7. The script will now connect to side B EMS. Enter the password for EMS Side B if promoted. Default password is “opticall”.

Step 2   The script will prompt to reconnect to Side B CA.

Please enter y when you are ready to reconnect to <secca>

Are you ready to reconnect? [y]

Step 3   Enter (“y”) and continue the upgrade as you have already swapped the disk on CA Side B (task 6 above). Enter password for CA Side B if promoted. The default password is “opticall”. The script will continue and you will see the following checkpoints:

*** CHECKPOINT setSecEmsSimplex ***

*** CHECKPOINT startSecApps ****** CHECKPOINT switchToSideB ***

Step 4	 Upgrade process will continue for a while as the data is being migrated from side A to Side B.

Note:  While this is going on, make sure that you have the IOS commands for working with the ITPs ready as they will be used shortly.

Step 5   You will be prompted with the following:

About to change platform to standby-active.  Continue? [y]

When you answer “y”, the process will start towards a switchover and the offnet outage occurs. A note will appear stating “Waiting for OMNI to stabilize. It may take 10 minutes......” After this period of time, the switchover will begin and the SS7 trunk groups will be taken out of service and the OFFNET OUTAGE will begin. ONNET calls will continue.

- When you receive the message “Please proceed after removing SS7 links and linkset from ITP. Continue?” Please execute IOS commands to first remove the SS7 E-link(s) and linkset, and second to add the SCTP configuration. Sample IOS command to remove link(s) and linkset is given below. Point code 7.7.7 is the BTS point code:

cs7 linkset to_bts10200 7.7.7

shut

no cs7 linkset to_bts10200 7.7.7

- Setting up SCTP configuration from ITPs -- Please execute the SCTP setting IOS commands on each ITP pair. The IOS command file should already be prepared in Chapter 2, Task 5 and detailed in Appendix D.

# From EMS side B (The Active EMS)

Step 6   Please change the AIN and POTS port number to Cisco fixed port.

For Example: You first showing what have been provisioned in the system, then use the information to change the port setting.

CLI> show feature-server

Reply : Success:  Entries 1-2 of 2 returned.

ID=FSPTC311

TSAP_ADDR_SIDEA=pots-HUB27PTC.voip.bhntampa.com:11311

STATUS=FORCED_ACTIVE_STANDBY

TYPE=POTS

FS_SWITCHOVER_ON_CA_CONN_LOSS=N

ID=FSAIN211

TSAP_ADDR_SIDEA=asm-HUB27AIN.voip.bhntampa.com:11211

STATUS=FORCED_ACTIVE_STANDBY

TYPE=AIN

FS_SWITCHOVER_ON_CA_CONN_LOSS=N

CLI> change feature-server ID=FSAIN211;TSAP_ADDR= asm-HUB27AIN.voip.bhntampa.com:11205;

CLI> change feature-server ID=FSPTC311; TSAP_ADDR= pots-HUB27PTC.voip.bhntampa.com:11235;

Step 7   After the ITP commands have been complete

Please press <Enter> key to continue on the EMS side A.

Note: The upgrade program will continue to run and will reach a point where synchronization will begin between the EMS Side B and the CA Side B.  This synchronization may take up to 10 minutes depending on the number of subscribers in a system.  During this time, however, normal call processing takes place on active Side B with release 4.5.

Step 8   Now the system synchronization has been complete

Please press <Enter> key to continue on the EMS side A at following prompt:

About to stop platforms on <hostname> and <hostname>.  Continue? [y]

Upon entering the <Enter> key, you should get following message:

"************ Exiting out of the application ************"

"Please swap the disk on Side A machines."

"The application must be re-started for the remaining upgrade."

"************ Exiting out of the application ************"

Now the automated upgrade process has stopped. The upgrade program has to be restarted as directed in this Chapter, Task 12.

You have now reached middle upgrade point and the Side B systems are active with release 4.5.  Please continue to halt CLI provisioning until the entire system is upgraded with a decision to keep the release 4.5.

You can now perform 24-hour soak to bake-in the system.

In the meantime, you should perform the steps in Appendix M to prepare the side A systems in the event a half system fallback is required.

After 24-hour soak or at certain point of the soak, a decision must be made whether to continue the upgrade on Side A to release 4.5 or fallback:

  - If the decision is to upgrade Side A, proceed to Step 9 (the next immediate step).

  - If the decision is to fallback to the 3.5.5 release, proceed to Appendix B.

Step 9   DNS changes:

From each Domain Name Server that is serving the BTS 10200 to be upgraded, change the IP addresses of MGCP domain name from 2 physical IPs plus 2 logical IPs (2 IPs from primary CA/FS plus 2 logical IPs:  2P + 2L) to 2 logical IPs. So the IPs for MGCP domain name is changed:  2P + 2L  2L.

Please refresh the DNS cache and make sure the new IP addresses are taking affect.

Step 10   Verify the DNS changes:

To verify the change has taken affect, please log in to both CA/FS machines and perform “nslookup” of the MGCP domain name.

For an example:

<hostname># nslookup ca-mgcp.twc.cisco.com

…

Name:    ca-mgcp.twc.cisco.com

Addresses:  13.200.10.13, 13.200.20.13

<hostname># ping 13.200.10.13

13.200.10.13 is alive       logical IP

<hostname># ping 13.200.20.13

13.200.20.13 is alive       logical IP

Note:  After the upgrade to release 4.5, only the 2 logical IP addresses will be used and they will always associate with the active CA.

Note:  Please perform Task 9 and Task 10 in parallel to shorten the upgrade time.

# Task 9: Upgrade EMS side A to the new release

# From EMS side A

Step 1   Shutdown the machine using following command. (Your login is from the console server)

<hostname># sync;sync;shutdown -i5 -g0 -y

Step 2   Remove disk0 from slot 0 off the machine and label it as “Release 3.5.5 EMS side A disk0”. Also remove disk1 from slot 1 off the machine and label it as “Release 3.5.5 EMS side A disk1”.

- SunFire V240 disk slot lay out:

| Disk 2 | Disk 3 |  |
| --- | --- | --- |
| Disk 0 | Disk 1 | DVD-ROM |

- SunFire V440 disk slot lay out:

| Disk 3 | DVD-ROM |
| --- | --- |
| Disk 2 |  |
| Disk 1 |  |
| Disk 0 |  |

- Sunfire 1280 disk slot lay out:

|  | DVD-ROM |
| --- | --- |
|  | Disk 1 |
|  | Disk 0 |

Step 3   Place new disk labeled as “Release 4.5 EMS side A disk0” in slot 0. Also place new disk labeled as “Release 4.5 EMS side A disk1” in slot 1.

Step 4   Power on the machine and allow the system to boot up by monitoring the boot process through console.

- For Sunfire 1280 machine, please execute the following command from console:

<lom> poweron

- For other type of hardware, please use the power button to turn on the power.

Step 5   Log in as root from console.

Step 6   Rebuild network interface hardware configuration on disk since the disks used for upgrade may have been jumpstarted in a different machine:

<hostname># cp –p /etc/path_to_inst /etc/path_to_inst.save

<hostname># egrep -iv “qfe|ce|eri|bge|hme|pci_pci” /etc/path_to_inst.save > /etc/path_to_inst

Step 7   Rebuild the hardware configuration since the disks used for upgrade may have been jumpstarted in a different machine:

<hostname># sync;sync;reboot -- -r

- Wait for the system to boot up. Then log in as root.

- Verify the Ethernet port assignment in the /etc/path_to_inst file. If the ports are mis-assigned, please make the correction. Please don’t reboot the machine at this point.

Step 8   Restore interfaces since the disks used for upgrade may have been jumpstarted in a different machine:

- <hostname># ifconfig <primary interface name> plumb

  - Use Interface Name recorded in Chapter 3, Task 3.

- <hostname># ifconfig <primary interface name> <Interface IP> netmask <NETMASK> broadcast + up

  - Use IP and NETMASK recorded in Chapter 3, Task 3.

- Add static routes to reach Domain Name Server and Network File Server:

  - <hostname># echo “route add -net <network to reach DNS> <gateway IP>” >> /etc/rc3.d/S96StaticRoutes

  - <hostname># /etc/rc3.d/S96StaticRoutes

Example:

  - If the primary network manager interface for the machine is qfe0, run “ifconfig qfe0”, you should have following output:

qfe0: flags=1000843<UP,BROADCAST,RUNNING,MULTICAST,IPv4> mtu 1500 index 2 inet 10.89.183.148 netmask ffffff00 broadcast 10.89.183.255 ether 0:3:ba:9d:68:85

  - If the DNS server is on network “10.89.0.0”, then add gateway 10.89.183.254 to reach that network with following command:

<hostname># echo “route add -net 10.89.0.0 10.89.183.254” >> /etc/rc3.d/S96StaticRoutes

<hostname># /etc/rc3.d/S96StaticRoutes

Step 9   Reset ssh keys:

<hostname># \rm /.ssh/known_hosts

Step 10   sftp required files:

<hostname># sftp <Side B EMS IP>

sftp> get /etc/resolv.conf /etc/resolv.conf

sftp> get /etc/opticall.cfg /etc/opticall.cfg

sftp> quit

Step 11   Run script program to replace the hostname

<hostname># cd /opt/ems/upgrade

<hostname># DoTheChange -p

- The system will reboot when the script DoTheChange completes its run

Step 12   Wait for the system to boot up. Then Log in as root.

Step 13   Verify interface hardware configuration match to the host configuration:

<hostname># egrep –i “qfe|ce|eri|bge|hme” /etc/path_to_inst

<hostname># ls -l /etc/hostname.*

- If the interface names do Match, please continue on Step 14.

- If the interface names do NOT match, please match them by changing the postfix of hostname.*.  Then run the following command:

<hostname># sync; sync; reboot

  - Wait for the system to boot up. Then log in as root.

For an example:

Output from “egrep –i “qfe|ce|eri|bge|hme” /etc/path_to_inst” is:

"/pci@1f,4000/network@1,1" 0 "hme"

"/pci@1f,4000/pci@4/SUNW,qfe@0,1" 0 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@1,1" 1 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@2,1" 2 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@3,1" 3 "qfe"

Output from “ls -l /etc/hostname.*” is:

-rw-r--r--   1 root     other         14 May 16 16:03 /etc/hostname.hme0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.hme0:1

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.eri0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.eri0:1

After change, the output should be:

-rw-r--r--   1 root     other         14 May 16 16:03 /etc/hostname.hme0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.hme0:1

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.qfe0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.qfe0:1

Note:  If there are patches need to be applied, please apply the patches now.  Each patch should be applied by following the steps come with each patch.

Step 14   CDR delimiter customization is not retained after software upgrade. If this system has been customized, either the Customer or Cisco Support Engineer must manually customize again to keep the same customization.

- <hostname># cd /opt/bdms/bin

- <hostname># vi platform.cfg

- Find the section for the command argument list for the BMG process

- Customize the CDR delimiters in the “Args=” line

- Example:

Args=-port 15260 -h localhost -u optiuser -p optiuser -fmt default_formatter -UpdIntvl 3300 -ems_local_dn blg-aSYS14EMS.cisco.com -FD semicolon -RD verticalbar

# Task 10: Upgrade CA/FS Side A to the new release

# From CA/FS side A

Step 1   Login to the CA/FS Side A machine through console server and shutdown the machine.

<hostname># sync;sync;shutdown -i5 -g0 -y

Step 2   Remove disk0 from slot 0 off the machine and label it as “Release 3.5.5 CA/FS side A disk0”. Also remove disk1 from slot 1 off the machine and label it as “Release 3.5.5 CA/FS side A disk1”.

- SunFire V240 disk slot lay out:

| Disk 2 | Disk 3 |  |
| --- | --- | --- |
| Disk 0 | Disk 1 | DVD-ROM |

- SunFire V440 disk slot lay out:

| Disk 3 | DVD-ROM |
| --- | --- |
| Disk 2 |  |
| Disk 1 |  |
| Disk 0 |  |

- Sunfire 1280 disk slot lay out:

|  | DVD-ROM |
| --- | --- |
|  | Disk 1 |
|  | Disk 0 |

Step 3   Place new disk labeled as “Release 4.5 CA/FS side A disk0” in slot 0. Also place new disk labeled as “Release 4.5 CA/FS side A disk1” in slot 1.

Step 4   Power on the machine and allow the system to boot up by monitoring the boot process thru console.

- For Sunfire 1280 machine, please execute the following command from console:

<lom>poweron

- For other type of hardware, please use the power button to turn on the power.

Step 5   Log in as root.

Step 6   Rebuild network interface hardware configuration on disk since the disks used for upgrade may have been jumpstarted in a different machine:

<hostname># cp –p /etc/path_to_inst /etc/path_to_inst.save

<hostname># egrep -iv “qfe|ce|eri|bge|hme|pci_pci” /etc/path_to_inst.save > /etc/path_to_inst

Step 7   Rebuild the hardware configuration since the disks used for upgrade may have been jumpstarted in a different machine:

<hostname># sync;sync;reboot -- -r

- Wait for the system to boot up. Then log in as root.

- Verify the Ethernet port assignment in the /etc/path_to_inst file. If the ports are mis-assigned, please make the correction. Please don’t reboot the machine at this point.

Step 8   Restore interfaces since the disks used for upgrade may have been jumpstarted in a different machine:

- <hostname># ifconfig <interface name> plumb

  - Use Interface Name recorded in Chapter 3, Task 3.

- <hostname># ifconfig <interface name> <Interface IP> netmask <NETMASK> broadcast + up

  - Use IP and NETMASK recorded in Chapter 3, Task 3

- Add static routes to reach Domain Name Server and Network File Server using “route add …” command:

  - Example: route add -net 10.89.0.0 10.89.232.254

Where: 10.89.0.0 is the destination DNS server network IP
              10.89.232.254 is the gateway IP

Step 9   Reset ssh keys:

<hostname># \rm /.ssh/known_hosts

Step 10   sftp required files:

<hostname># sftp <Side B CA/FS IP>

sftp> get /etc/resolv.conf /etc/resolv.conf

sftp> get /etc/opticall.cfg /etc/opticall.cfg

sftp> quit

Step 11   Run script program to replace the hostname

<hostname># cd /opt/ems/upgrade

<hostname># DoTheChange -p

- The system will reboot when the script DoTheChange completes its run

Step 12   Wait for the system to boot up. Then Log in as root.

Step 13   Verify interface hardware configuration match to the host configuration:

<hostname># egrep –i “qfe|ce|eri|bge|hme” /etc/path_to_inst

<hostname># ls -l /etc/hostname.*

- If the interface names do NOT match, please match them by changing the postfix of hostname.*. The run the following command:

<hostname># sync; sync; reboot

  - Wait for the system to boot up. Then log in as root.

For an example:

Output from “egrep –i “qfe|ce|eri|bge|hme” /etc/path_to_inst” is:

"/pci@1f,4000/network@1,1" 0 "hme"

"/pci@1f,4000/pci@4/SUNW,qfe@0,1" 0 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@1,1" 1 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@2,1" 2 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@3,1" 3 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@0,1" 4 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@1,1" 5 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@2,1" 6 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@3,1" 7 "qfe"

Output from “ls -l /etc/hostname.*” is:

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.hme0

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.eri0

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri1

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri1:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.eri1:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.eri1:3

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri2

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri2:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.eri2:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.eri2:3

After change, the output should be:

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.hme0

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.qfe0

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe1

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe1:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.qfe1:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.qfe1:3

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe2

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe2:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.qfe2:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.qfe2:3

Step 14   Install the applications

<hostname># cd /opt/Build

<hostname># install.sh –upgrade

Step 15   Answer "y" when prompted. This process will take up to 15 minutes to complete.

- Answer "y" if prompted for reboot

- Wait for the system to boot up. Then Log in as root.

Step 16   Update CA/FS platform.cfg files:

<hostname># cd /opt/Build/btsauto

<hostname># update_platform.sh

Note:  If there are patches need to be applied, please apply the patches now.  Each patch should be applied by following the steps come with each patch.

# Task 11: Restore customized cron jobs

Please restore customized cron jobs by using the files saved on the network file server during system preparation stage in Chapter 3, Task 3. Please don’t copy the old crontab file over the new one. You may need to compare the back up version of the crontab file to the new crontab file to restore the previous settings. This should be done for all machines in the system.

# Task 12: Re-start the Upgrade program on EMS Side A

# From EMS side A

Step 1	  Log in as root to EMS side A thru Console

Step 2   <hostname># cd /opt/Build/btsauto

Step 3	 <hostname># bts_upgrade.exp –diskSwap

Step 4   Enter password if prompted. The default password is “opticall”.

Answer “y” at the following prompt:

About to change platform to active-standby.  Continue? [y]

Step 5   From this point onwards the upgrade process is self driven. Please follow the instruction/messages by the program to perform any manual steps and inputs.  Part of this step is to copy database from side B to side A.  This will take a while depending on DB data pattern and data fill.

# Task 13: Check audit report and sync DB if necessary

# From EMS side A

# Step 1   Check audit report to see if there are any mismatches

# Step 2   If mismatches are found, run the following step to synchronize DB between EMS and CA/FS.

<hostname># cd /opt/Build/btsauto

<hostname># sync_audited_data.sh /opt/ems/report/Audit_*_btsuser.sql

Step 3   Run the audit against the tables with mismatched records to verify there is no more DB mismatches between EMS and CA/FS.

You have completed the Cisco BTS 10200 system upgrade process successfully.

Now the entire system is upgraded to release 4.5, if for any reason the system is unable to process calls or having high call failures, please contact Cisco support immediately. A disaster recovery process may be required to restore the system to a previous release.

Chapter 6

Post Upgrade Tasks

This chapter covers tasks that to be executed only after completing software upgrade successfully from release 3.5.5 to release 4.5. Cisco recommends that these tasks to be performed in a separate maintenance window to allow the time for BTS to stabilize.

# Task 1: Reset TTL value from DNS

Step 1   DNS changes:

From each Domain Name Server that is serving the upgraded BTS 10200, please change the Time-to-Live value from 600 seconds to 86400 seconds.

Please refresh the DNS cache and make sure the new TTL value is taking affect.

Step 2   Verify the DNS changes:

To verify the new TTL value has taken affect from Primary CA/FS, please login as root and use the actual Domain Name for parameter DNS_FOR_CA_MGCP_COM from /etc/opticall.cfg.

For an example:

- Verify the TTL for all 2 IPs is 86400 seconds

<hostname># dig ca-mgcp.twc.cisco.com

- The TTL value will be shown in the second field in the ANSWER SECTION

; <<>> DiG 9.2.4 <<>> ca-mgcp.twc.cisco.com

;; global options:  printcmd

;; Got answer:

;; ->>HEADER<<- opcode: QUERY, status: NOERROR, id: 1664

;; flags: qr aa rd ra; QUERY: 1, ANSWER: 3, AUTHORITY: 2, ADDITIONAL: 2

;; QUESTION SECTION:

; ca-mgcp.twc.cisco.com.       IN      A

;; ANSWER SECTION:

ca-mgcp.twc.cisco.com. 86400 IN A     13.200.10.13

ca-mgcp.twc.cisco.com. 86400 IN A     13.200.20.13

# Task 2: Change the password

If you choose to keep the default password “opticall”, there is no action needed on your part. Otherwise, please reset the password on each machine.

# Task 3: Provisioning new features

# From EMS side A

#### Add new announcements

Step 1   The new announcements should already been prepared in Chapter 3, Task 8. Please follow steps specified in Appendix F to place the new announcements to appropriate Media Gateways.

#### Associate announcements

Step 1   Place the announcement CLI file ‘cfg_announcement’ under /opt/ems/ftp/deposit directory.

<hostname># chown optiuser:staff /opt/Build/btsauto/cli/*

<hostname># cd /opt/ems/ftp/deposit

<hostname># cp –p /opt/Build/btsauto/cli/cfg_announcement .

Note: If the file is not found in the above location, copy from /tmp/cli/announcement directory.

- Wait for the batch process to pick up the script from /opt/ems/ftp/deposit directory and provision it into the system.

- Keep running “ls” command from /opt/ems/ftp/deposit directory and till the file is gone

- Check the report file for results.

<hostname># cd /opt/ems/report

<hostname># egrep Failed:0 <filename>.html

- Verify the system returns an output. If there is no output returned, please check for errors in the report file.

#### Create new features and update existing features

Step 1   Place the CLI file under /opt/ems/ftp/deposit directory.

<hostname># cd /opt/ems/ftp/deposit

<hostname># cp –p /opt/Build/btsauto/cli/cfg_callfwd .

Note: If the file is not found in the above location, copy from /tmp/cli/callfwd directory

- Wait for the batch process to pick up the script from /opt/ems/ftp/deposit directory and provision it into the system.

- Keep running “ls” command from /opt/ems/ftp/deposit directory and till the file is gone

- Check the report file for results.

<hostname># cd /opt/ems/report

<hostname># egrep Failed:0 <filename>.html

- Verify the system returns an output. If there is no output returned, please check for errors in the report file.

# Task 4: Restore platform.cfg with logical IPs

# From EMS side A

Step 1   log in as root

Step 2   <hostname># cd /opt/Build/btsauto

Step 3   <hostname># restore_platform_cfg.exp

- Please enter the password for root when prompted

- The program will perform the task and should terminate without any error.

# Task 5: Check disk mirroring status

From each machine, please run “/opt/utils/resync_status” to check for disk mirroring status.

# Task 6: Verify DB Heartbeat process on both EMS

If following alarm is reported frequently in EMS, manually start DB heartbeat process; otherwise skip to next task.

ID=14846

TYPE=DATABASE

NUMBER=12

DESCRIPTION=EMS database process died

SEVERITY=CRITICAL

ALARM_STATUS=ON

TIMESTAMP=2006-06-13 08:15:03

ORIGIN=DATABASE

THREAD=UNSPECIFIED THREAD

COMPONENT_ID=DBHeartBeat

HOSTNAME=RDU123EM1

DATABASENAME=optical1

ERRORSOURCE=DBHeartBeat

MESSAGE=Contact_support_immediately

# From EMS side A and B

Step 1   Login to as oracle user

Step 2   <hostname>$ dbinit –H –i start

- Verify the reported alarm is cleared.

Step 3   <hostname>$ exit

# Task 7: Verify SSH daemon and listening port status

# From each machine in the system

Step 1   Login as root user if not already

Step 2   Verify sshd running status:

<hostname># ps –ef | grep sshd

If it is not running:

<hostname># /etc/rc2.d/S72local_sshd start

Step 3   Verify sshd daemon is listening on both management IP addresses

<hostname># netstat –an | grep LISTEN | grep .22

For example:

iad-ems-1# netstat -an | grep LISTEN | grep .22

127.0.0.1.22               *.*                0      0 49152      0 LISTEN

10.30.4.13.22             *.*                0      0 49152      0 LISTEN

10.30.4.5.22               *.*                0      0 49152      0 LISTEN

10.10.122.4.1521       *.*                0      0 49152      0 LISTEN

- If both management IP addresses are not listed in above output, verify and update /opt/BTSossh/etc/sshd_config to make sure both IP addresses are configured.

- For example:

iad-ca-1# cat /opt/BTSossh/etc/sshd_config | grep Listen

ListenAddress 10.30.4.3

ListenAddress 10.30.4.11

ListenAddress 127.0.0.1

# Task 8: Install CORBA Application

Please follow steps in the Appendix L to install CORBA.

The system is now in normal operation state, if for any reason the system is unable to process calls or having high call failures, please contact Cisco support immediately. Disaster recovery process may be required to restore the system to a previous release.

Appendix A

System Disaster Recovery Procedure

Warning   Executing this procedure will result in lost of all provisioning data which includes north bound handset data from the point where the side B system disks were swapped out. The system will also incur approximate 25-30 minutes of total system service outage. This procedure uses disks saved on side B sytems to restore the services and data or disk swap backout procedure.

The disaster recovery procedure is provided in a separate document. Please get a copy of the procedure from the following CCO link: http://lbj/push_targets1/ucdit/cc/td/doc/product/voice/bts10200/bts4_5/upgrade/index.htm

Appendix B

Half System Fallback

Warning   All provisioned data including handset provisioning are lost if following this procedure is executed.

# Introduction

This procedure allows you to back out of the upgrade for any reason. This procedure is intended for the scenario in which the side-B systems have been upgraded to the new load, while the side-A systems are still at the previous load. The procedure will back out the side-B systems to the previous load.

Following steps are executed during half system fallback:

- Control SCTP associations Out of Service

- Restore EMS side A Hub communication

- Shutdown side B

- Restore MGCP physical IPs

- Start Side A applications

- Restore Side A EMS DB replication

- Remove SCTP configuration and restore E-links and linkset from ITPs

- Activate Omni SS7 A-links on CA/FS side A

- Restore side B and save a copy of CA/FS shared memory

- Activate Omni SS7 A-links on CA/FS side B

- Reset TTL value from DNS

- Verify system status

- Post fallback clean up steps

Note:   In addition to performing this backout procedure, you should contact Cisco Support when you are ready to retry the upgrade procedure.

# Task 1: Control SCTP associations Out of Service

Execute CLI commands and take all sctp associations out of service.

# From Active EMS Side B

- Step 1   Login to Active EMS as optiuser

Step 2   Control SCTP associations from CA/AIN/PTC to mated ITP pair out of service as shown below:

CLI > control sctp-assoc id=itp1-sgp1-ca; mode=forced; target-state=OOS;

CLI > control sctp-assoc id=itp2-sgp1-ca; mode=forced; target-state=OOS;

CLI > control sctp-assoc id=itp1-sgp1-ain; mode=forced; target-state=OOS;

CLI > control sctp-assoc id=itp2-sgp1-ain; mode=forced; target-state=OOS;

CLI > control sctp-assoc id=itp1-sgp1-ptc; mode=forced; target-state=OOS;

CLI > control sctp-assoc id=itp2-sgp1-ptc; mode=forced; target-state=OOS;

CLI > exit

Note:   It is recommended to perform Task 3 and Task 4 in parallel.

# Task 2: Restore EMS side A Hub communication

# From EMS side A

Step 1   Log in as root.

Step 2   <hostname># /opt/ems/utils/updMgr.sh -restore_hub

# Task 3: Shutdown side B

# From CA/FS side B

Step 1   Log in as root.

Step 2   <hostname># sync; sync; shutdown -i5 -g0 -y

# From EMS side B

Step 1   Log in as root.

Step 2   <hostname># sync; sync; shutdown –i5 –g0 –y

# Task 4: Restore MGCP physical IPs

Step 1   DNS changes:

From each Domain Name Server that is serving the BTS 10200, restore the IP addresses for MGCP domain name from 2 physical IPs from primary CA/FS and 2 logical IPs: 2P + 2L back to 2 physical IPs from primary CA/FS and 2 physical IPs from secondary CA/FS: 2P + 2S. So the IPs for MGCP domain name is changed:  2P + 2L  2P + 2S.

Please make sure the DNS setting for physical IPS are:

- 600 seconds of Time-to-Live (TTL) value

Please refresh the DNS cache and make sure the new IP addresses are taking affect.

Step 2   Verify the DNS changes:

To verify the change has taken affect, please log in to both CA/FS machines and perform “nslookup” of the MGCP domain name.

For an example:

<hostname># nslookup ca-mgcp.twc.cisco.com

…

Name:    ca-mgcp.twc.cisco.com

Addresses:  13.200.10.11, 13.200.20.11, 13.200.10.12, 13.200.20.12

<hostname># ping 13.200.10.11

13.200.10.13 is alive       physical IP

<hostname># ping 13.200.10.12

13.200.20.13 is alive       physical IP

<hostname># ping 13.200.20.11

13.200.10.13 is alive       physical IP

<hostname># ping 13.200.20.12

13.200.20.13 is alive       physical IP

Note:  After the fallback to release 3.5.5, physical IP addresses are used.

# Verify Time-to-Live (TTL) from Primary CA/FS

Step 1   Login to primary CA/FS as root

Step 2   Check TTL setting. Please use the actual Domain Name for parameter DNS_FOR_CA_MGCP_COM from /etc/opticall.cfg.

For an example:

- The MGCP Domain Name “ca-mgcp.twc.cisco.com” is configured with 4 physical IPs (2 from Primary CA/FS and 2 from Secondary CA/FS).

- Verify the TTL for all 4 IPs is 600 seconds

<hostname># nslookup -debug ca-mgcp.twc.cisco.com

- The TTL will be shown in the ANSWERS section as: ttl = <time to live in seconds>

…

ANSWERS:

->  ca-mgcp.twc.cisco.com

internet address = 13.200.10.11               Primary CA/FS physical IP

ttl = 600 (10M)

->  ca-mgcp.twc.cisco.com

internet address = 13.200.10.12               Secondary CA/FS physical IP

ttl = 600 (10M)

->  ca-mgcp.twc.cisco.com

internet address = 13.200.20.11              Primary CA/FS physical IP

ttl = 600 (10M)

->  ca-mgcp.twc.cisco.com

internet address = 13.200.20.12              Secondary CA/FS physical IP

ttl = 600 (10M)

# Task 5: Start Side A applications

# From CA/FS side A

Step 1

<hostname># platform start -noping

# From EMS side A

Step 1   <hostname># platform start -noping

# Task 6: Restore Side A EMS DB replication

# Task 7: Remove SCTP configuration and restore E-links and linkset from ITPs

# Remove SCTP configuration from ITPs

Please remove all SCTP settings from ITPS by executing IOS commands. The IOS command file should already be prepared in Chapter 2, Task 5. If not, please remove the configuration manually.

# Restore OMNI SS7 link(s) and linkset from ITP

Restore SS7 link(s) and linkset before switching over the application platforms from release 4.5 Secondary Side B to release 3.5.5 Primary Side A. Sample IOS command is given below:

cs7 as TB12-LNP sua

shut

no cs7 as TB12-LNP sua

!

cs7 as TB12-CNAM sua

shut

no cs7 as TB12-CNAM sua

!

cs7 as TB12-ISUP m3ua

shuT

no cs7 as TB12-ISUP m3ua

!

cs7 as TB12-800A sua

shut

no cs7 as TB12-800A sua

!

cs7 as TB12-800T sua

shut

no cs7 as TB12-800T sua

!

cs7 as TB12-ACAR sua

shut

no cs7 as TB12-ACAR sua

!

cs7 asp tb12CAPRI 11146 2905 m3ua

shut

no cs7 asp tb12CAPRI 11146 2905 m3ua

!

cs7 asp tb12CASEC 11146 2905 m3ua

shut

no cs7 asp tb12CASEC 11146 2905 m3ua

!

cs7 asp tb12AinPRI 12205 14001 sua

shut

no cs7 asp tb12AinPRI 12205 14001 sua

!

cs7 asp tb12AinSec 12205 14001 sua

shut

no cs7 asp tb12AinSec 12205 14001 sua

!

cs7 asp tb12PotPri 12235 14001 sua

shut

no cs7 asp tb12PotPri 12235 14001 sua

!

cs7 asp tb12PotSec 12235 14001 sua

shut

no cs7 asp tb12PotSec 12235 14001 sua

!

cs7 linkset to_TB12 1.12.1

description linkset to TB-12

link 0 Serial1/1/5:0

link 1 Serial1/1/6:0

no shut

!

cs7 linkset to_inet3 8.203.103

description to_inet3_pod_1 for TB-12

link 0 Serial1/1/3:0

no shut

cs7 linkset to_bts10200 7.7.7

description linkset to bts10200

link 0 Serial1/1/5:0

link 1 Serial1/1/6:0

no shut

# Task 8: Activate Omni SS7 A-links on CA/FS side A

# From CA/FS side A

Step 1   Activate SS7 Links on CA/FS side A.

1. <hostname># termhandler -node a7n1

- OMNI [date] #1:actv-slk:slk=< SS7 link on CA/FS side A >;

- Enter y to continue.

- Repeat above two steps for each inactive link

2. OMNI [date] #2:display-slk;

- Enter y to continue

- Verify that the state of each link is ACTIVE.

3. OMNI[date]#3:quit

# Task 9: Restore side B and save a copy of CA/FS shared memory

# From CA/FS side B

Step 1   Remove “Release 4.5 CA/FS side B disk0” from slot 0 off the machine. Also remove “Release 4.5 CA/FS side B disk1” from slot 1off the machine.

Step 2   Place disk labeled “Release 3.5.5 CA/FS side B disk0” in slot 0. Also place disk labeled “Release 3.5.5 CA/FS side B disk1” in slot 1.

Step 3   Power on the machine

- For a Sunfire 1280 machine, please execute the following command from console:

<lom>poweron

- For other type of hardware, please use the power button to turn on the power.

Step 4   <hostname># platform start -copy

Step 5   <hostname># mv /etc/rc3.d/_S99platform /etc/rc3.d/S99platform

# From EMS side B

Step 1   Remove “Release 4.5 EMS side B disk0” from slot 0 off the machine. Also remove “Release 4.5 EMS side B disk1” from slot 1 off the machine.

Step 2   Place disk labeled “Release 3.5.5 EMS side B disk0” in slot 0. Also place disk labeled “Release 3.5.5 EMS side B disk1” in slot 1.

Step 3   Power on the machine

- For a Sunfire 1280 machine, please execute the following command from console:

<lom>poweron

- For other type of hardware, please use the power button to turn on the power.

Step 4   <hostname># platform start

Step 5   <hostname># mv /etc/rc3.d/_S99platform /etc/rc3.d/S99platform

# Task 10: Activate Omni SS7 A-links on CA/FS side B

# From CA/FS side B

Step 1   Activate SS7 Links on CA/FS side B.

1. <hostname># termhandler -node a7n1

- OMNI [date] #1:actv-slk:slk=< SS7 link on CA/FS side B >;

- Enter y to continue.

- Repeat above two steps for each inactive link

2. OMNI [date] #2:display-slk;

- Enter y to continue

- Verify that the state of each link is ACTIVE.

3. OMNI[date]#3:quit

# Task 11: Reset TTL value from DNS

Step 1   DNS changes:

From each Domain Name Server that is serving the upgraded BTS 10200, please change the Time-to-Live value from 600 seconds to 86400 seconds.

Please refresh the DNS cache and make sure the new TTL value is taking affect.

Step 2   Verify the DNS changes:

To verify the new TTL value has taken affect from Primary CA/FS, please login as root and use the actual Domain Name for parameter DNS_FOR_CA_MGCP_COM from /etc/opticall.cfg.

For an example:

- Verify the TTL for all 4 IPs is 86400 seconds

<hostname># nslookup -debug ca-mgcp.twc.cisco.com

- The TTL will be shown in the ANSWERS section as: ttl = <time to live in seconds>

…

ANSWERS:

->  ca-mgcp.twc.cisco.com

internet address = 13.200.10.11              Primary CA/FS physical IP

ttl = 86400 (1D)

->  ca-mgcp.twc.cisco.com

internet address = 13.200.10.12              Secondary CA/FS physical IP

ttl = 86400 (1D)

->  ca-mgcp.twc.cisco.com

internet address = 13.200.20.11              Primary CA/FS physical IP

ttl = 86400 (1D)

->  ca-mgcp.twc.cisco.com

internet address = 13.200.20.12              Secondary CA/FS physical IP

ttl = 86400 (1D)

# Task 12: Verify system status

Verify that the system is operating properly before you leave the site.

# From EMS side A

Step 1   Check for critical and major alarms

- Log in as CLI user

- CLI> show alarm severity=MAJOR;

- CLI> show alarm severity=CRITICAL;

  - Check report.  Please determine each major and critical condition, contact Cisco support if necessary.

Step 2   Perform full database audit between EMS and CA/FS:

- CLI> audit database;

  - Check audit report.  If there are any mismatches found, please contact Cisco support immediately.

- CLI> exit

Step 3   Performance Oracle DB audit between EMSs:

<hostname># su - oracle

<hostname>$ dbadm -C db

  - You should expect to see following:

Optical1:priems36:/opt/orahome$ dbadm -C db

OPTICAL1::Deftrandest is empty?         YES

OPTICAL1::dba_repcatlog is empty?     YES

OPTICAL1::Deferror is empty?              YES

OPTICAL1::Deftran is empty?               YES

OPTICAL1::Has no broken job?             YES

OPTICAL1::JQ Lock is empty?              YES

OPTICAL2::Deftrandest is empty?         YES

OPTICAL2::dba_repcatlog is empty?     YES

OPTICAL2::Deferror is empty?             YES

OPTICAL2::Deftran is empty?               YES

OPTICAL2::Has no broken job?            YES

OPTICAL2::JQ Lock is empty?             YES

You are connecting to OPTICAL1, remote DB is OPTICAL2

…

Number of tables to be checked: 195

Number of tables checked OK:    195

Number of tables out-of-sync:   0

  - If “Number of tables out-of-sync” is not 0, please contact Cisco support immediately.

<hostname>$ exit

Step 4   If you answered NO to any of the above questions (Step 1 through Step 4), Contact Cisco Support for assistance.

# Task 13: Post fallback clean up steps

Step 1	 Clean up checkpoints from both EMSs

<hostname># cd /opt/.upgrade

<hostname># \rm –rf *

Step 2	 Clean up files under /tmp directory

Step 3	 Check free disk space with command “du –sk” and remove files no longer needed.

You have completed half system fallback process successfully.

Appendix C

Setting up MTP and M3UA Configuration on the Signaling Gateway

This section describes the steps a user must complete to setup the MTP and M3UA configuration on an ITP signaling gateway. This must be completed a week before upgrading from release 3.5.5 to release 4.5.

# Requirements and Prerequisites

- Two Cisco ITP signaling gateways

# Preparation

You must perform the following list of tasks in preparation for setting up the MTP and M3UA configuration:

- Obtain the SS7 configuration from the Omni stack.

- Obtain the IP addresses of the ITP(s).

- Obtain the point code of the ITP(s).

- Obtain the IP addresses of the primary and secondary Cisco BTS 10200 call agent boxes.

- Physical link connection(s) should be made between the STP and the ITP. A linkset (and all associated links) should be added to the STP. A route should be added to the STP towards the Cisco BTS 10200 via the ITP. Note that details concerning the method to configure the STP are specific to the STP manufacturer.

Note: - All commands to be executed on the ITP are marked in Italics. - As with all IOS commands, a question mark ‘?’ can be typed at any part of the command sequence to get a listing of the possible user inputs. - For details related to the ITP please refer to ITP Configuration Guide http://www.cisco.com/en/US/products/sw/iosswrel/ps5012/products_feature_guide_book09186a00801752aa.html

# Task 1: Define MTP Variant

In this task, you will define an MTP variant to be used by the ITP.

Step 1   Log in as root to ITP

Step 2   cs7 variant ANSI

- The format is as follows: cs7 variant <MTP variant>

# Task 2: Define Point Code

In this task, you will define the point code of the ITP. In the A-link setup, ITPs will have the same point code as the Cisco BTS 10200. In the D-link setup, each ITP will have its own point code.

Step 1   cs7 point-code 20.20.20

- The format is as follows:  cs7 point-code <local point code>

# Task 3: Define Ethernet Configuration

Step 1   interface Ethernet0/0

ip address 10.89.232.9 255.255.255.0

ip directed-broadcast

no ip mroute-cache

full-duplex

!

interface Ethernet0/1

ip address 10.89.233.41 255.255.255.0

ip directed-broadcast

no ip mroute-cache

full-duplex

!

- The format is as follows:
>interface Ethernet <interface# / interface#> {see back of ITP for labeling 0/0 or 0/1}
>ip address <local ip address> <net mask>

# Task 4: Define SGMP (When SG Mated Pair is used)

In this task, you will define the SGMP which provides redundancy for the SG mated pair.

Step 1   cs7 sgmp 9101

local-ip 10.89.232.9

local-ip 10.89.233.41

!

- The format is as follows:

cs7sgmp  <Sigtran Adaption Layer> <local port #>

local-ip <local ip address 1>

local-ip <local ip address 2>

Step 2   cs7 mated-sg ITP2 9101

remote-ip 10.89.232.10

remote-ip 10.89.233.42

- The format is as follows:

cs7mated-sg  <name of mated signaling gateway> <remote port #>

remote-ip <remote  ip address 1>

remote-ip <remote ip address 2>

# Task 5: Define M3UA Port Number

In this task, you will define the local port number used by the M3UA layer.

Step 1   cs7 m3ua 2905

local-ip 10.89.232.9

local-ip 10.89.233.41

!

- The format is as follows:

cs7 <Sigtran Adaption Layer> <port #>

local-ip <local ip address 1>

local-ip <local ip address 2>

# Task 6: Define Application Server Process (ASP)

In this task, you will define the ASPs. Two ASPs are defined for each Cisco BTS 10200 call agent, one for the primary and one for secondary.

Step 1   cs7 asp caAspPri 11146 2905 m3ua

remote-ip 10.89.225.209

remote-ip 10.89.226.209

!

- This command defines the ASP for the primary call agent. The format is as follows:
cs7 asp <aspName> <remote IP port> <local IP port> <Sigtran Adaption layer>
remote-ip <remote IP address 1>
remote-ip <remote IP address 2>

Step 2   cs7 asp caAspSec 11146 2905 m3ua

remote-ip 10.89.225.210

remote-ip 10.89.226.210

!

- This command defines the ASP for the secondary call agent

# Task 7: Define SS7 Links

In this task, you will define SS7 links. The syntax of the commands is different for a 2651 and 7505. The equivalent Omni commands are given below.

CRTE-SLK: LSET=LNK0, LSET=LSET0, SLC=0, SPEED=56K, PORT=0, CHANNEL=1;

CRTE-SLK: LSET=LNK1, LSET=LSET1, SLC=0, SPEED=56K, PORT=48, CHANNEL=1;

For 2651:

Step 1   interface Serial0/0:0

no ip address

encapsulation mtp2

bandwidth 56

no ip route-cache

no ip mroute-cache

- The format is as follows:

Interface Serial Slot/Port

encapsulation <layer 2 protocol>

bandwidth <bandwith counted by Kb>

{Note that all ip related parameters are disabled by adding the ‘no’ prefix.}

Step 2   interface Serial0/1:0

no ip address

encapsulation mtp2

bandwidth 56

no ip route-cache

no ip mroute-cache

!

For 7507:

Step 1   interface Serial0/0/0

no ip address

encapsulation mtp2

bandwidth 56

no ip route-cache

no ip mroute-cache

- The format is as follows:

Interface Serial Slot/Bay/Port

encapsulation <layer 2 protocol>

{Note that all ip related parameters are disabled by adding the ‘no’ prefix.}

Step 2   interface Serial0/0/1

no ip address

encapsulation mtp2

bandwidth 56

no ip route-cache

no ip mroute-cache

!

# Task 8: Define Linkset

In this task you will define linksets. The equivalent Omni commands are given below.

CRTE-LSET: LSET=LSET0, PC=20-20-20, TYPE=A;

CRTE-LSET: LSET=LSET1, PC=30-30-30, TYPE=A;

20-20-20 and 30-30-30 are the STP point codes.

Step 1   cs7 linkset lset1 20.20.20

link 0 Serial0/0

!

Step 2   cs7 linkset lset2 30.30.30

link 0 Serial0/1

- The format is as follows:

cs7linkset <lset name> <adjacent point code>

link <slc number> <serial interface>

# Task 9: Define SS7 Route Sets

In this task you will define SS7 route sets to the DPCs.

Step 1   Configure routes for DPCs behind the STP (APC)

cs7 route-table system

update route 1.1.2 255.255.255 linkset lset1 priority 1

update route 1.1.2 255.255.255 linkset lset2 priority 1

update route 1.1.3 255.255.255 linkset lset1 priority 1

update route 1.1.3 255.255.255 linkset lset2 priority 1

!

- The format is as follows:
update route <PSTN DPC> <point code mask value> <linkset to APC> <route priority>

- These commands define two routes of equal priority to each DPC with point codes 1.1.2, and 1.1.3 through linksets lset1 and lset2. The equivalent Omni commands are given below.

CRTE-RSET:RSET=RSET2,PC=1-1-2,RTES=CLSET0;

ALW-RSET:RSET=RSET2;

CRTE-RSET:RSET=RSET3,PC=1-1-3,RTES=CLSET0;

ALW-RSET:RSET=RSET3;

Step 2   Check the status of the links to STP (APC) and DPC

show cs7 route

- This command displays the status of the links to STPs and DPCs. All links in the system routing table should have the status of “avail”.

Step 3   Save the routes

cs7 save route-table flash:routedata.txt

- This command saves the routes configured in steps 1 and 2 in the file named routedata.txt. The saved information can be used to configure another ITP.

# Task 10: Define Routing Key

In this task, you will create a routing key used by M3UA layer for each type of following services:  CNAM, LNP, IN1 Toll Free or AIN Toll Free, AC and AR.

Step 1   cs7 as btsAs m3ua

routing-key 100 7.7.7

asp caAspPrimary

asp caAspSecondary

!

- The format is as follows:
cs7 as <AS Name> <Sigtran Adaption layer>
routing-key <routing context value> <Cisco BTS 10200 point code>
asp <ASP name for primary Cisco BTS 10200>
asp <ASP name for secondary Cisco BTS 10200>

- 100 is the routing context value. This value must match the value defined later on in the Cisco BTS 10200 Call Agent.

- Additional routing key values can be added to the routing-key command to further filter the routing from the ITP to the Cisco BTS 10200. For details concerning adding additional keys please see ITP Configuration Guide http://www.cisco.com/en/US/products/sw/iosswrel/ps5012/products_feature_guide_book09186a00801752aa.html

Appendix D

Setting up SCTP configuration on the Signaling Gateway

This section describes the tasks you must perform to setup the SCTP configuration on the ITP signaling gateway. This must be completed a week before upgrading from release 3.5.5 to release 4.5.

Note: All commands to be executed on the ITP are marked in Italics.

# Task 1: Define SUA Port Number

In this task, you will define the local port number used by the SUA layer.

Step 1   Log in as root to ITP

Step 2   cs7 sua 14001

local-ip 10.89.232.9

local-ip 10.82.233.41

!

# Task 2: Define ASP

In this task, you will define the ASPs. Two ASPs are defined for each Cisco BTS 10200 AIN and POTS feature servers, one for the primary and one for secondary.

Step 1   cs7 asp fsAinAspPri 12205 14001 m3ua

remote-ip 10.89.225.209

remote-ip 10.89.226.209

!

- This command defines the ASP for the primary AIN feature server

- 14001 is the local port

- 12205 is the remote port

- 10.89.225.209 and 10.89.226.209 are the IP addresses of the primary Cisco BTS 10200

Step 2   cs7 asp fsAinAspSec 12205 14001 m3ua

remote-ip 10.89.225.210

remote-ip 10.89.226.210

!

- This command defines the ASP for the secondary AIN feature server.

- 14001 is the local port

- 12205 is the remote port

- 10.89.225.210 and 10.89.226.210 are the IP addresses of the secondary Cisco BTS 10200

Step 3  cs7 asp fsPotsAspPri 12235 14001 m3ua

remote-ip 10.89.225.209

remote-ip 10.89.226.209

!

- This command defines the ASP for the secondary POTS feature server.

- 14001 is the local port

- 12235 is the remote port

- 10.89.225.209 and 10.89.226.209 are the IP addresses of the primary Cisco BTS 10200

Step 4   cs7 asp fsPotsAspSec 12235 14001 m3ua

remote-ip 10.89.225.210

remote-ip 10.89.226.210

!

- This command defines the ASP for the secondary POTS feature server.

- 14001 is the local port

- 12235 is the remote port

- 10.89.225.210 and 10.89.226.210 are the IP addresses of the secondary Cisco BTS 10200

# Task 3: Define Routing Keys for Various Services

In this task, you will define routing keys for CNAM, LNP, IN1 toll free, AIN toll free, and AC and AR services. All the routing context values associated with each routing key must match the routing context values defined later on in the Cisco BTS 10200 AIN and POTS feature servers.

Step 1   cs7 as cnamAs sua

routing-key 103 2-42-10

asp fsPtcAspPri

asp fsPtcAspSec

!

- This command defines the routing key used for CNAM service

- 103 is the routing context value

Step 2   cs7 as lnpAs sua

routing-key 101 2-42-10

asp fsAinAspPri

asp fsAinAspSec

!

- This command defines the routing key used for LNP service

- 101 is the routing context value

Step 3   cs7 as in1TfAs sua

routing-key 102 2-42-10

asp fsAinAspPri

asp fsAinAspSec

!

- This command defines the routing key used for IN1 toll free service

###### 102 is the routing context value

Step 4   cs7 as in1TfAs sua

routing-key 104 2-42-10

asp fsAinAspPri

asp fsAinAspSec

!

- This command defines the routing key used for AIN toll free service

###### 104 is the routing context value

Step 5   cs7 as acarAs sua

routing-key 105 2-42-10

asp fsPtcAspPri

asp fsPtcAspSec

!

- This command defines the routing key used for AC and AR services

###### 105 is the routing context value

# Task 4: Define GTT Configuration

In this task, you will define the GTT configuration for AC and AR service. The equivalent Omni commands are given below. All the commands have a subsystem number of 251.

CREATE-GT:TT=251,NP=ISDN-TEL,DIG="972",PC=238-14-250,SSN=251,RI=DEF;

CREATE-GT:TT=251,NP=ISDN-TEL,DIG="469",PC=238-14-251,SSN=251,RI=DEF;

CREATE-GT:TT=251,NP=ISDN-TEL,DIG="214",PC=238-14-252,SSN=251,RI=DEF;

CREATE-REMSSN: PC=238-14-250,SSN=251;

CREATE-REMSSN: PC=238-14-251,SSN=251;

CREATE-REMSSN: PC=238-14-252,SSN=251;

Step 1   conf t

cs7 gtt selector acar_sel tt 251

gta 972 pcssn 238.14.250 pcssn ssn 251

gta 469 pcssn 238.14.251 pcssn ssn 251

gta 214 pcssn 238.14.252 pcssn ssn 251

!

Step 2   Save the GTT configuration

cs7 save gtt-table flash:gttdata.txt

- This command saves the GTT configuration in the file named gttdata.txt. This information can be used to load the GTT configuration during every reload.

Step 3   Load GTT configuration from a file

cs7gtt load gtt-table flash:gttdata.txt

- This command loads the GTT configuration from the gttdata.txt file during every reload.

# Task 5: Saving the Configuration

In this task, you will save all the ITP configuration changes made so far.

Step 1 wr

Appendix E

# Preparing Disks for Upgrade

This software upgrade will need 8 disks: 2 for each machine. Each set of 2 disks must have the same model number in order for disk mirroring to work.

The NIDS information required for disk preparation must be different from that used on the system to be upgraded.

# Task 1: Locate CD-ROM Discs

Please locate following 3 CD-ROM Discs:

- Locate CD-ROM Disc labeled as “BTS 10200 Application”

- Locate CD-ROM Disc labeled as “BTS 10200 Database”

- Locate CD-ROM Disc labeled as “BTS 10200 Oracle Engine”

# Task 2: Locate and label the Disks

# Label disks for EMS Side A

Locate two new disk drives identical to the machine to be upgraded and label the first disk drive as “Release 4.5 EMS side A disk0” and the second disk drive as “Release 4.5 EMS side A disk1”.  Please follow the steps below to prepare the two disk drives. The second disk drive will be used as the backup in case the first disk drive goes bad.

# Label Disks for EMS Side B

Locate two new disk drives identical to the machine to be upgraded and label the first disk drive as “Release 4.5 EMS side B disk0” and the second disk drive as “Release 4.5 EMS side B disk1”.  Please follow the steps below to prepare the two disk drives. The second disk drive will be used as the backup in case the first disk drive goes bad.

# Label Disks for CA/FS Side A

Locate two new disk drives identical to the machine to be upgraded and label the first disk drive as “Release 4.5 CA/FS side A disk0” and the second disk drive as “Release 4.5 CA/FS side A disk1”.  Please follow the steps below to prepare the two disk drives. The second disk drive will be used as the backup in case the first disk drive goes bad.

# Label Disks for CA/FS Side B

Locate two new disk drives identical to the machine to be upgraded and label the first disk drive as “Release 4.5 CA/FS side B disk0” and the second disk drive as “Release 4.5 CA/FS side B disk1”.  Please follow the steps below to prepare the two disk drives. The second disk drive will be used as the backup in case the first disk drive goes bad.

# Task 3: Disk slot lay out

- SunFire V240 disk slot lay out:

| Disk 2 | Disk 3 |  |
| --- | --- | --- |
| Disk 0 | Disk 1 | DVD-ROM |

- SunFire V440 disk slot lay out:

| Disk 3 | DVD-ROM |
| --- | --- |
| Disk 2 |  |
| Disk 1 |  |
| Disk 0 |  |

- Sunfire 1280 disk slot lay out:

|  | DVD-ROM |
| --- | --- |
|  | Disk 1 |
|  | Disk 0 |

# Task 4: Construct opticall.cfg

Step 1   Get a copy of Cisco BTS 10200 Software Release 4.5 Network Information Data Sheets (NIDS). Please follow the link below for NIDS and opticall.cfg information: http://lbj/push_targets1/ucdit/cc/td/doc/product/voice/bts10200/bts4_5/install/nids_45.xls

Step 2   Fill in NIDS information for the BTS system used for disk preparation. The NIDS information must be different from that used on the system to be upgraded.

Step 3   Get a copy of Cisco BTS 10200 Software Release 4.5 opticall.cfg

Step 4   Fill in parameters in opticall.cfg from NIDS and placed it on a server which will be accessible from the BTS system used for disk preparation.

# Task 5: Disk preparation

Repeat the steps in Task 5 to prepare the second disk.

Note:  Please perform disk preparation for each machine in parallel.

# For both EMS side A and B

Step 1   Locate a system with the identical hardware as the machine to be upgraded

Step 2   Place disk0 to slot 0 and disk 1 to slot 1

Step 3   Jumpstart the machine with Solaris 10 OS by following the jumpstart procedure: http://lbj/push_targets1/ucdit/cc/td/doc/product/voice/bts10200/bts4_5/install/sol10cdj.doc

Step 4   Configured the machine with 4/2 network configuration

Step 5   Stage Cisco BTS 10200 Software Release 4.5 to the /opt/Build directory.

From EMS Side B:

- Log in as root

- Put Disc labeled as BTS 10200 Application Disc in the CD-ROM drive

- Remove old files and mount CD-ROM drive

<hostname># cd /

<hostname># \rm -rf /opt/Build

<hostname># mkdir -p /cdrom

<hostname># mount -o ro -F hsfs /dev/dsk/c0t0d0s0 /cdrom

- Copy file from the CD-ROM to the /opt directory.

<hostname># cp –f /cdrom/K9-opticall.tar.gz  /opt

- Verify that the check sum values match with the values located in the “checksum.txt” file located on Application Disc

<hostname># cat /cdrom/checksum.txt

<hostname># cksum /opt/K9-opticall.tar.gz

<hostname># umount /cdrom

- Manually eject the CD-ROM and take out Disc from drive

- Put Disc labeled as BTS 10200 Database Disc in the CD-ROM drive

- Mount and copy file

<hostname># mount -o ro -F hsfs /dev/dsk/c0t6d0s0 /cdrom

<hostname># cp –f /cdrom/K9-btsdb.tar.gz  /opt

<hostname># cp –f /cdrom/K9-extora.tar.gz  /opt

- Verify that the check sum values match with the values located in the “checksum.txt” file located on Oracle CD-ROM.

<hostname># cat /cdrom/checksum.txt

<hostname># cksum /opt/K9-btsdb.tar.gz

<hostname># cksum /opt/K9-extora.tar.gz

<hostname># umount /cdrom

- Manually eject the CD-ROM and take out Disc from drive

- If you have Customer Built Oracle DB engine, please use the Disc provided by the customer. Otherwise, put Disc labeled as BTS 10200 Oracle Engine Disc in the CD-ROM drive

- Mount and copy file

<hostname># mount -o ro -F hsfs /dev/dsk/c0t6d0s0 /cdrom

<hostname># cp –f /cdrom/K9-oraengine.tar.gz  /opt

- Verify that the check sum values match with the values located in the “checksum.txt” file located on Oracle CD-ROM.

<hostname># cat /cdrom/checksum.txt

<hostname># cksum /opt/K9-oraengine.tar.gz

<hostname># umount /cdrom

- Manually eject the CD-ROM and take out Disc from drive

- Ftp files to EMS side A

<hostname># cd /opt

<hostname># sftp <EMS Side A hostname>

sftp> cd /opt

sftp> put K9-opticall.tar.gz

sftp> put K9-btsdb.tar.gz

sftp> put K9-extora.tar.gz

sftp> put K9-oraengine.tar.gz

sftp> exit

- Extract tar files.

<hostname># gzip -cd K9-opticall.tar.gz | tar -xvf -

<hostname># gzip -cd K9-btsdb.tar.gz | tar -xvf -

<hostname># gzip -cd K9-extora.tar.gz | tar -xvf -

<hostname># gzip -cd K9-oraengine.tar.gz | tar -xvf -

|  | Note: Each file will take up 5-10 minutes to extract. |
| --- | --- |

From EMS Side A:

- Log in as root

- Remove old files and extract new files

<hostname># cd /opt

<hostname># \rm -rf Build

<hostname># gzip -cd K9-opticall.tar.gz | tar -xvf -

<hostname># gzip -cd K9-btsdb.tar.gz | tar -xvf -

<hostname># gzip -cd K9-extora.tar.gz | tar -xvf -

<hostname># gzip -cd K9-oraengine.tar.gz | tar -xvf -

|  | Note: Each file will take up 5-10 minutes to extract. |
| --- | --- |

Step 6   ftp opticall.cfg from the server (where the file was placed in Task 4 above) and place it under /etc directory.

<hostname># /opt/Build/checkCFG

- Verify the information in /etc/opticall.cfg is free of errors.

Note: The EMS side A and side B installation must be started in parallel.

From both EMS:

Step 7   Install Cisco BTS 10200 Software Release 4.5 application software.

<hostname># /opt/Build/install.sh

  - Answer “y” when prompt. This installation process could take up to 1hour and 30 minutes.

  - Answer "y” when prompt for “reboot”

  - Wait for the system to boot up. Then Log in as root

Step 8   <hostname># mv /etc/rc3.d/S99platform /etc/rc3.d/_S99platform

Step 9   Shutdown the applications

<hostname># platform stop all

Step 10   Mirror the disks.

<hostname># /opt/setup/setup_mirror_ems

<hostname># sync;sync;reboot -- -r

  - Wait for the system to boot then log in to EMS Side B as root.

<hostname># /opt/setup/sync_mirror

Note: It takes 2 to 2.5 hours to complete the disk mirroring process on each machine. Please run “/opt/utils/resync_state” to check for the disk mirroring status. The display will show the resyncing in progress and reports resync completion.

Step 11   Remove all network interface configuration information from the machine:

<hostname>#  \rm /etc/hostname.*

Step 12   Shutdown and power off the machine then remove the disks to be used for upgrade.

<hostname># sync;sync;shutdown –i5 –g0 -y

# For both CA/FS side A and B

Step 1   Locate a system with the identical hardware as the machine to be upgraded

Step 2   Place disk0 to slot 0 and disk 1 to slot 1

Step 3   Jumpstart the machine with Solaris 10 OS by following the jumpstart procedure: http://lbj/push_targets1/ucdit/cc/td/doc/product/voice/bts10200/bts4_5/install/sol10cdj.doc

Step 4   Configured the machine with 4/2 network configuration

Step 5   Stage Cisco BTS 10200 Software Release 4.5 to the /opt/Build directory.

From CA/FS Side B:

- Log in as root

- Put Disc labeled as BTS 10200 Application Disc in the CD-ROM drive

- Remove old files and mount CD-ROM drive

<hostname># cd /

<hostname># \rm -rf /opt/Build

<hostname># mkdir -p /cdrom

<hostname># mount -o ro -F hsfs /dev/dsk/c0t0d0s0 /cdrom

- Copy file from the CD-ROM to the /opt directory.

<hostname># cp –f /cdrom/K9-opticall.tar.gz  /opt

- Verify that the check sum values match with the values located in the “checksum.txt” file located on Application Disc

<hostname># cat /cdrom/checksum.txt

<hostname># cksum /opt/K9-opticall.tar.gz

<hostname># umount /cdrom

- Manually eject the CD-ROM and take out Disc from drive

- Ftp files to CA/FS side A

<hostname># cd /opt

<hostname># sftp <CA/FS Side A hostname>

sftp> cd /opt

sftp> put K9-opticall.tar.gz

sftp> exit

- Extract tar files.

<hostname># gzip -cd K9-opticall.tar.gz | tar -xvf –

|  | Note: The file will take up 5-10 minutes to extract. |
| --- | --- |

From CA/FS Side A:

- Log in as root

- Remove old files and extract new files

<hostname># cd /opt

<hostname># \rm -rf Build

<hostname># gzip -cd K9-opticall.tar.gz | tar -xvf -

|  | Note: The file will take up 5-10 minutes to extract. |
| --- | --- |

From both CA/FS:

Step 6   ftp opticall.cfg from the server (where the file was placed in Task 4 above) and place it under /etc directory.

Step 7   Install BTSbase, BTSinst, BTSossh and BTShard packages:

- <hostname># cd /opt/Build

- <hostname># pkgadd –d . BTSbase

- Answer “y” when prompt

- <hostname># pkgadd –d . BTSinst

- Answer “y” when prompt

- <hostname># pkgadd –d . BTSossh

- Answer “y” when prompt

- <hostname># pkgadd –d . BTShard

- Answer “y” when prompt

- <hostname># cd /etc/rc3.d

- <hostname># mv S99platform _S99platform

- <hostname># sync;sync;reboot

- Wait for system to boot up and login as root.

Step 8   Update version information:

<hostname># echo “900-03.05.05.V02” > /opt/ems/utils/Version

Step 9   Mirror the disks.

<hostname># /opt/setup/setup_mirror_ca

<hostname># sync;sync;reboot -- -r

  - Wait for the system to boot then log in to EMS Side B as root.

<hostname># /opt/setup/sync_mirror

Note: It takes 2 to 2.5 hours to complete the disk mirroring process on each machine. Please run “/opt/utils/resync_state” to check for the disk mirroring status. The display will show the resyncing in progress and reports resync completion.

Step 10   Remove all network interface configuration information from the machine:

<hostname>#  \rm /etc/hostname.*

Step 11   Shutdown and power off the machine then remove the disks to be used for upgrade.

<hostname># sync;sync;shutdown –i5 –g0 -y

Appendix F

Add New Announcements

This procedure describes steps to record new announcements and place each announcement file onto proper media gateways.

# Task 1: Record Announcements

Step 1   "Your Anonymous Call Rejection service is now on. All incoming calls will be checked for number privacy before they are allowed to complete to your line". Set filename to "ann_id_540.au"

Step 2   "Your Anonymous Call Rejection service can not be successfully activated in your line. Please check with your service provider and try again.". Set filename to "ann_id_541.au"

Step 3   "Your Anonymous Call Rejection service is now off. Incoming calls will not be checked for number privacy status". Set filename to "ann_id_542.au"

Step 4   "Your Anonymous Call Rejection service can not be successfully deactivated in your line". Set filename to "ann_id_543.au"

# Task 2: Place Announcements

# From Active EMS

Step 1   Log in as CLI user.

Step 2   Show a list of announcement trunk groups.

CLI> show trunk-grp tg-type=annc;

Step 3   For each trunk group ID shown in step 2, show a list of Media Gateways being used for announcements. These media gateways are where you would place the announcement files.

CLI> show trunk tgn-id=[Trunk Group ID];

Appendix G

SS7 Migration

# Initial State

The initial state has existing SS7 connections with:

- Two type “A” linksets – each with one link

- The two linksets form a combined linkset

- Routes to any number of other network elements (such as, Class 5, SCP, STP) via the combined linkset

Figure 1: Initial state

BTS-1

OPC=7.7.7

STP

PC=20.20.20

PC=30.30.30

CA-A

CA-B

SS7 Network

A-Link

A-Link

C-Link

STP

# Transition State

Step 1   Configure the ITP in active-active mode

- Connect two ITPs as SG Mated Pair

- Assign each ITP with its own point code and acts as an STP which connects to STPs in the SS7 network via D-links.

Step 2   Add E-links between BTS 10200 and ITP to serve as a backup in case all A-link connections become unavailable.

Step 3   Add E-links to all routes between BTS and ITP

Figure 2: E-links added

STP

BTS-1

OPC=7.7.7

PC=20.20.20

PC=30.30.30

CA-A

CA-B

SS7 Network

PC=5.5.5

PC=4.4.4

E-Link

E-Link

A-Link

A-Link

SG Mated

Pair

STP

All routes via combined linkset

Add E-links to routes

C-Link

Step 4   Add D-links between ITP and STP

Step 5   Add D-links to all routes between ITP and STP

Figure 3: D-links added

STP

BTS-1

OPC=7.7.7

PC=20.20.20

PC=30.30.30

CA-A

CA-B

SS7 Network

PC=5.5.5

PC=4.4.4

E-Link

E-Link

A-Link

A-Link

SG Mated

Pair

D-Link

D-Link

C-Link

C-Link

STP

Step 6   Remove A-links from routes

Figure 4: A-links removed

STP

BTS-1

OPC=7.7.7

PC=20.20.20

PC=30.30.30

CA-A

CA-B

SS7 Network

PC=5.5.5

PC=4.4.4

E-Link

E-Link

A-Link

A-Link

SG Mated

Pair

D-Link

D-Link

C-Link

C-Link

STP

Step 7   Delete A-links and linksets

Figure 5: A-links and linksets are deleted

STP

BTS-1

OPC=7.7.7

PC=20.20.20

PC=30.30.30

CA-A

CA-B

SS7 Network

PC=5.5.5

PC=4.4.4

E-Link

E-Link

SG Mated

Pair

D-Link

D-Link

C-Link

C-Link

STP

# Final State

Step 8   Delete E-links

Step 9   Enable Sigtran

Figure 6: Final state

BTS-1

OPC=7.7.7

DPC1=1.1.2

DPC2=1.1.3

PC=20.20.20

Capability PC=40.40.40

PC=30.30.30

Capability PC=40.40.40

CA-A

CA-B

SS7 Network

ITP1 (STP1)

PC=5.5.5

ITP2 (STP2)

PC=4.4.4

Sigtran

Sigtran

SG Mated

STP Pair

D-Link

D-Link

SSP

1.1.2

SSP

1.1.3

STP

C-Link

C-Link

STP

# ITP configuration information

A configuration example for the SG Mate Pair Profile is given below for the SS7 network illustrated in Figure-6.

For Cisco recommended ITP configuration information, please refer to the ITP documentation posted on CCO.

########################################################################

#

# ITP1 - The first ITP in the sg-pair (each ITP in the sg-pair functions as an STP).

#

Current configuration : 3470 bytes

!

version 12.2

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname ITP1

!

boot-start-marker

boot system flash c2600-itp-mz.topsail_s_nightly_040915

boot-end-marker

!

redundancy inter-device

!

enable secret 5 $1$XCoU$j0Y2wFRoks2pocHa1gHhi0

enable password cisco

!

memory-size iomem 20

ip subnet-zero

!

ip domain-name cisco.com

ip name-server 10.0.0.6

!

#

# Note that for the Mated STP Pair configuration (SG Mated Pair) configuration,

# the local point code value is 5.5.5, which is different than the BTS OPC and the

# other ITP (ITP2) that makes up the SG Mated Pair.

#

cs7 variant ANSI

cs7 point-code 5.5.5

!

controller T1 1/0/0

framing esf

clock source internal

linecode b8zs

channel-group 0 timeslots 1 speed 56

channel-group 1 timeslots 2 speed 56

!

controller T1 1/0/1

framing esf

clock source internal

linecode b8zs

channel-group 0 timeslots 1 speed 56

channel-group 1 timeslots 2 speed 56

!

controller T1 1/0/2

framing esf

clock source internal

linecode b8zs

channel-group 0 timeslots 1 speed 56

!

controller T1 1/0/3

shutdown

!

controller T1 1/0/4

shutdown

!

interface Serial0/0:0

description connect to link 0 of STP 20-20-20

no ip address

encapsulation mtp2

no clns route-cache

!

interface Serial0/1:0

description connect to link 0 of STP 30-30-30

no ip address

encapsulation mtp2

no clns route-cache

interface FastEthernet0/0

ip address 10.0.1.54 255.255.0.0

speed auto

full-duplex

no clns route-cache

!

interface FastEthernet0/1

ip address 10.128.1.239 255.255.0.0

speed auto

full-duplex

no clns route-cache

#

# In the Mated STP Pair configuration, define the local-peer which is the local

# definition for the C-link connection between the two ITPs that make up the

# redundant STP pair.

#

cs7 local-peer 7000

local-ip 10.0.1.54

local-ip 10.128.1.239

#

# Linkset definitions:

# Note: the number after ‘link’ represents SLC

#

cs7 linkset lset1chn 20.20.20

link 0 Serial0/0:0

!

cs7 linkset lset2chn 30.30.30

link 0 Serial0/1:0

#

# C-link linkset definition. Here the point code value and IP information for

# the mated-sg is defined. Note that the local IP information is defined in

# the local-peer definition above.

#

cs7 linkset c-link 4.4.4

link 0 sctp 10.0.1.55 10.128.1.240 7000 7000

#

# SS7 ROUTE DEFINITIONS

#

# In the following entries, note:

# 1) All of the routes towards all DPCs are configured with equal priority

#     when using lset1chn or lset2chn.

# 2) There are lower priority routes towards each destination across the C-link.

# 3) Routing towards the Capability PC of the adjacent STPs is treated as

#      if the Capability PC is a DPC beyond the STP.

#

cs7 route-table system

update route 1.1.2 255.255.255 linkset lset1chn priority 1

update route 1.1.2 255.255.255 linkset lset2chn priority 1

update route 1.1.3 255.255.255 linkset lset1chn priority 1

update route 1.1.3 255.255.255 linkset lset2chn priority 1

# Lower priority C-link routes

update route 1.1.2 255.255.255 linkset c-link priority 2

update route 1.1.3 255.255.255 linkset c-link priority 2

# Routing to Capability Pt Codes of adjacent STPs

update route 40.40.40 255.255.255 linkset lset1chn priority 1

update route 40.40.40 255.255.255 linkset lset2chn priority 1

#

# With the mated-sg (Mated STP Pair configuration), you must also define a

# connection between the ITPs to pass Sigtran specific state information and other

# data. This is done by defining the local IP information in the "cs7 sgmp"

# configuration and the peer IP information in the "cs7 mated-sg" configuration.

#

#

cs7 sgmp 9101

local-ip 10.0.1.54

local-ip 10.128.1.239

!

cs7 mated-sg ITP2 9101

remote-ip 10.0.1.55

remote-ip 10.128.1.240

#

# The M3UA definition that declares local IP addresses and port #

#

cs7 m3ua 2905

local-ip 10.0.1.54

local-ip 10.128.1.239

keepalive 2000

#

# Here as with all configurations, there are *at least* two ASPs defined

# for each AS (one for the primary BTS node and one for the Secondary).

# In reality, there will be at least one for each "User Part" on the BTS 10200.

# So if you have a TCAP Service going over SUA and ISUP traffic, you will

# have a total of at least four ASPs. Primary ISUP, Secondary ISUP, Primary

# TCAP Service, Secondary TCAP Service.

#

# Note that the remote port value of 11146 is configured on the BTS 10200 in the

# platform.cfg file (as an SGA command line argument). 2905 is the local port

# value. The remote IP addresses are the BTS IP addresses. They are also obtained

# through the FQDN that is an SGA command line argument.

#

cs7 asp PriCaIsupAsp 11146 2905 m3ua

remote-ip 10.0.1.5

remote-ip 10.128.1.2

!

cs7 asp SecCaIsupAsp 11146 2905 m3ua

remote-ip 10.0.1.6

remote-ip 10.128.1.3

#

# Note that the routing key is a very simple one. It has a

# routing context of 1 defined, the DPC (BTS OPC) of 7.7.7 defined

# and a service indicator of ISUP defined. This means that all traffic

# coming from the SS7 Service Provider Network that has a DPC of 7.7.7

# and a service indicator of ISUP will be sent to either PrimaryBtsIsupAsp

# or SecondaryBtsIsupAsp (depending on which one is active).

#

# The traffic mode is always set to override (not loadshared)

#

cs7 as CaIsupAs m3ua

routing-key 1 7.7.7 si isup

asp PriCaIsupAsp

asp SecCaIsupAsp

traffic-mode override

#

# The SUA definition that declares local IP addresses and port #

#

cs7 sua 14001

local-ip 10.0.1.54

local-ip 10.128.1.239

keepalive 2000

#

# Here we are defining an ASPs that will process AIN related traffic. Note that

# the remote port 12205 is a TSA command line parameter in platform.cfg on the

# BTS. 14001 is the local port number.

#

#

cs7 asp PriFsAinAsp 12205 14001 sua

remote-ip 10.0.1.5

remote-ip 10.128.1.2

!

cs7 asp SecFsAinAsp 12205 14001 sua

remote-ip 10.0.1.6

remote-ip 10.128.1.3

!

cs7 asp PriFsPtcAsp 12235 14001 sua

remote-ip 10.0.1.5

remote-ip 10.128.1.2

!

cs7 asp SecFsPtcAsp 12235 14001 sua

remote-ip 10.0.1.6

remote-ip 10.128.1.3

!

#

# The following AS is defined for LNP related message flows. The routing context

# value is 4, the DPC (BTS OPC) is 7.7.7, the service indicator is SCCP and the

# Subsystem number is 247. This means that any message received from the SS7

# Service Provider that has a DPC of 7.7.7, a service indicator of SCCP and an

# SSN of 247 will be sent to either PrimaryBtsAinAsp or SecondaryBtsAinAsp

# (depending on which one is active).

#

#

cs7 as FsAinLnpAs sua

routing-key 4402 7.7.7 si sccp ssn 247

asp PriFsAinAsp

asp SecFsAinAsp

traffic-mode override

!

cs7 as FsAin800TAs sua

routing-key 4404 7.7.7 si sccp ssn 254

asp PriFsAinAsp

asp SecFsAinAsp

traffic-mode override

!

cs7 as FsAin800AAs sua

routing-key 4403 7.7.7 si sccp ssn 248

asp PriFsAinAsp

asp SecFsAinAsp

traffic-mode override

!

cs7 as FsPtcCnamAs sua

routing-key 4404 7.7.7 si sccp ssn 232

asp PriFsPtcAsp

asp SecFsPtcAsp

traffic-mode override

!

cs7 as FsPtcAcarAs sua

routing-key 4405 7.7.7 si sccp ssn 251

asp PriFsPtcAsp

asp SecFsPtcAsp

traffic-mode override

!

cs7 gtt load flash:gttdata.txt

!

#######################################################################

#

# ITP2 --The second ITP in the sg-pair (each ITP in the sg-pair acts as an STP).

#

#  For the ITP2 configuration, please refer to the comments that were made in 
#  in the ITP1 configuration.

#

Current configuration : 4054 bytes

!

version 12.2

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname ITP2

!

boot-start-marker

boot system flash 2600/c2600-itp-mz.topsail_s_nightly_040915

boot-end-marker

!

redundancy inter-device

!

enable secret 5 $1$B6u2$gI4fFgjOQo5XppDSWJDfI.

enable password cisco

!

memory-size iomem 20

ip subnet-zero

!

ip domain-name cisco.com

ip name-server 10.0.0.6

!

#

# Note that for the Mated STP Pair configuration (SG Mated Pair) configuration,

# the local point code value is 4.4.4, which is different than the BTS OPC and the

# other ITP (ITP1) that makes up the SG Mated Pair.

#

cs7 variant ANSI

cs7 point-code 4.4.4

!

controller T1 1/0/0

framing esf

clock source internal

linecode b8zs

channel-group 0 timeslots 1 speed 56

channel-group 1 timeslots 2 speed 56

!

controller T1 1/0/1

framing esf

clock source internal

linecode b8zs

channel-group 0 timeslots 1 speed 56

channel-group 1 timeslots 2 speed 56

!

controller T1 1/0/2

framing esf

clock source internal

linecode b8zs

channel-group 0 timeslots 1 speed 56

!

controller T1 1/0/3

shutdown

!

controller T1 1/0/4

shutdown

!

interface FastEthernet0/0

ip address 10.0.1.55 255.255.0.0

speed auto

full-duplex

no clns route-cache

!

interface FastEthernet0/1

ip address 10.128.1.240 255.255.0.0

speed auto

full-duplex

no clns route-cache

!

interface Serial0/0:0

description connect to link 1 of STP 20-20-20

no ip address

encapsulation mtp2

no clns route-cache

interface Serial0/1:0

description connect to link 1 of STP 30-30-30

no ip address

encapsulation mtp2

no clns route-cache

!

#

# local-peer definition

#

cs7 local-peer 7000

local-ip 10.0.1.55

local-ip 10.128.1.240

!

#

# Linkset definitions. Note: the number after ‘link’ represents SLC

#

cs7 linkset lset1chn 20.20.20

link 1 Serial0/0:0

!

cs7 linkset lset2chn 30.30.30

link 1 Serial0/1:0

!

#

# C-link linkset definition.

#

cs7 linkset c-link 5.5.5

link 0 sctp 10.0.1.54 10.128.1.239 7000 7000

!

cs7 route-table system

update route 1.1.2 255.255.255 linkset lset1chn priority 1

update route 1.1.2 255.255.255 linkset lset2chn priority 1

update route 1.1.3 255.255.255 linkset lset1chn priority 1

update route 1.1.3 255.255.255 linkset lset2chn priority 1

# C-link routes

update route 1.1.2 255.255.255 linkset c-link priority 2

update route 1.1.3 255.255.255 linkset c-link priority 2

# Routing to Capability Pt Codes of adjacent STPs

update route 40.40.40 255.255.255 linkset lset1chn priority 1

update route 40.40.40 255.255.255 linkset lset2chn priority 1

!

cs7 sgmp 9101

local-ip 10.0.1.55

local-ip 10.128.1.240

!

cs7 mated-sg ITP1 9101

remote-ip 10.0.1.54

remote-ip 10.128.1.239

cs7 m3ua 2905

local-ip 10.0.1.55

local-ip 10.128.1.240

!

cs7 asp PriCaIsupAsp 11146 2905 m3ua

remote-ip 10.0.1.5

remote-ip 10.128.1.2

!

cs7 asp SecCaIsupAsp 11146 2905 m3ua

remote-ip 10.0.1.6

remote-ip 10.128.1.3

!

cs7 as CaIsupAs m3ua

routing-key 1 7.7.7 si isup

asp PriCaIsupAsp

asp SecCaIsupAsp

traffic-mode override

network-appearance 1

!

cs7 sua 14001

local-ip 10.0.1.55

local-ip 10.128.1.240

keepalive 2000

!

cs7 asp PriFsAinAsp 12205 14001 sua

remote-ip 10.0.1.5

remote-ip 10.128.1.2

!

cs7 asp SecFsAinAsp 12205 14001 sua

remote-ip 10.0.1.6

remote-ip 10.128.1.3

!

cs7 asp PriFsPtcAsp 12235 14001 sua

remote-ip 10.0.1.5

remote-ip 10.128.1.2

!

cs7 asp SecFsPtcAsp 12235 14001 sua

remote-ip 10.0.1.6

remote-ip 10.128.1.3

!

cs7 as FsAinLnpAs sua

routing-key 4402 7.7.7 si sccp ssn 247

asp PriFsAinAsp

asp SecFsAinAsp

traffic-mode override

!

cs7 as FsAin800TAs sua

routing-key 4401 7.7.7 si sccp ssn 254

asp PriFsAinAsp

asp SecFsAinAsp

traffic-mode override

!

cs7 as FsAin800AAs sua

routing-key 4403 7.7.7 si sccp ssn 248

asp PriFsAinAsp

asp SecFsAinAsp

traffic-mode override

!

cs7 as FsPtcCnamAs sua

routing-key 4404 7.7.7 si sccp ssn 232

asp PriFsPtcAsp

asp SecFsPtcAsp

traffic-mode override

!

cs7 as FsPtcAcarAs sua

routing-key 4405 7.7.7 si sccp ssn 251

asp PriFsPtcAsp

asp SecFsPtcAsp

traffic-mode override

!

cs7 gtt load flash:gttdata.txt

!

# BTS 10200 Provisioning for the SG Mated Pair

Note that the local IP addresses and ports are determined by command line arguments that are passed to the SGA process and TSA processes when they start up. This information is contained in CA/FS platform.cfg. For instance, an example SGA command line is:

Args=-t 1 -h crit-aSYS12CA.ipclab.cisco.com -p 11146 -mdldir ../mdl -mdltracedir ../mdltrace -mdltestmode 0 -mdlloadmdo 0 -mdltriggertimer 200 -mdlgarbagetimer 5146 -resetcics 1 -fcmtimer 900 -fcmparalleljobs 4

In this list of arguments, the –h argument: crit-aSYS12CA.ipclab.cisco.com, is a FQDN that resolves to two local IP addresses. In most cases the FQDN can be viewed in the /etc/hosts file. To determine the IP addresses to which the FQDN resolves, type nslookup <FQDN>.

########################################################################

#

# CA Configuration

#

add ca-config type=MGCP-INIT-TERMS;value=160;datatype=integer;

add ca-config type=MGCP-INIT-DURATION;value=5;datatype=integer;

add ca-config type=MGCP-ICMP-PING-RETRANSMIT-DURATION; value=5; datatype=integer;

add ca-config type=MGCP-ICMP-PING-RETRY-COUNT;value=5;datatype=integer;

add ca-config type=MGCP-MAX-UNREACH-COUNT;value=5;datatype=integer;

add ca-config type=MGCP-MAX-FAULT-COUNT;value=5;datatype=integer;

add ca-config type=MGCP-ADM-RESP-TIME;value=300;datatype=integer;

add ca-config type=MGCP-SIG-TOS-LOWDELAY;value=Y;datatype=boolean;

add ca-config type=MGCP-SIG-TOS-PRECEDENCE;value=1;datatype=integer;

add ca-config type=MGCP-SIG-TOS-RELIABILITY;value=Y;datatype=boolean;

add ca-config type=MGCP-SIG-TOS-THROUGHPUT;value=Y;datatype=boolean;

#

# CA & FS

#

add call-agent id=CA146; tsap-addr-sidea=bts-prica; mgw-monitoring-enabled=N;

add feature-server id=FSAIN205; tsap-addr-sidea=bts-prica:11205; type=AIN;

add feature-server id=FSPTC235; tsap-addr-sidea=bts-prica:11235; type=PTC;

#

# Sigtran components

#

add user-part-variant id=ANSISS7_GR317;

#

# Note for the Mated STP Pair configuration, there are two SGs defined for

# redundancy. They are essentially mated STPs. This is different than the A, F, or E

# link configurations which derive redundancy at the SGP level.

#

add sg id=sg1; description=Signaling gateway 1;

add sg id=sg2; description=Signaling gateway 2;

#

# In the Mated STP Pair configuration The SG-GRP has two SGs defined in the

# SG-GRP. The A, F, and E link configurations *must* only have one SG defined in

# an SG-GRP.

#

add sg-grp id=sg-grp1; sg1-id=sg1; sg2-id=sg2 description=SG group 1;

#

# In the Mated STP Pair configuration, there is *only* one SGP per SG. Note that

# the two SGPs defined here have a one-to-one correspondence to the SGs that were

# defined above. This is in contrast to the A, F, and E link configurations

# which must have two SGPs per SG.

#

add sgp id=sg1-sgp1 ; sg-id=sg1; description=SG process 1 for sg1;

add sgp id=sg2-sgp1 ; sg-id=sg2; description=SG process 1 for sg2;

add opc id=opc1; point-code=7-7-7; description=OPC; point-code-type=ANSI_CHINA;

add dpc id=dpc1; point-code=1-1-2; description=DPC 1; point-code-type=ANSI_CHINA;

add dpc id=dpc2; point-code=1-1-3; description=DPC 2; point-code-type=ANSI_CHINA;

#

# THE ISUP ROUTING KEYS

#

add routing-key id=rk1; opc-id=opc1; sg-grp-id=sg-grp1; si=ISUP; rc=1; platform-id=CA146;

add call-ctrl-route id=dpc1-route1; dpc-id=dpc1; routing-key-id=rk1; si=isup; user-part-variant-id= ANSISS7_GR317

add call-ctrl-route id=dpc2-route1; dpc-id=dpc2; routing-key-id=rk1; si=isup; user-part-variant-id= ANSISS7_GR317;

add sctp-assoc-profile id=sctp-prof;

#

# THE SCTP ASSOCIATIONS

# Note that the chosen id name in this statement reflects the fact that this is the

# sctp association for SGP1 of SG1

#

add sctp-assoc id=sg1-sgp1-sctp; sgp-id=sg1-sgp1; sctp-assoc-profile-id=sctp-prof; platform-id=CA146; remote-port=2905; remote-tsap-addr1=10.0.1.54; remote-tsap-addr2=10.128.1.239; dscp=AF11; ip-tos-precedence=ROUTINE;

add sctp-assoc id=sg2-sgp1-sctp; sgp-id=sg2-sgp1; sctp-assoc-profile-id=sctp-prof; platform-id=CA146; remote-port=2905; remote-tsap-addr1=10.0.1.55; remote-tsap-addr2=10.128.1.240; dscp=AF11; ip-tos-precedence=ROUTINE;

#

# Dial plan profile

#

add digman-profile id=pretrans;

add digman id=pretrans; rule=1; match-string=^*; replace-string=&; match-noa=any; replace-noa=VSC;

add digman id=pretrans; rule=2; match-string=^#; replace-string=&; match-noa=any; replace-noa=VSC;

add digman-profile id=ani_20;

add digman id=ani_20; rule=1; match-string=^20; replace-string=none;

add dial-plan-profile id=dp-1; nanp-dial-plan=Y; description=NA dial plan profile; dnis-digman-id=pretrans; ani-digman-id=ani_20;

#

# SS7 TG

#

add ss7-ansi-tg-profile ID=ansi-tg-prof;

add trunk-grp ID=1; call_agent_id=CA146; tg_type=SS7; direction=BOTH; tg_profile_id=ansi-tg-prof; call-ctrl-route-id=dpc1-route1; dial-plan-id=dp-1; description=TG to DPC 1;  MGCP_PKG_TYPE=T;

add trunk-grp ID=2; call_agent_id=CA146; tg_type=SS7; direction=BOTH; tg_profile_id=ansi-tg-prof; call-ctrl-route-id=dpc2-route1; dial-plan-id=dp-1; description=TG to DPC 2;  MGCP_PKG_TYPE=T;

#

# MGW

#

add mgw-profile id=as5300-prof; vendor=Cisco; mgcp-hairpin-supp=n; MGCP_RSIPSTAR_SUPP=N; MGCP_TERM_INIT_LEVEL=0; RBK_ON_CONN_SUPP=N; MGCP_VERSION=MGCP_1_0; mgcp-max2-retries=3; fax-t38-camode-supp=Y; mgcp-keepalive-interval=60; mgcp-keepalive-retries=10; mgcp-t-tran=400; mgcp-max1-retries=2; mgcp-t-longtran=5; mgcp-default-pkg=NONE; MGCP_3WAY_HSHAKE_SUPP=N; mgw_type=AS5300; PC_MPTIME_SUPP=N;

MGCP_VERSION=MGCP_1_0; PC_MPTIME_SUPP=N;

add mgw id=va-5350-23; tsap-addr=va-5350-23.hrndevtest.cisco.com; call-agent-id=CA146; mgw-profile-id=as5300-prof; type=TGW;

#

# SS7 terminations and trunks

#

add termination prefix=S3/DS1-4/; port-start=1; port-end=31; type=trunk; mgw-id=va-5350-23;

add termination prefix=S3/DS1-5/; port-start=1; port-end=31; type=trunk; mgw-id=va-5350-23;

add trunk cic-start=1; cic-end=31; tgn-id=1; mgw-id=va-5350-23; termination-prefix=S3/DS1-4/; termination-port-start=1; termination-port-end=31;

add trunk cic-start=1; cic-end=31; tgn-id=2; mgw-id=va-5350-23; termination-prefix=S3/DS1-5/; termination-port-start=1; termination-port-end=31;

#

# SS7 routes, route guides and destinations

#

add route id=dpc1-route; tg_selection=RR; tgn1_id=1;

add route id=dpc2-route; tg_selection=RR; tgn1_id=2;

add route-guide id=dpc1-rg; policy-type=ROUTE; policy-id=dpc1-route;

add route-guide id=dpc2-rg; policy-type=ROUTE; policy-id=dpc2-route;

add destination dest-id=dpc1-dest; call-type=LOCAL; route-type=ROUTE; route-guide-id=dpc1-rg;

add destination dest-id=dpc2-dest; call-type=LOCAL; route-type=ROUTE; route-guide-id=dpc2-rg;

######################################

# TCAP/SUA Provisioning for FSAIN, FSPTC

######################################

add sctp-assoc id=sg1-sgp1-sctp-ain; sgp-id=sg1-sgp1; sctp-assoc-profile-id=sctp-prof; platform-id=FSAIN205; remote-port=14001; remote-tsap-addr1=10.0.1.54; remote-tsap-addr2=10.128.1.239; dscp=AF11; ip-tos-precedence=ROUTINE;

add sctp-assoc id=sg2-sgp1-sctp-ain; sgp-id=sg2-sgp1; sctp-assoc-profile-id=sctp-prof; platform-id=FSAIN205; remote-port=14001; remote-tsap-addr1=10.0.1.55; remote-tsap-addr2=10.128.1.240; dscp=AF11; ip-tos-precedence=ROUTINE;

add sctp-assoc id=sg1-sgp1-sctp-ptc; sgp-id=sg1-sgp1; sctp-assoc-profile-id=sctp-prof; platform-id=FSPTC235; remote-port=14001; remote-tsap-addr1=10.0.1.54; remote-tsap-addr2=10.128.1.239; dscp=AF11; ip-tos-precedence=ROUTINE;

add sctp-assoc id=sg2-sgp1-sctp-ptc; sgp-id=sg2-sgp1; sctp-assoc-profile-id=sctp-prof; platform-id=FSPTC235; remote-port=14001; remote-tsap-addr1=10.0.1.55; remote-tsap-addr2=10.128.1.240; dscp=AF11; ip-tos-precedence=ROUTINE;

add sccp-nw id=1;NET_IND=NATIONAL;SUB_SVC=NATIONAL;HOP_COUNT=3;

add subsystem-profile id=SS_LNP;  platform-id=FSAIN205; description=LNP subsystem;

add subsystem-profile id=SS_800A; platform-id=FSAIN205; description=AIN 800 subsystem;

add subsystem-profile id=SS_800T; platform-id=FSAIN205; description=IN1 800 subsystem;

add subsystem-profile id=SS_CNAM; platform-id=FSAIN205; description=CNAM subsystem;

add subsystem-profile id=SS_ACAR; platform-id=FSPTC235; description=ACAR subsystem;

add subsystem id=SS_LNP;  opc-id=opc1; local-ssn=247; remote-ssn=247; sccp-nw-id=1; SCCP_VERSION=ANS92; TCAP_VERSION=ANS92; APPLICATION_VERSION=AIN01;

add subsystem id=SS_800A; opc-id=opc1; local-ssn=248; remote-ssn=248; sccp-nw-id=1; SCCP_VERSION=ANS92; TCAP_VERSION=ANS92; APPLICATION_VERSION=AIN01;

add subsystem id=SS_CNAM; opc-id=opc1; local-ssn=232; remote-ssn=232; sccp-nw-id=1; SCCP_VERSION=ANS92; TCAP_VERSION=ANS92; APPLICATION_VERSION=IN1;

add subsystem id=SS_800T; opc-id=opc1; local-ssn=254; remote-ssn=254; sccp-nw-id=1; SCCP_VERSION=ANS92; TCAP_VERSION=ANS92; APPLICATION_VERSION=IN1;

add subsystem id=SS_ACAR; opc-id=opc1; local-ssn=251; remote-ssn=251; sccp-nw-id=1; SCCP_VERSION=ANS92; TCAP_VERSION=ANS92; APPLICATION_VERSION=IN1;

add routing-key id=rk_lnp;  opc-id=opc1; sg-grp-id=sg_grp1; si=SCCP; rc=4402; PLATFORM_ID=FSAIN205; ssn-id=SS_LNP;

add routing-key id=rk_800a; opc-id=opc1; sg-grp-id=sg_grp1; si=SCCP; rc=4403; PLATFORM_ID=FSAIN205; ssn-id=SS_800A;

add routing-key id=rk_cnam; opc-id=opc1; sg-grp-id=sg_grp1; si=SCCP; rc=4404; PLATFORM_ID=FSAIN205; ssn-id=SS_CNAM;

add routing-key id=rk_800t; opc-id=opc1; sg-grp-id=sg_grp1; si=SCCP; rc=4401; PLATFORM_ID=FSAIN205; ssn-id=SS_800T;

add routing-key id=rk_acar; opc-id=opc1; sg-grp-id=sg_grp1; si=SCCP; rc=4405; PLATFORM_ID=FSPTC235; ssn-id=SS_ACAR;

##########################################

# Provisioned DPC is the STP Capability Pt Code

##########################################

add dpc id=stp_cap_pc; point-code=40-40-40; point-code-type=ANSI_CHINA; description=Capability Point Code of STPs

add feature fname=LNP; feature-server-id=FSAIN205; description=Local number portability; tdp1=COLLECTED_INFORMATION; tid1=LNP_TRIGGER; ttype1=R;

add ported-office-code digit-string=301-612; in-call-agent=n;

add CA-Config type=DEFAULT-LNP-SLHR-ID; datatype=string;  value=slhr_lnp;

add slhr-profile id=slhr_800t; description=Service Logic Host Routing Table for IN1 800 Service;

add slhr-profile id=slhr_lnp;  description=Service Logic Host Routing Table for AIN LNP Service;

add slhr-profile id=slhr_800a; description=Service Logic Host Routing Table for AIN 800 Service;

add slhr-profile id=slhr_cnam; description=Service Logic Host Routing Table for IN1 CNAM Service;

add slhr-profile id=slhr_acar; description=Service Logic Host Routing Table for ACAR Service;

add slhr id=slhr_800t; opc-id=opc1; dpc-id=stp_cap_pc; ssn-id=SS_800T; gtt-req=Y; tt=254; GTT_ADDR_TYPE=CDPN; GTT_ADDR=3;

add slhr id=slhr_lnp;  opc-id=opc1; dpc-id=stp_cap_pc; ssn-id=SS_LNP;  gtt-req=Y; tt=11; GTT_ADDR_TYPE=CDPN; GTT_ADDR=3;

add slhr id=slhr_800a; opc-id=opc1; dpc-id=stp_cap_pc; ssn-id=SS_800A; gtt-req=Y; tt=8; GTT_ADDR_TYPE=CDPN; GTT_ADDR=3;

add slhr id=slhr_cnam; opc-id=opc1; dpc-id=stp_cap_pc; ssn-id=SS_CNAM; gtt-req=Y; tt=5; GTT_ADDR_TYPE=CLGN; GTT_ADDR=3;

add slhr id=slhr_acar; opc-id=opc1; dpc-id=stp_cap_pc; ssn-id=SS_ACAR; gtt-req=Y; tt=251; GTT_ADDR_TYPE=CDPN; GTT_ADDR=10;

add sccp-route opc-id=opc1; dpc-id=stp_cap_pc; ssn-id=SS_800T; rk-id=rk_800t;

add sccp-route opc-id=opc1; dpc-id=stp_cap_pc; ssn-id=SS_800A; rk-id=rk_800a;

add sccp-route opc-id=opc1; dpc-id=stp_cap_pc; ssn-id=SS_LNP;  rk-id=rk_lnp;

add sccp-route opc-id=opc1; dpc-id=stp_cap_pc; ssn-id=SS_CNAM; rk-id=rk_cnam;

add sccp-route opc-id=opc1; dpc-id=stp_cap_pc; ssn-id=SS_ACAR; rk-id=rk_acar;

add sccp-route SSN_ID=SS_ACAR; OPC_ID=opc1; DPC_ID=dpc1; RK_ID=rk_acar

add sccp-route SSN_ID=SS_ACAR; OPC_ID=opc1; DPC_ID=dpc2; RK_ID=rk_acar

add pop ID=50901; STATE=tx; COUNTRY=US; TIMEZONE=CDT; LOCAL_7D_DIALING=Y; ITP=N; ZERO_MINUS=LEC; BLOCK_EAWOPIC=Y; CNAM_OPTION=EXT_LIDB; PIC2_REQD=N; MY_LRN=4692559999; TREAT_IMS_ANONYMOUS=N; OPC_ID=opc1; ZERO_PLUS_LOCAL=N

###################################################

# Control network entities in-service for ANSI SS7

###################################################

control trunk-grp id=1; mode=forced; target-state=ins;

control trunk-grp id=2; mode=forced; target-state=ins;

equip trunk-termination tgn-id=1; cic=all;

equip trunk-termination tgn-id=2; cic=all;

control trunk-termination tgn-id=1; cic=all; target-state=INS;  mode=FORCED;

control trunk-termination tgn-id=2; cic=all; target-state=INS;  mode=FORCED;

control sctp-assoc id=sg1-sgp1-sctp; mode=forced; target-state=INS;

control sctp-assoc id=sg2-sgp1-sctp; mode=forced; target-state=INS;

control sctp-assoc id=sg1-sgp1-sctp-ain; mode=forced; target-state=INS;

control sctp-assoc id=sg2-sgp1-sctp-ain; mode=forced; target-state=INS;

control sctp-assoc id=sg1-sgp1-sctp-ptc; mode=forced; target-state=INS;

control sctp-assoc id=sg2-sgp1-sctp-ptc; mode=forced; target-state=INS;

control subsystem id=SS_800T; opc-id=opc; target-state=UIS;  mode=FORCED;

control subsystem id=SS_LNP;  opc-id=opc; target-state=UIS;  mode=FORCED;

control subsystem id=SS_800A; opc-id=opc; target-state=UIS;  mode=FORCED;

control subsystem id=SS_CNAM; opc-id=opc; target-state=UIS;  mode=FORCED;

control subsystem id=SS_ACAR; opc-id=opc; target-state=UIS;  mode=FORCED;

##########################################################

# Status commands

##########################################################

#status trunk-grp id=1;

#status trunk-grp id=2;

#status trunk-termination tgn-id=1; cic=all;

#status trunk-termination tgn-id=2; cic=all;

#status sctp-assoc id=sg1-sgp1-sctp;

#status sctp-assoc id=sg2-sgp1-sctp;

#status sctp-assoc id=sg1-sgp1-sctp-ain;

#status sctp-assoc id=sg2-sgp1-sctp-ain;

#status sctp-assoc id=sg1-sgp1-sctp-ptc;

#status sctp-assoc id=sg2-sgp1-sctp-ptc;

#status subsystem id=SS_800T; opc-id=opc;

#status subsystem id=SS_LNP;  opc-id=opc;

#status subsystem id=SS_800A; opc-id=opc;

#status subsystem id=SS_CNAM; opc-id=opc;

#status subsystem id=SS_ACAR; opc-id=opc;

Appendix H

Provisioning Release 4.5 Specific Configuration on the Cisco BTS 10200 Call Agent

This chapter shows a template cli file.

# Preparation

- Obtain the IP addresses of the ITP(s).

Template CLI file (to be used for creating cfg_ss7 file)

# define signaling gateway components, which include signaling gateway, signaling

# gateway process, and signaling gateway group. These components are common to call # agent and feature servers and need not be defined multiple times for each platform.

###### add sg id=sg-1; description=Signaling Gateway 1;

add sg id=sg-2; description=Signaling Gateway 2;

add sgp id=sgp-1; sg-id=sg-1; description=Signaling Gateway Process ITP 1;

add sgp id=sgp-2; sg-id=sg-2; description=Signaling Gateway Process ITP 2;

add sg-grp id=sg-grp-1; sg1-id=sg-1; sg2-id=sg-2; description=Signaling Gateway Group consisting of both sg-1 and sg-2 for redundancy;

# define SCTP association profile and SCTP association to each of the signaling gateway # processes. Remote TSAP addresses 1 and 2 are the ITP’s redundant IP addresses.

add sctp-assoc-profile id=sctp-prof;

add sctp-assoc id=ca-sctp-assoc1; sgp-id=sgp-1; sctp-assoc-profile-id=sctp-prof; platform-id=CAxxx; remote-tsap-addr1=<ITP 1 First IP Address>; remote-port=2905; dscp=AF11; ip-tos-precedence=ROUTINE;  remote-tsap-addr2=<ITP 1 Second IP Address>;

add sctp-assoc id=ca-sctp-assoc2; sgp-id=sgp-2; sctp-assoc-profile-id=sctp-prof; platform-id=CAxxx; remote-tsap-addr1=<ITP 2 First IP Address>; remote-port=2905; dscp=AF11; ip-tos-precedence=ROUTINE; remote-tsap-addr2=<ITP 2 Second IP Address>;

# define the ISUP variant. In release 3.5.5 only ANSI variant was supported.

add user-part-variant id=ANSISS7_GR317;

# define the OPC of Cisco BTS 10200. The equivalent Omni command is given below.

# CRTE-OSPC:PC=7-7-7, NI=NAT0; Point code 7-7-7 is an example

add opc id=opc; point-code=7-7-7;

# define a routing key used by the Cisco BTS 10200 call agent. The routing context

# defined by the value of the parameter “rc” must be same as the value defined in the ITP # (Appendix C, Task 10, Step 1).

add routing-key id=rk1; opc-id=opc; sg-grp-id=sg-grp-1; si=ISUP; rc=100; platform-id=CAxxx;

# Bring the SCTP associations in service.

control sctp-assoc id=ca-sctp-assoc1; target-state=ins; mode=forced;

control sctp-assoc id=ca-sctp-assoc2; target-state=ins; mode=forced;

# define DPC. The equivalent Omni commands are given below.

# CRTE-RSET:RSET=RSET2, PC=1-1-2, RTES=CLSET0;

# ALW-RSET:RSET=RSET2;

# CRTE-RSET:RSET=RSET3, PC=1-1-3, RTES=CLSET0;

# ALW-RSET:RSET=RSET3; Point code 1-1-2 and 1-1-3 are examples

add dpc id=dpc1; point-code=1-1-2; description=DPC 1;

add dpc id=dpc2; point-code=1-1-3; description=DPC 2;

.

# define call control routes to the DPCs defined in the previous task.

add call-ctrl-route id=route-dpc1; dpc-id=dpc1; routing-key-id=rk1; si=ISUP; user-part-variant-id=ANSISS7_GR317;

add call-ctrl-route id=route-dpc2; dpc-id=dpc2; routing-key-id=rk1; si=ISUP; user-part-variant-id=ANSISS7_GR317;

Appendix I

Provisioning Release 4.5 Specific Configuration on the Cisco BTS 10200 Feature Server

This chapter describes the template clis for setting up SCCP configuration on the BTS feature server.

# define the one SCTP association to each of the signaling gateway processes for AIN

# and POTS feature servers. The SCTP association profile is already defined as part of

# the call agent configuration.

# define SCTP associations for the AIN feature server.

add sctp-assoc id=ain-sctp-assoc1; sgp-id=sgp-1; sctp-assoc-profile-id=sctp-prof; platform-id=FSAINzzz; remote-tsap-addr1=<IP Address>; remote-port=14001; dscp=AF11; ip-tos-precedence=ROUTINE; remote-tsap-addr2=<IP Address>;

add sctp-assoc id=ain-sctp-assoc2; sgp-id=sgp-2; sctp-assoc-profile-id=sctp-prof; platform-id=FSAINzzz; remote-tsap-addr1=<IP Address>; remote-port=14001; dscp=AF11; ip-tos-precedence=ROUTINE; remote-tsap-addr2=<IP Address>;

# define SCTP associations for the POTS feature server.

add sctp-assoc id=pots-sctp-assoc1; sgp-id=sgp-1; sctp-assoc-profile-id=sctp_prof; platform-id=FSPTCyyy; remote-tsap-addr1=<IP Address>; remote-port=14001; dscp=AF11; ip-tos-precedence=ROUTINE;  remote-tsap-addr2=<IP Address>;

add sctp-assoc id=pots-sctp-assoc2; sgp-id=sgp-2; sctp-assoc-profile-id=sctp_prof; platform-id=FSPTCyyy; remote-tsap-addr1=<IP Address>; remote-port=14001; dscp=AF11; ip-tos-precedence=ROUTINE; remote-tsap-addr2=<IP Address>;

# bring the SCTP associations in service.

control sctp-assoc id=ain-sctp-assoc1; target-state=ins; mode=forced;

control sctp-assoc id=ain-sctp-assoc2; target-state=ins; mode=forced;

control sctp-assoc id=pots-sctp-assoc1; target-state=ins; mode=forced;

control sctp-assoc id=pots-sctp-assoc2; target-state=ins; mode=forced;

.

# define a SCCP network.

add sccp-nw id=1; net-ind=NATIONAL; sub-svc=NATIONAL; hop-count=15;

# change the old pop table to include the opc-id. If opc-id is not added in the pop table for # Rel4.x, the related features (AC, AR, Toll-Free, CNAM, LNP) will not work.

change pop id=1; opc-id=opc;

# define a CNAM service. The equivalent Omni commands are given below. All of them # have a subsystem number of 232.

# CREATE-GT:TT=5,NP=ISDN-TEL,DIG="201",PC=238-3-0,SSN=232,RI=DEF;

# CREATE-GT:TT=5,NP=ISDN-TEL,DIG="202",PC=238-3-0,SSN=232,RI=DEF;

# CREATE-GT:TT=5,NP=ISDN-TEL,DIG="203",PC=238-3-0,SSN=232,RI=DEF;

# CREATE-GT:TT=5,NP=ISDN-TEL,DIG="205",PC=238-3-0,SSN=232,RI=DEF;

add dpc id=stp1; point-code=238-3-0; description=STP for CNAM service;

add subsystem-grp id=ss_cnam; tcap-version=ANS92;platform-id=FSPTCyyy; description=CNAM Subsystem;

add subsystem id=ss_cnam; opc_id=opc; local-ssn=232; remote-ssn=232; sccp-nw-id=1; sccp-version=ANS92; application-version=IN1;

# the routing context defined by the value of the parameter “rc” must be same as the

# value defined in the ITP (Appendix D, Task 3, Step 1).

add routing-key id=rk_cnam; opc-id=opc; subsystem-grp-id=ss_cnam; sg-grp-id=sg-grp-1; si=SCCP; rc=103; platform-id=FSPTCyyy;

add sccp-route opc-id=opc; dpc-id=stp1; subsystem-grp-id=ss_cnam; rk-id=rk_cnam;

add slhr-profile id=slhr_cnam; description=Service Logic Host Routing Table for IN1 CNAM service;

add slhr id=slhr_cnam; opc-id=opc; dpc-id=stp1; subsystem-grp-id=ss_cnam; gtt-req=Y; tt=5; gtt_addr_type=CDPN;

add ca-config type=DEFAULT-LIDB-SLHR-ID; datatype=string; value=slhr_cnam;

# this command puts the subsystem in service

control subsystem-grp id=ss_cnam; mode=forced; target-state=uis;

# define a LNP service. The equivalent Omni commands are given below. All of them

# have a subsystem number of 247.

# CREATE-GT:TT=11,NP=ISDN-TEL,DIG="334",PC=238-153-0,SSN=247,RI=DEF;

# CREATE-GT:TT=11,NP=ISDN-TEL,DIG="678",PC=238-153-0,SSN=247,RI=DEF;

# CREATE-GT:TT=11,NP=ISDN-TEL,DIG="404",PC=238-153-0,SSN=247,RI=DEF;

# CREATE-GT:TT=11,NP=ISDN-TEL,DIG="706",PC=238-153-0,SSN=247,RI=DEF;

# CREATE-GT:TT=11,NP=ISDN-TEL,DIG="770",PC=238-153-0,SSN=247,RI=DEF;

add dpc id=stp2; point-code=238-153-0; description=STP for LNP service;

add subsystem-grp id=ss_lnp; tcap-version=ANS92; platform-id=FSAINzzz; description=LNP Subsystem;

add subsystem id=ss_lnp; opc_id=opc; local-ssn=247; remote-ssn=247; sccp-nw-id=1; sccp-version=ANS92; application-version=AIN01;

# the routing context defined by the value of the parameter “rc” must be same as the

# value defined in the ITP (Appendix D, Task 3, Step 2).

add routing-key id=rk_lnp; opc-id=opc; subsystem-grp-id=ss_lnp; sg-grp-id=sg-grp; si=SCCP; rc=101; platform-id=FSAINzzz;

add sccp-route opc-id=opc; dpc-id=stp2; subsystem-grp-id=ss_lnp; rk-id=rk_lnp;

add slhr-profile id=slhr_lnp; description=Service Logic Host Routing Table for AIN LNP service;

add slhr id=slhr_lnp; opc-id=opc; dpc-id=stp2; subsystem-grp-id=ss_lnp; gtt-req=Y; tt=11;

add ca-config type=DEFAULT-LNP-SLHR-ID; datatype=string; value=slhr_lnp;

# this command puts the subsystem in service

control subsystem-grp id=ss_lnp; mode=forced; target-state=uis;

# define IN1 toll free service. The equivalent Omni commands are given below. All of

# them have a subsystem number of 254.

# CREATE-GT:TT=254,NP=ISDN-TEL,DIG="800",PC=238-14-

# 254,SSN=254,RI=DEF;

# CREATE-GT:TT=254,NP=ISDN-TEL,DIG="866",PC=238-14-

# 254,SSN=254,RI=DEF;

# CREATE-GT:TT=254,NP=ISDN-TEL,DIG="888",PC=238-14-

# 254,SSN=254,RI=DEF;

# CREATE-GT:TT=254,NP=ISDN-TEL,DIG="877",PC=238-14-

# 254,SSN=254,RI=DEF;

add dpc id=stp3; point-code=238-14-254; description=STP for Toll Free service;

add subsystem-grp id=ss_tf; tcap-version=ANS92; platform-id=FSAINzzz; description=Toll Free Subsystem;

add subsystem id=ss_tf; opc_id=opc; local-ssn=254; remote-ssn=254; sccp-nw-id=1; sccp-version=ANS92; application-version=IN1;

# the routing context defined by the value of the parameter “rc” must be same as the

# value defined in the ITP (Appendix D, Task 3, Step 3).

add routing-key id=rk_tf; opc-id=opc; subsystem-grp-id=ss_tf; sg-grp-id=sg-grp; si=SCCP; rc=102; platform-id=FSAINzzz;

add sccp-route opc-id=opc; dpc-id=stp3; subsystem-grp-id=ss_tf; rk-id=rk_tf;

add slhr-profile id=slhr_tf; description=Service Logic Host Routing Table for IN1 Toll Free service;

add slhr id=slhr_tf; opc-id=opc; dpc-id=stp3; subsystem-grp-id=ss_tf; gtt-req=Y; tt=254;

add ca-config type=DEFAULT-TOLL-FREE-SLHR-ID; datatype=string; value=slhr_tf;

# this command puts the subsystem in service

control subsystem-grp id=ss_tf; mode=forced; target-state=uis;

# define an AIN toll free service. The equivalent Omni commands are given below. All

# of them have a subsystem number of 248.

# CREATE-GT:TT=8,NP=ISDN-TEL,DIG="800",PC=238-14-253,SSN=248,RI=DEF;

# CREATE-GT:TT=8,NP=ISDN-TEL,DIG="866",PC=238-14-253,SSN=248,RI=DEF;

# CREATE-GT:TT=8,NP=ISDN-TEL,DIG="888",PC=238-14-253,SSN=248,RI=DEF;

# CREATE-GT:TT=8,NP=ISDN-TEL,DIG="877",PC=238-14-253,SSN=248,RI=DEF;

add dpc id=stp4; point-code=238-14-253; description=STP for Toll Free service;

add subsystem-grp id=ss_aintf; tcap-version=ANS92; platform-id=FSAINzzz; description=Toll Free Subsystem;

add subsystem id=ss_aintf; opc_id=opc; local-ssn=248; remote-ssn=248; sccp-nw-id=1;  sccp-version=ANS92; application-version=IN1;

# The routing context defined by the value of the parameter “rc” must be same as the

# value defined in the ITP (Appendix D, Task 3, Step 4).

add routing-key id=rk_aintf; opc-id=opc; subsystem-grp-id=ss_ aintf; sg-grp-id=sg-grp; si=SCCP; rc=104; platform-id=FSAINzzz;

add sccp-route opc-id=opc; dpc-id=stp4; subsystem-grp-id=ss_aintf; rk-id=rk_aintf;

add slhr-profile id=slhr_aintf; description=Service Logic Host Routing Table for IN1 Toll Free service;

add slhr id=slhr_aintf; opc-id=opc; dpc-id=stp4; subsystem-grp-id=ss_aintf; gtt-req=Y; tt=8;

add ca-config type=DEFAULT-TOLL-FREE-SLHR-ID; datatype=string; value=slhr_aintf;

# this command puts the subsystem in service.

control subsystem-grp id=ss_aintf; mode=forced; target-state=uis;

# define AC and AR services. The equivalent Omni commands are defined below. All of # them have a subsystem number of 251.

#CREATE-GT:TT=251,NP=ISDN-TEL,DIG="972",PC=238-14-250,SSN=251,RI=DEF;

#CREATE-GT:TT=251,NP=ISDN-TEL,DIG="469",PC=238-14-251,SSN=251,RI=DEF;

#CREATE-GT:TT=251,NP=ISDN-TEL,DIG="214",PC=238-14-252,SSN=251,RI=DEF;

#CREATE-GT:TT=251,NP=ISDN-TEL,DIG="919",PC=238-10-201,SSN=251,RI=DEF;

#CREATE-REMSSN: PC=238-14-250,SSN=251;

#CREATE-REMSSN: PC=238-14-251,SSN=251;

#CREATE-REMSSN: PC=238-14-252,SSN=251;

#CREATE-REMSSN: PC=238-10-201,SSN=251;

#CREATE-GT:TT=251,NP=ISDN-TEL,DIG="512",PC=2-42-10,SSN=251,RI=DEF;

add dpc id=itp1; point-code=7-7-7; description=ITP for AC and AR services;

add subsystem-grp id=ss_acar; tcap-version=ANS92; platform-id=FSPTCyyy; description=AC AR Subsystem;

add subsystem id=ss_acar; opc_id=opc; local-ssn=251; remote-ssn=251; sccp-nw-id=1;  sccp-version=ANS92; application-version=IN1;

# the routing context defined by the value of the parameter “rc” must be same as the

# value defined in the ITP (Appendix D, Task 3, Step 5).

add routing-key id=rk_acar; opc-id=opc; subsystem-grp-id=ss_acar; sg-grp-id=sg-grp; si=SCCP; rc=105; platform-id=FSPTCyyy;

add sccp-route opc-id=opc; dpc-id=itp1; subsystem-grp-id=ss_acar; rk-id=rk_acar;

add slhr-profile id=slhr_acar; description=Service Logic Host Routing Table for IN1 Toll Free service;

add slhr id=slhr_acar; opc-id=opc; dpc-id=itp1; subsystem-grp-id=ss_acar; gtt-req=Y; tt=251;

add ca-config type=ACAR -SLHR-ID; datatype=string; value=slhr_acar;

add dpc id=sw_mask1; point-code=238-14-0; description=ITP for AC and AR services;

add dpc id=sw1; point-code=238-10-201; description=ITP for AC and AR services;

add sccp-route opc-id=opc; dpc-id=sw_mask; subsystem-grp-id=ss_acar; rk-id=rk_acar;

add sccp-route opc-id=opc; dpc-id=sw1; subsystem-grp-id=ss_acar; rk-id=rk_acar;

# this command puts the subsystem in service.

control subsystem-grp id=ss_acar; mode=forced; target-state=uis;

Appendix J

Check Oracle Database Replication and Error Correction

Perform the following steps on the Active EMS side A to check the Oracle database and replication status.

# Check Oracle DB replication status

# From EMS side A

Step 1   Log in as root.

Step 2   Log in as oracle.

<hostname># su – oracle

Step 3   Enter the command to check replication status and compare contents of tables on the side A and side B EMS databases:

<hostname>$ dbadm –C rep

Step 4   Verify that “Deferror is empty?” is “YES”.

OPTICAL1::Deftrandest is empty?           YES

OPTICAL1::dba_repcatlog is empty?       YES

OPTICAL1::Deferror is empty?              YES Make sure it is “YES”

OPTICAL1::Deftran is empty?                  YES

OPTICAL1::Has no broken job?               YES

OPTICAL1::JQ Lock is empty?                YES

OPTICAL2::Deftrandest is empty?          YES

OPTICAL2::dba_repcatlog is empty?      YES

OPTICAL2::Deferror is empty?             YES Make sure it is “YES”

OPTICAL2::Deftran is empty?                YES

OPTICAL2::Has no broken job?              YES

OPTICAL2::JQ Lock is empty?               YES

Step 5   If the “Deferror is empty?” is “NO”, please try to correct the error using steps in “Correct replication error” below. If you are unable to clear the error or if any of the individual steps fails, please contact Cisco Support.

# Correct replication error

|  | Note: You must run the following steps on standby EMS side B first, then on active EMS side A. |
| --- | --- |

# From EMS Side B

Step 1   Log in as root

Step 2   <hostname># su – oracle

Step 3   <hostname>$ dbadm –C db

Step 4   For each table that is out of sync, please run the following step:

<hostname>$ dbadm -A copy -o <OWNER> -t <TABEL NAME>

- Enter “y” to continue

- Please contact Cisco Support if the above command fails.

Step 5   <hostname>$ dbadm –A truncate_deferror

- Enter “y” to continue

# From EMS Side A

Step 1   <hostname>$ dbadm –A truncate_deferror

- Enter “y” to continue

Step 2   Re-verify that “Deferror is empty?” is “YES” and none of tables is out of sync.

<hostname>$dbadm –C db

OPTICAL1::Deftrandest is empty?           YES

OPTICAL1::dba_repcatlog is empty?       YES

OPTICAL1::Deferror is empty?              YES  Make sure it is “YES”

OPTICAL1::Deftran is empty?                  YES

OPTICAL1::Has no broken job?               YES

OPTICAL1::JQ Lock is empty?                YES

OPTICAL2::Deftrandest is empty?          YES

OPTICAL2::dba_repcatlog is empty?      YES

OPTICAL2::Deferror is empty?             YES  Make sure it is “YES”

OPTICAL2::Deftran is empty?                YES

OPTICAL2::Has no broken job?              YES

OPTICAL2::JQ Lock is empty?               YES

Appendix K

4/2 Port Configuration Reference Chart

Appendix L

CORBA Installation

This procedure describes how to install the OpenORB Common Object Request Broker Architecture (CORBA) application on Element Management System (EMS) of the Cisco BTS 10200 Softswitch.

Note: This installation process is to be used for both EMS side A and EMS side B.

Caution This CORBA installation will remove existing CORBA application on EMS machines. Once you have executed this procedure, there is no backout. Do not start this procedure until you have proper authorization. If you have questions, contact Cisco TAC.

# Task 1: Install OpenORB CORBA Application

### Remove Installed OpenORB Application

Step 1   Log in as root to EMS.

Step 2   Enter the following command to remove the existing OpenORB CORBA

<hostname># pkgrm BTScis

  - Answer “y” when prompt

<hostname># pkgrm BTSoorb

  - Answer “y” when prompt

Step 3   Enter the following command to verify that the CORBA application is removed:

<hostname># pgrep cis3

The system will respond by displaying no data, or by displaying an error message. This verifies that the CORBA application is removed.

### Install OpenORB Packages

The CORBA application files are available for installation once the Cisco BTS 10200 Softswitch is installed.

Step 1   Log in as root to EMS

Step 2   <hostname># cd /opt/Build

Step 3   <hostname># cis-install.sh

You should expect to following message:

The NameService & CIS modules listen on a specific host interface. ***WARNING*** This host name or IP address MUST resolve on the CORBA client machine in the OSS. Otherwise, communication failures may occur. Enter the host name or IP address [ local hostname ]:

Step 4   Confirm the “local hostname” is the machine you are on, then press return:

Enter the host name or IP address [ local hostname ]:

  - Answer “y” when prompt

Step 5   It will take about 5-8 minutes for the installation to complete.

Step 6   Verify CORBA Application is running On EMS:

<hostname># init q

<hostname># pgrep ins3

Note: System will respond by displaying the Name Service process ID, which is a number between 2 and 32,000 assigned by the system during CORBA installation. By displaying this ID, the system confirms that the ins3 process was found and is running.

<hostname># pgrep cis3

Note: The system will respond by displaying the cis3 process ID, which is a number between 2 and 32,000 assigned by the system during CORBA installation. By displaying this ID, the system confirms that the cis3 process was found and is running.

Step 7   If you do not receive both of the responses described in Step 6, or if you experience any verification problems, do not continue. Contact your system administrator. If necessary, call Cisco TAC for additional technical assistance.

Appendix M

Prepare Side A System for Fallback

This procedure describes the steps to prepare the shared memory on side A systems in anticipation of half system fallback if it is required.

# Task 1: Restore CA/FS side A shared memory data

# From CA/FS side A

Step 1   Log in as root.

Step 2   Clean up shared memory:

<hostname># \rm -rf /opt/OptiCall/FSAINzzz/bin/data

<hostname># \rm -rf /opt/OptiCall/FSPTCyyy/bin/data

<hostname># \rm -rf /opt/OptiCall/CAxxx/bin/data

Step 3    Restore shared memory:

<hostname># cp -rfp /opt/databackup/data.CA /opt/OptiCall/CAxxx/bin/data

<hostname># cp -rfp /opt/databackup/data.FSAIN /opt/OptiCall/FSAINzzz/bin/data

<hostname># cp -rfp /opt/databackup/data.FSPTC /opt/OptiCall/FSPTCyyy/bin/data

# Task 2: Clean up EMS side A shared memory data

# From EMS side A

Step 1   Log in as root.

Step 2   Clean up shared memory:

<hostname># \rm -rf /opt/ems/bin/data

<hostname># \rm -rf /opt/bdms/bin/data

# Task 3: Restore EMS side A oracle DB replication

# From EMS side A

Step 1   Start Oracle DB:

<hostname>$ platform start -i oracle

Step 2   Restore Oracle DB replication:

<hostname># su - oracle

<hostname>$ cd /opt/oracle/admin/utl

<hostname>$ rep_toggle -s optical1 -t set_duplex

- Answer “y” when prompt

- Answer “y” again when prompt

Where -- optical1 is alphabet “optical” followed by a numeric 1.

<hostname>$ exit

Step 3   Stop Oracle DB to clean up DB connections:

<hostname># platform stop -i oracle