# Activity 7.4.2: Challenge DHCP and NAT Configuration

## Topology Diagram

## Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| R1 | S0/0/0 | 172.16.0.1 | 255.255.255.252 |
|  | Fa0/0 | 172.16.10.1 | 255.255.255.0 |
|  | Fa0/1 | 172.16.11.1 | 255.255.255.0 |
| R2 | S0/0/0 | 172.16.0.2 | 255.255.255.252 |
|  | S0/0/1 | 209.165.201.1 | 255.255.255.252 |
|  | Fa0/0 | 172.16.20.1 | 255.255.255.0 |
| ISP | S0/0/1 | 209.165.201.2 | 255.255.255.252 |

## Learning Objectives

Upon completion of this lab, you will be able to:

- Prepare the network

- Perform basic router configurations

- Configure a Cisco IOS DHCP server

- Configure static and default routing

- Configure static NAT

- Configure dynamic NAT with a pool of addresses

- Configure NAT overload

## Scenario

In this lab, configure the IP address services using the network shown in the topology diagram. If you need assistance, refer back to the basic DHCP and NAT configuration lab. However, try to do as much on your own as possible.

Task 1: Perform Basic Router Configurations

Configure the R1, R2, and ISP routers according to the following guidelines:

- Configure the device hostname.

- Disable DNS lookup.

- Configure a privileged EXEC mode password.

- Configure a message-of-the-day banner.

- Configure a password for the console connections.

- Configure a password for all vty connections.

- Configure IP addresses on all routers. The PCs receive IP addressing from DHCP later in the lab.

- Enable RIPv2 on R1 and R2. Do not advertise the 209.165.200.224/27 network.

For all devices:

enable

conf t

no ip domain-lookup

enable secret class

banner motd $Authorized Access Only!$

!

line con 0

logging synchronous

password cisco

login

!

line vty 0 4

password cisco

login

end

copy run start

R1:

hostname R1

int fa0/0

ip address 172.16.10.1 255.255.255.0

no shut

int fa0/1

ip address 172.16.11.1 255.255.255.0

no shut

int s0/0/0

ip address 172.16.0.1 255.255.255.252

clock rate 125000

no shut

!

router rip

version 2

network 172.16.0.0

no auto-summary

R2:

hostname R2

int fa0/0

ip address 172.16.20.1 255.255.255.0

no shut

int s0/0/0

ip address 172.16.0.2 255.255.255.252

no shut

int s0/0/1

ip address 209.165.201.1 255.255.255.252

clock rate 125000

no shut

!

router rip

version 2

network 172.16.0.0

no auto-summary

ISP:

hostname ISP

int s0/0/1

ip address 209.165.201.2 255.255.255.252

no shut

!

Task 2: Configure a Cisco IOS DHCP Server

Configure R1 as the DHCP server for the two directly attached LANs.

Step 1. Exclude statically assigned addresses.

Exclude the first three addresses from each pool.

R1(config)#ip dhcp excluded-address 172.16.10.1 172.16.10.3

R1(config)#ip dhcp excluded-address 172.16.11.1 172.16.11.3

Step 2. Configure the DHCP pool.

- Create two DHCP pools. Name one of them R1_LAN10 for the 172.16.10.0/24 network, and name the other R1_LAN11 for the 172.16.11.0/24 network.

- Configure each pool with a default gateway and a simulated DNS at 172.16.20.254.

R1(config)#ip dhcp pool R1_LAN10

R1(dhcp-config)#network 172.16.10.0 255.255.255.0

R1(dhcp-config)#default-router 172.16.10.1

R1(dhcp-config)#dns-server 172.16.20.254

R1(dhcp-config)#ip dhcp pool R1_LAN11

R1(dhcp-config)#network 172.16.11.0 255.255.255.0

R1(dhcp-config)#default-router 172.16.11.1

R1(dhcp-config)#dns-server 172.16.20.254

Step 3. Verify the DHCP configuration.

R1#show ip dhcp binding

IP address       Client-ID/              Lease expiration        Type

Hardware address

172.16.10.4      00E0.F70C.7E1E           --                     Automatic

172.16.11.4      0009.7CB0.39E6           --                     Automatic

Task 3: Configure Static and Default Routing

- Configure ISP with a static route for the 209.165.201.0/27 network. Use the exit interface as an argument.

ISP(config)#ip route 209.165.201.0 255.255.255.224 serial 0/0/1

- Configure a default route on R2 and propagate the route in OSPF. Use the next-hop IP address as an argument.

R2(config)#ip route 0.0.0.0 0.0.0.0 209.165.201.2

R2(config)#router rip

R2(config-router)#default-information originate

Task 4: Configure Static NAT

Step 1. Statically map a public IP address to a private IP address.

Statically map the inside server IP address to the public address 209.165.201.30.

R2(config)#ip nat inside source static 172.16.20.254 209.165.201.30

Step 2. Specify inside and outside NAT interfaces.

R2(config)#interface serial 0/0/1

R2(config-if)#ip nat outside

R2(config-if)#interface fa0/0

R2(config-if)#ip nat inside

Step 3. Verify the static NAT configuration.

R2#show ip nat translations

Pro  Inside global     Inside local       Outside local      Outside global

---  209.165.201.30    172.16.20.254      ---                ---

Task 5: Configure Dynamic NAT with a Pool of Addresses

Step 1. Define a pool of global addresses.

Create a pool named NAT_POOL for the IP addresses 209.165.201.9 through 209.165.201.14 using a /29 subnet mask.

R2(config)#ip nat pool NAT_POOL 209.165.201.9 209.165.201.14 netmask 255.255.255.248

Step 2. Create a standard named access control list to identify which inside addresses are translated.

Use the name NAT_ACL and allow all hosts attached to the two LANs on R1.

Note: The .10 LAN must be configured first, then the .11 LAN. Otherwise, Packet Tracer will not grade the ACL as correct.

R2(config)#ip access-list standard NAT_ACL

R2(config-std-nacl)#permit 172.16.10.0 0.0.0.255

R2(config-std-nacl)#permit 172.16.11.0 0.0.0.255

Step 3. Establish dynamic source translation.

Bind the NAT pool to the ACL and allow NAT overloading.

R2(config)#ip nat inside source list NAT_ACL pool NAT_POOL overload

Step 4. Specify the inside and outside NAT interfaces.

Verify that the inside and outside interfaces are all correctly specified.

R2(config)#interface serial 0/0/0

R2(config-if)#ip nat inside

Step 5. Verify the dynamic NAT configuration by pinging from PC1 and PC2 to ISP.

R2#show ip nat translations

Pro  Inside global     Inside local       Outside local      Outside global

icmp 209.165.201.9:2   172.16.10.4:2      209.165.201.2:2    209.165.201.2:2

icmp 209.165.201.9:1024172.16.11.4:2      209.165.201.2:2    209.165.201.2:1024

---  209.165.201.30    172.16.20.254      ---                ---

Task 6: Document the Network

On each router, issue the show run command and capture the configurations.